# Q&A for . . . 							November 5, 2014

# Roadway Design & 3D Modeling- Ch5 Template Design (Part 1 of 3)

Q: Where can I find the recording for this webinar series?

A: All ECSO’s Weekly Webinars are recorded for downloading from the ECSO website, along with any electronic copies of power points, Q&A documents, or other attachments, at: http://www.dot.state.fl.us/ecso/downloads/GoToMeetingTraining/PostedWebinars.shtm#loadSection.

Q: How long will it take the webinar to become available?

A: The webinar should be available by the end of today or tomorrow.

Q: Most times, handrails are display when the trigger component is a fill slope of 1:2 slope. How can we make the trigger the slope instead of a vertical distance?

A: As delivered, the “trigger” is a plan element i.e. railing element, that defines the location on the 2d plan where the designer/engineer has determined that a railing is required.  That said:  the template library component display rules will allow for vertical difference between points if the designer/engineer would prefer that the template automates the display based on the terrain.

Q: Please have another templates webinar!!

A: We will get this scheduled for Jan 7, 2014!

Q: Could you explain the need selecting the superelevation flag?

A: In SS3 when associating superelevation to a corridor, btw: a future webinar, the points that have the superelvation flags checked will be the points used to apply the superelevation control lines.

Q: Where can I see all the points in the template that are targeting the same things in different end conditions?

A: I don’t think that is possible. You must open the component properties dialog or hover over the components individually to see the target.

Q: For the priorities do we need to keep track the name of them, among all the other END COMPONENTS?

A: Priorities are used only when end conditions have the same begin point and target (as a rule).

However, we still try to use unique priorities on other end conditions just as a habit for good house keeping