# Q&A for . . .

# FDOT Civil 3D 2015 DGN to DWG file Conversion

Q: How do we download the FDOT ribbon?

A: You will need to download install the State Kit from our website.  http://www.dot.state.fl.us/ecso/downloads/software/software.shtm

Q: When will the 2015 state kit be released?

A: January 2015.

Q: Where can I see all of the webinars?

A: http://www.dot.state.fl.us/ecso/downloads/GoToMeetingTraining/PostedWebinars.shtm http://www.dot.state.fl.us/ecso/downloads/GoToMeetingTraining/ScheduledWebinars.shtm

Q: are we supposed to set up the dwg file while working on Layout# or Model space?

A: Model space. Layout Tabs don't come into play until you near the end of the project.

Q: Is that a 2d template you used?

A: All templates are in 3D in Civil 3D. Microstation uses 2d and 3D seed files.  An FDOTSeed3D.dgn file is provided in the State Kit install.

Q: Can dgn's be just an xref instead of importing?

A: Yes. As an overlay if you are just using it as a guide or to trace something. If you are making it part of your design it is recommended that you convert it to dwg

Q: Does the DGN file get copied to project, or remain referenced across network?

A: It resides in your project folder

Q: Can Microstation just open the first Civil3d file, why the extra export to autocad2007?

A: It depends on the version of Microstation you are using. The reason he chose 2007 is just to cover all versions of MS.  The reason for the export though is so it will explode the AEC objects so you can open up the drawing without empty AEC boxes.

Q: If the DGN file is not prepared before export will you have problems with exporting the entities into Civil 3D?

A: You should not.  The line type scales may be off a little. In most cases there wasn’t a noticeable difference.