LRE REVIEW GUIDELINE (RRR)

(Updated February /2018)

All projects must have the name of the FDOT project manager.

1. Landscaping – Contact either Elisabeth Hassett or Leslie Wetherell to verify projects that should include this work.

2. Mobilization Factor 8%

3. Maintenance of Traffic 10%

4. Drainage - Check with the Drainage Department to see if drainage maps are needed. Check for inquiries and impacts. Drainage work will be limited to isolated areas which affect safety or pavement stability.

5. Check with Local Agency for additional work or future work associated with project including utilities. All projects should have the standard 999-25 contingency. 
Projects that have a LF contribution, need to have an EX-item for 20% contingency (of the LF amount).  This is in addition to the standard 999-25 contingency.

6. Turn Lanes – Include additional quantities for asphalt as a x-item. Turn lanes modifications or ramp extensions should only be funded by RRR dollars if verified accident data and/or located at signalized intersections. Other locations may be addressed if recommended and potentially funded by the safety review or study. If you have numerous left/right turn lanes you may add an additional lane for resurfacing as an x-item. Bike lanes should be added if there is adequate Right of Way.

7. Signalization – Mast arm upgrade should not be included as part of the LRE until verified as deficient based on signalization replacement criteria. (See Knowledge Based Website under “Design Process - Strain pole design/replacement Criteria (Mast Arm policy)”

8. Lighting – Lighting plans should not be included as part of the LRE until verified as needed based on verified crash history. The addition of proposed lighting should be a recommendation from a Safety Review or a Safety Study.

9. Median Modifications - As a rule, access Management construction will only be done at those locations which have verified accident history. (See safety review recommendations.)

10. Cantilever Signs – Include at intersections of State Roads if required.

11. Bus Bays - Include costs for additional asphalt and additional concrete (Benches and Shelters are to be paid for by the local Transit Authority).

12. Noise walls – Include square footage of walls including height and any special foundations required.

13. Brick Pavers – Include proper Pay Items and criteria. Pavers are not to be placed across State roads. If Patterned Pavement is proposed, an MOA is required, except for Green Bike Lanes.

14. Special Detours – Include estimated costs for each detour. Refer to Estimates Bulletin 14-06 for when to include Special Detours.

15. Bridges – Increase or decrease multiplier based on complexity. Justification for the increase/decrease of the bridge multiplier is required.

Bridge rails need to be verified as up to date or replaced. If you are not sure include in the estimate for each bridge/structure.

16. EX-Items – Are only used for specialty work with no Pay Items, such as Toll Gantries and Toll Buildings.

17. Illuminated Signs - Update all signalized intersections.

18. Resurfacing Projects - Lane Mile cost range will vary depending on the complexity of the project. Minimum 2" milling and 2" resurfacing.

19. Resurfacing Projects - Areas where prevalent rutting is observed should be accounted for in milling thickness and additional tonnage required. This should be included as an x items with the associated sequence.

20. Please remember we do not use the contingencies anymore.

21. You may check the LRE Designer Interface for price information.

22. Include Non-Bid items

LRE's over $2 million: Include pay item 999-16 with a quantity of 2 days

LRE's over $15 million: Include both pay items 999-20-1 and 999-20-2 with a quantity as per BOE

23. Off-System roadways are not to have pay items for Mowing and Litter Removal

ADDITIONAL GUIDELINES

1. If a consultant needs to access the LRE of a project, please provide the FDOT Project Manager, through an email, the financial project number, firm name, user id, email address and name of the person doing the input.

2. For a FDOT person or an FDOT Project Manager who needs access to an existing LRE, please provide Final Plans, through an email, the FM project number, user id and name of the requested person to have access.

3. If you are submitting a Trns*port estimate for your project, you are still required to provide a cost estimate. You may get estimated costs from the LRE Designer Interface or from estimated costs provided by others/consultants, or States Estimates website.

4. Part of the RRR process is the creation of an initial scope history based on "concept report" developed for each candidate project.

5. Adjacent project or recently completed project in the project limits needs to be noted in the scoping report and described in the LRE.