INSTRUCTIONS

[THIS PAGE TO BE REMOVED UPON COMPLETION OF THE REPORT]

This Design Noise Study Report (DNSR) template may be used to create Florida Department of Transportation (FDOT) Noise Study Reports. The following “user-inputs” are used in this document to guide the author on writing the report, listed in Table Ex.-1.

Table Ex.-1 – User Inputs

| Text Input Object | How does Writer/Author Respond? |
| --- | --- |
| Blue italicized text | This instructional text guides the writer on how to compile the document. This temporary text should be removed before the report is finalized. |
| [BLOCK ENTRIES] | These are items that need to be replaced by the parameter indicated inside the block (i.e., “[ROADWAY NAME]” would be replaced by “U.S. 27”). |
| Dropdown boxes (e.g., “is/is not”, “would/would not”) | The appropriate item should be chosen from the drop-down list using the down arrow. |
| If/Then paragraphs | The appropriate paragraph should be used. These If/Then statements are listed directly in the text as well as in tables. Any unused/inapplicable paragraphs should be removed. |

Version Created: June 2026

Design Noise Study Report

Florida Department of Transportation

District [NUMBER]

[PROJECT TITLE]

[LIMITS OF THE PROJECT]

[COUNTY NAME], Florida

Financial Management Number: [FM NUMBER]

ETDM Number: [ETDM NUMBER]

[DATE]

The environmental review, consultation, and other actions required by applicable federal environmental laws for this project are being, or have been, carried out by the Florida Department of Transportation (FDOT) pursuant to 23 U.S.C. § 327 and a Memorandum of Understanding dated May 26, 2022, and executed by the Federal Highway Administration and FDOT.

EXECUTIVE SUMMARY

The Florida Department of Transportation (FDOT) District [NUMBER] is designing roadway improvements to [ROADWAY NAME]  from [ROADWAY NAME] to [ROADWAY NAME] (Financial Project Identification Number [FPID] [NUMBER]), in [COUNTY NAMES] County, a distance of approximately [NUMBER OF MILES] miles. The proposed improvements consist of [PROPOSED IMPROVEMENTS DECRIPTIONS]. The project improvements qualify for a Type I noise project due to [LIST QUALIFYING PROJECT ACTIVITIES FROM THE TYPE I PROJECT LIST].

This Design Noise Study Report (DNSR) presents the methodology and results of the highway traffic noise evaluation for [PROJECT NAME] (Financial Management Number [FM NUMBER]). The purpose of this noise study is to identify noise sensitive sites that would be impacted by the preferred alternative, evaluate abatement measures at impacted noise sensitive sites, and determine where noise abatement (i.e., noise barriers) should be included in the design plans.

If there are no impacted receptors within the project, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The Federal Highway Administration (FHWA) Traffic Noise Model (TNM), version 2.5, was utilized to predict noise levels at [NUMBER] receptor points representing [NUMBER] residences and [NUMBER] special land uses (SLUs). For the year [YEAR] Build condition, noise levels are not predicted to approach, meet, or exceed the Noise Abatement Criteria (NAC) at any residence or SLUs within the project limits. Additionally, a substantial increase of 15 dB(A) is not predicted to occur at any residence or SLU. Based on the noise analyses performed to date, there appear to be no impacted areas within the project that require abatement consideration.

If there are impacted receptors within the project, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The Federal Highway Administration (FHWA) Traffic Noise Model (TNM), version 2.5, was utilized to predict noise levels at  receptor points representing [NUMBER] residences and [NUMBER] special land uses (SLUs).  For the year [YEAR] Build condition, noise levels are predicted to approach, meet, or exceed the Noise Abatement Criteria (NAC) at [NUMBER] residences and [NUMBER] SLUs within the project limits. These impacted noise sensitive sites were evaluated to determine the feasibility and reasonableness of providing barriers to reduce traffic noise. Additionally, a substantial increase of 15 dB(A) is/is not predicted to occur at any residence or SLU.

If impacts were identified and noise barriers were evaluated but unable to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The noise barrier evaluation process identified that a noise barrier/noise barriers would/would not provide at least a 5 decibel (dB[A]) reduction to [NUMBER] impacted residences and [NUMBER] impacted SLUs. However/Additionally, the noise barriers would/would not meet the Noise Reduction Design Goal (NRDG). However/Therefore, the noise barriers do not meet the criteria of feasibility and/or reasonableness to warrant the construction of a noise barrier. Based on the noise analyses performed to date, there are no feasible solutions available to mitigate the noise impacts at the locations identified in Section ‎3.3.

If noise barriers were able to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The noise barrier evaluation process identified that noise barriers are a feasible and reasonable form of abatement and could potentially provide at least a 5 dB(A) reduction at [NUMBER] of the [NUMBER] impacted residences ([NOISE ANALYSIS AREA NAMES]), and at  [NUMBER] of the [NUMBER] impacted SLUs ([SLU NAMES/DESCRIPTIONS]) at a cost below the reasonable limit. Additionally, the noise barrier achieves the Noise Reduction Design Goal (NRDG). Noise barriers were found to be potentially feasible and reasonable at the following Noise Analysis Areas (NAA):

- [NAA # AND DESCRIPTION OF RESIDENTIAL AREAS AND/OR SLUS]

- [NAA # AND DESCRIPTION OF RESIDENTIAL AREAS AND/OR SLUS]

- [NAA # AND DESCRIPTION OF RESIDENTIAL AREAS AND/OR SLUS]

Noise barriers cannot provide at least a 5 dB(A) reduction to [NUMBER] impacted residences and [NUMBER] impacted SLUs because they do not meet the criteria of feasibility and/or reasonableness to warrant the construction of a noise barrier.

Table ES-1

Feasible and Reasonable Noise Barriers [EXAMPLE]

| Barrier | Location | Stationing | Barrier Length (ft) | Barrier Height (ft) |
| --- | --- | --- | --- | --- |
| R-1 Southbound | 7-ft within Right-of-Way | Begin STA. 3267+50, 88.50’ (LT) End STA. 3271+86.20, 88.50’ (LT) | 1,718 | 12 |
| XXX | XXX | XXX | XXX | XXX |
| XXX | XXX | XXX | XXX | XXX |

Table of Contents

To update Table of Contents, right-click and select “Update Field” and then “Update entire table”.

SECTION 1	Introduction	1

1.1	Project Description	1

1.2	Purpose and Need	3

1.3	Existing Facility	3

1.4	Proposed Improvements	3

1.5	Summary of PD&E Results and Commitments	3

SECTION 2	Methodology	7

2.1	Noise Metrics	7

2.2	Traffic Data	7

2.3	Noise Abatement Criteria	8

2.4	Noise Abatement Measures	10

2.4.1	Noise Barriers	10

2.4.2	Noise Barriers for Special Land Uses (Non-residential)	11

2.4.3	Existing Noise Barriers	12

2.5	Noise Model	14

2.5.1	Model Validation	14

2.6	Noise Sensitive Sites	15

2.6.1	Common Noise Environments and Noise Analysis Areas	16

SECTION 3	Traffic Noise Analysis Results	17

3.1	Existing Noise Levels	17

3.2	Substantial Increase in Noise	20

3.3	Noise Abatement Criteria Evaluation	20

3.3.1	NAA #1	20

SECTION 4	Outdoor Advertising	25

SECTION 5	Conclusions	26

5.1	Impacted Noise Sensitive Sites	26

5.2	Recommended Noise Barriers	26

SECTION 6	Construction Noise and Vibration	28

SECTION 7	Community Coordination	29

7.1	Public Involvement	29

7.1.1	Noise Barrier Surveys	29

REFERENCES	30

APPENDIX A Traffic Data	A-1

APPENDIX B Project Aerials	B-1

APPENDIX C Predicted Noise Levels	C-1

APPENDIX D Model Validation Field Sheets	D-1

APPENDIX E SLU Equivalent Residence Calculation Worksheet	E-1

APPENDIX F Engineering Review	F-1

APPENDIX G Noise Barrier Survey Package	G-1

Note: modify the list of appendices based on the project. Not every project will require all the currently listed appendices. For instance, if noise model validation is not discussed in the DNSR, remove Appendix D, and change Appendix E to D.

List of Figures

Figure ‎1-1: Project Location Map [EXAMPLE]	2

Figure ‎1-2: Existing Typical Sections [EXAMPLE]	5

Figure ‎1-3: Proposed Typical Sections [EXAMPLE]	6

List of Tables

Table ‎2-1 Noise Abatement Criteria	9

Table ‎2-2 Typical Noise Levels	10

Table ‎2-3 Feasible and Reasonable Requirements for a Noise Barrier	11

Table ‎2-4 Existing Noise Barriers (EXAMPLE)	12

Table ‎2-5 Noise Model Validation	15

Table ‎3-1 Ambient Noise Monitoring (EXAMPLE)	19

Table ‎3-2 NAA #1: Noise Barrier Evaluation Results	24

Table ‎5-1 Potentially Feasible and Reasonable Noise Barriers	27

Table ‎5-2 Noise Analysis Areas Not Qualifying for Abatement	27

ACRONYMS

Acronyms may be added or removed based on use in this document.

BER		Benefited Equivalent Residence

CFR		Code of Federal Regulations

CNE		Common Noise Environment

dB		Decibels

dB(A)		A weighted decibels

DPK		Date of Public Knowledge

ER		Equivalent Residence

ETDM		Efficient Transportation Decision Making

FDOT		Florida Department of Transportation

FHWA		Federal Highway Administration

FM No.		Financial Management Number

ft.		Feet

Leq(h)		Hourly equivalent noise levels

LOS		Level of Service

mph		Miles per hour

MSE		Mechanically-Stabilized Earth

NAA		Noise Analysis Area

NAC		Noise Abatement Criteria

NEPA		National Environmental Policy Act

NRDG		Noise Reduction Design Goal

NSR		Noise Study Report

PER		Preliminary Engineering Report

PD&E		Project Development and Environment

ROW		Right-of-way

SLM		Sound Level Meter

SLU		Special Land Use

sq. ft.		Square Feet

TER		Total Equivalent Residence

TNM		Traffic Noise Model

# Introduction

The Design Noise Study Report should be written from the context of a PD&E Study having already been conducted; thus, this document would be considered a re-evaluation. For instances where the project is in the design phase, needs noise analysis/documentation, BUT did not undergo PD&E Study, adjust the text of this section and subsequent sections accordingly, e.g., re-evaluation vs. evaluation, design concept vs. Build Alternative, design plans vs. PER, etc.

The Florida Department of Transportation (FDOT) District [NUMBER] is preparing  a Project Development and Environment (PD&E) Study evaluating proposed improvements to [ROAD NAME] from [ROAD NAME] to [ROAD NAME], in [COUNTY NAMES] County, a distance of about [NUMBER] miles. The project limits are shown in Figure ‎1-1.

This Design Noise Study Report (NSR) presents the methodology and results of the highway traffic noise re-evaluation for [PROJECT NAME] (Financial Management Number [FM NUMBER]). [Phase II/Phase III] Plan sets dated [NUMBER] were utilized as a basis for this evaluation.

The purpose of this noise study is to identify noise sensitive sites that would be impacted by the preferred alternative, evaluate abatement measures at impacted noise sensitive sites, and determine where noise abatement (i.e., noise barriers) should be included with the project and re-evaluated in the design phase.

Secondary objectives of this study include the consideration of construction noise and vibration impacts as well as the development of noise contours, that can be used in the future by local municipal and county government agencies to identify compatible land uses along the project roadways.

This report also provides technical documentation for the findings described in the project’s Preliminary Engineering Report (PER) and the [TYPE OF ENVIRONMENTAL DOCUMENT] .

## Project Description

[STANDARDIZED PROJECT DESCRIPTION]

Figure ‎1-1: Project Location Map [EXAMPLE]

## Purpose and Need

The Purpose and Need Section should be verbatim to the environmental document. However, if doing so would cause this section to become too lengthy, coordinate with the District Noise Specialist on what information to include.

The purpose of this project, as stated in the approved  [TYPE OF ENVIRONMENTAL DOCUMENT]  is to:

[STANDARDIZED PURPOSE AND NEED STATEMENT FOR PROJECT]

## Existing Facility

As shown in the existing typical sections in Figure ‎1-2, the existing facility consists of [EXISTING FACILITY DECRIPTION]

## Proposed Improvements

The proposed improvements consist of [PROPOSED IMPROVEMENTS DECRIPTION].

As shown in Figure ‎1-3, the typical section consists of [PROPOSED TYPICAL SECTION DECRIPTION]. The concept plans for the Build Alternative(s) is/are included in the PER.

## Summary of PD&E Results and Commitments

If there were no impacted receptors within the project, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

As part of the project’s PD&E Study phase, a Noise Study Report (NSR) was prepared in [PD&E NSR DATE] . The NSR concluded that for the year [YEAR] Build condition, noise levels were not predicted to approach, meet, or exceed the Noise Abatement Criteria (NAC) at any residence or non-residential Special Land Use (SLU) within the project limits. Additionally, a substantial increase of 15 dB(A) was not predicted to occur at any residence or SLU. Based on the noise analyses performed as part of the PD&E Study, there appeared to be no impacted areas within the project that required abatement consideration.

If impacts were identified and noise barriers were evaluated but unable to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

As part of the project’s PD&E Study phase, a Noise Study Report (NSR) was prepared in [PD&E NSR DATE] . The NSR concluded that a noise barrier/noise barriers would/would not provide at least a 5 dB(A) reduction to [NUMBER] impacted residences and [NUMBER] impacted SLUs. However/Additionally, the noise barriers would/would not meet the Noise Reduction Design Goal (NRDG). However/Therefore, the noise barriers did not meet the criteria of feasibility and/or reasonableness criteria and were not recommended for further consideration.

The following commitments were made in the [TYPE OF ENVIRONMENTAL DOCUMENT]

[NOISE RELATED COMMITMENT(S) and/or STATEMENT OF LIKELIHOOD]

If impacts were identified and noise barriers were able to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

As part of the project’s PD&E Study phase, a Noise Study Report (NSR) was prepared in [PD&E NSR DATE] . The NSR concluded that a noise barrier/noise barriers would provide at least a 5 dB(A) reduction, achieve the Noise Reduction Design Goal (NRDG), and meet the cost-effectiveness criteria utilized at the time of the study.

[NAA #, DESCRIPTION OF RESIDENTIAL/SLU, BARRIER HEIGHT/LENGTH, NUMBER of BENEFITS]

[NAA #, DESCRIPTION OF RESIDENTIAL/SLU, BARRIER HEIGHT/LENGTH, NUMBER of BENEFITS]

[NAA #, DESCRIPTION OF RESIDENTIAL/SLU, BARRIER HEIGHT/LENGTH, NUMBER of BENEFITS]

[NAA #, DESCRIPTION OF RESIDENTIAL/SLU, BARRIER HEIGHT/LENGTH, NUMBER of BENEFITS]

The NSR also concluded that a noise barrier/noise barriers would/would not meet feasible and reasonable criteria at [NUMBER] impacted residences and [NUMBER] impacted SLUs.

The following commitments were made in the [TYPE OF ENVIRONMENTAL DOCUMENT] ”

[NOISE RELATED COMMITMENT(S) and/or STATEMENT OF LIKELIHOOD]

If more than two typical sections need to be included, it is recommended to put all typical section graphics into an Appendix.

Figure ‎1-2: Existing Typical Sections [EXAMPLE]

Figure ‎1-3: Proposed Typical Sections [EXAMPLE]

# Methodology

This highway traffic noise study was completed in accordance with Title 23, Code of Federal Regulations, Part 772 (23 CFR 772), Procedures for Abatement of Highway Traffic Noise and Construction Noise following methodology and procedures established by the FDOT in the PD&E Manual, Part 2, Chapter 18, Highway Traffic Noise, and the FDOT Traffic Noise Modeling and Analysis Practitioners’ Handbook.

The FHWA’s Traffic Noise Model (TNM), version 2.5, was used to predict existing and future traffic noise levels and to analyze the effectiveness of noise barriers, where warranted. This model estimates the acoustic intensity at noise sensitive receptor sites from a series of roadway segments (the source).

Representative receptor sites were used as inputs to the TNM to estimate noise levels associated with existing and future conditions within the project limits. These sites were chosen based on noise sensitivity, roadway proximity, anticipated impacts from the proposed project, and homogeneity (e.g., the site is representative of other nearby sites). For single-family residences, traffic noise levels were predicted at the edge of the dwelling unit closest to the nearest primary roadway. For other noise sensitive sites that may be impacted (e.g., non-residential land uses), traffic noise levels were predicted where the exterior use/activity occurs. The next two sentences should be removed if interior analysis was not warranted. For the prediction of interior noise levels, the receptor site was placed inside the building at the corner of the façade closest to the roadway or in the interior area determined to be the area of frequent use. Building noise reduction factors and window conditions identified in Figure 18-3 in Part 2, Chapter 18 of the PD&E Manual were used to determine the potential for interior noise impacts.

## Noise Metrics

Noise levels developed for this traffic noise study are expressed in decibels (dB) using an “A”-scale [dB(A)] weighting. This scale most closely approximates the response characteristics of the human ear to typical traffic noise levels. All reported noise levels are hourly equivalent noise levels [Leq(h)]. The Leq(h) is defined as the equivalent steady-state sound level that, in an hourly period, contains the same acoustic energy as the time-varying sound level for the same hourly period. Use of these metrics is consistent with the requirements of 23 CFR 772.

## Traffic Data

This section should be modified to fit the context of the purpose of the DNSR. Typically, a design-level noise analysis will only be conducted for the existing condition and concept undergoing re-evaluation. Instances where the project is in the design phase, needs noise analysis/documentation, BUT did not undergo a PD&E Study, analysis of the No-Build condition may be warranted. Coordination with the District Noise Specialist may be necessary.

Traffic data is of primary importance in a traffic noise prediction model. FDOT traffic data for the existing [YEAR] and [YEAR] design concept/Build Alternative were obtained from [FDOT/SOURCE] and based upon data found in [RESOURCE]. The traffic data were reviewed to identify forecasted traffic volumes that would yield the highest traffic noise impact. As required in the FDOT’s PD&E Manual, when the predicted design year peak hour directional demand traffic volumes equal or exceed Level of Service (LOS) C, LOS C hourly traffic volumes were utilized. For roadway segments where the predicted peak hour directional demand traffic volumes is less than LOS C, the peak hour demand volumes were utilized. For this re-evaluation, the [YEAR] Build condition was modeled with peak hour directional demand volumes for [ROAD NAMES] while LOS C volumes were modeled for [ROAD NAMES].

Traffic volumes and speeds used in the analysis are included in Appendix A.

## Noise Abatement Criteria

Noise sensitive land uses are areas where there is frequent human use that may be impacted by traffic noise levels that approach, meet, or exceed the Noise Abatement Criteria (NAC) – levels established by the Federal Highway Administration (FHWA) at which abatement must be considered. Typical noise sensitive land uses include residences, schools, places of worship, commercial properties with outdoor areas of use, and recreational areas. As shown in Table ‎2-1, the NAC vary by activity category.

The FDOT NAC is defined as being within one dB(A) of the FHWA’s NAC to reflect values that “approach” the FHWA criteria. For perspective on the decibel values, Table ‎2-2 provides typical noise levels of common indoor and outdoor activities.

Noise abatement measures must also be considered when a substantial increase in traffic noise is predicted to occur as a direct result of a transportation project. FDOT defines a substantial increase as 15 dB(A) or more above existing conditions. A substantial increase typically occurs in areas where traffic noise is a minor component of the existing noise environment but would become a major component after the project is constructed. This occurs more frequently with new alignment projects.

Table ‎2-1
Noise Abatement Criteria

| Activity Category | Activity Leq(h) |  | Evaluation Location | Description of Land Use Activity Category |
| --- | --- | --- | --- | --- |
|  | FHWA | FDOT |  |  |
| A | 57 | 56 | Exterior | Lands on which serenity and quiet are of extraordinary significance and serve an important public need and where the preservation of those qualities is essential if the area is to continue to serve its intended purpose. |
| B | 67 | 66 | Exterior | Residential. |
| C | 67 | 66 | Exterior | Active sports areas, amphitheaters, auditoriums, campgrounds, cemeteries, day care centers, hospitals, libraries, medical facilities, parks, picnic areas, places of worship, playgrounds, public meeting rooms, public or nonprofit institutional structures, radio studios, recording studios, recreation areas, Section 4(f) sites, schools, television studios, trails, and trail crossings. |
| D | 52 | 51 | Interior | Auditoriums, day care centers, hospitals, libraries, medical facilities, places of worship, public meeting rooms, public or nonprofit institutional structures, radio studios, recording studios, schools, and television studios. |
| E | 72 | 71 | Exterior | Hotels, motels, offices, restaurants/bars, and other developed lands, properties or activities not included in A – D or F. |
| F | ------ | ------ | ------ | Agriculture, airports, bus yards, emergency services, industrial, logging, maintenance facilities, manufacturing, mining, rail yards, retail facilities, shipyards, utilities (water resources, water treatment, electrical), and warehousing. |
| G | ------ | ------ | ------ | Undeveloped lands that are not permitted. |
| (Based on Table 1 of 23 CFR 772 and FDOT PD&E Manual Chapter 18) 1 The Leq(h) Activity Criteria values are for impact determination only, and are not design standards for noise abatement measures. 2 Includes undeveloped lands permitted for this activity category. Note: FDOT defines that a substantial noise increase occurs when the existing noise level is predicted to be exceeded by 15 decibels or more as a result of the transportation improvement project. When this occurs, the requirement for abatement consideration will be followed. |  |  |  |  |

Table ‎2-2
Typical Noise Levels

| Common Outdoor Activities | Noise Level dB(A) | Common Indoor Activities |
| --- | --- | --- |
| Jet Fly-over at 1000 ft. Gas Lawn Mower at 3 ft. Diesel Truck at 50 ft., at 50 mph Noise Urban Area (Daytime) Gas Lawn Mower at 100 ft. Commercial Area Heavy Traffic at 300 ft. Quiet Urban Daytime Quiet Urban Nighttime Quiet Suburban Nighttime Quiet Rural Nighttime Lowest Threshold of Human Hearing | ---110--- ---100--- ---90--- ---80--- ---70--- ---60--- ---50--- ---40--- ---30--- ---20--- ---10--- ---0--- | Rock Band Food Blender at 3 ft. Garbage Disposal at 3 ft. Vacuum Cleaner at 10 ft. Normal Speech at 3 ft. Large Business Office Dishwasher Next Room Theater, Large Conference Room (Background) Library Bedroom at Night, Concert Hall (Background) Lowest Threshold of Human Hearing |

Source:  California Dept. of Transportation Technical Noise Supplement, September 2013, Page 2-20.

## Noise Abatement Measures

If the District desires a detailed outline of all noise abatement measures, refer to the PD&E NSR Template, Section 2.4 and associated text. Otherwise, the following text, or variation thereof, can be used.

Noise abatement is considered at all noise sensitive sites (represented by receptors) predicted to meet or exceed the FDOT NAC, as stipulated by 23 CFR 772. Abatement measures considered include traffic management, alignment modifications, noise buffer zones through application of land use controls and noise barriers.

Noise abatement measures including traffic management, alignment shifts, buffer zones, and noise barriers were given further consideration for the impacted noise sensitive sites during the PD&E phase. However, noise barriers were the only feasible and reasonable form of abatement for impacted noise sensitive sites.

### Noise Barriers

Noise barriers reduce noise levels by interrupting the sound path between a highway and noise sensitive site. Barriers can cause sound waves to bend around and over the barrier (diffraction) which produces a “shadow zone” behind the barrier itself. To effectively reduce traffic noise, a barrier must be relatively long, continuous (with no intermittent gaps), and of sufficient height. A noise barrier must be considered a feasible and reasonable noise abatement measure to be suggested for inclusion into the project. Feasibility factors are related to the acoustical and engineering properties of an abatement measure. Reasonableness factors are related to the environmental, economic, and social properties. For a noise barrier to be considered feasible and cost reasonable, the following minimum conditions should be met (Table ‎2-3):

Table ‎2-3 Feasible and Reasonable Requirements for a Noise Barrier

| Feasible | Reasonable |
| --- | --- |
| At least two impacted receptors must be provided a noise reduction of 5 dB(A). | A noise barrier must also attain the Noise Reduction Design Goal (NRDG), which states that a minimum noise reduction of 7 dB(A) for at least one benefited receptor must be achieved. This receptor may also have been previously identified as meeting the feasibility requirement of receiving a 5 dB(A) reduction. |
| Engineering factors (design/construction, safety, access, ROW, maintenance, drainage, and utility) must be considered, and all conflicts must be resolved. | The cost of the noise barriers should not exceed $64,000 per benefited receptor. This is the upper cost limit established by FDOT. A benefited receptor is defined as a recipient of an abatement measure that experiences at least a 5 dB(A) reduction as a result of providing a noise barrier. The current unit cost used to evaluate cost reasonableness is $40 per square foot (sq. ft.). |
| -- | The viewpoints of benefited receptors must be considered. |

Within the project limits, noise barrier locations are evaluated as follows:

Right-of-way (ROW) noise barriers located outside the clear zone, which is defined in the FDOT Design Manual as ‘the amount of recoverable area beyond the traveled way’ but within the ROW, are initially considered at heights ranging from 8 ft. to 22 ft. in 2 ft. increments. According to the FDOT Design Manual, noise barriers outside the clear zone shall not exceed a maximum height of 22 ft.

If a ROW barrier cannot provide at least a 5 dB(A) reduction to an impacted receptor or the barrier is not feasible due to engineering and/or construction limitations, then a shoulder barrier is evaluated. According to the FDOT Design Manual, shoulder barriers shall not exceed 14 ft. in height when on embankment and 8 ft. in height when on structure or Mechanically Stabilized Earth (MSE).

The length and height of the noise barriers are optimized based on the benefit provided to noise sensitive sites with predicted noise levels that meet or exceed the FDOT NAC.

### Noise Barriers for Special Land Uses (Non-residential)

This section, or a variation thereof, can be used if barriers were evaluated for Special Land Uses (non-residential land uses) as part of the final design process. Otherwise, it should be removed.

The methodology used to evaluate noise barrier systems for non-residential sites differs from that used for residential locations. The standard procedure for determining the feasibility and reasonableness of a noise barrier for a special land use (SLU) site is documented in Methodology to Evaluate Traffic Noise at Special Land Uses (FDOT 2024). In this methodology, SLUs are assigned an Equivalent Residence (ER) value based on the person-hours of use at the SLU in order to evaluate the reasonableness and feasibility of a noise barrier. This SLU evaluation is a multi-step process, as summarized below.

- If an impacted SLU receptor is not adjacent to impacted residences or other impacted SLUs such that a single noise barrier would not be a practical form of abatement for all impacted properties, it is considered isolated. An optional preliminary screening analysis may be applied to isolated impacted SLUs to determine if it has enough person-hour usage to equate to at least two residences to be found feasible for noise abatement. To meet the feasibility requirement, the isolated SLU must have at least 44,326 person-hours of use per year (equivalent to the number of person-hours per year for two single family residences) in the benefited area for a noise barrier to be found as a feasible form of noise abatement.

- A noise barrier is evaluated if the preliminary screening results indicate that a full analysis is warranted or if the impacted SLU is adjacent to other impacted SLUs or residences.

- A noise barrier is found feasible and reasonable if the requirements of Table ‎2-3 are met, which can be determined through the use of the FDOT SLU Worksheet. Once it is determined that impacted SLUs benefit from the analyzed noise barrier, the FDOT SLU Worksheet is utilized to assess whether a noise barrier is a reasonable form of abatement. The SLU Worksheet includes all residences and SLUs that would receive a benefit from the noise barrier. This methodology allows the combined evaluation of all impacted land use activity categories that would potentially benefit from a single noise barrier system.

### Existing Noise Barriers

If existing noise barriers are present in the project limits, include this section with the following text or variation thereof (otherwise, remove this section and the following statement).

Of importance, [NUMBER] existing noise barrier is/barriers are present within the proposed project limits and are listed in Table ‎2-4.

Table ‎2-4
Existing Noise Barriers (EXAMPLE)

| Existing Noise Barrier Number | NAAs | Location | Approx. Height (ft.) | Approx. Length (ft.) | Construction Year |
| --- | --- | --- | --- | --- | --- |
| 1 | ABC, XYZ, and PQR Neighborhood | ROW | 22 | 2,500 | 2014 |
| 2 | Residences from ABC Street to XYZ Road | Shoulder | 14 | 1,700 | 2014 |
| 3 | Residences from ABC Street to XYZ Road | Shoulder/Structure | 14/8 | 1,000/500 | 2014 |

This section, or a variation thereof, can be used if existing barriers were evaluated as part of the final design process. Otherwise, it should be removed.

The standard procedure for determining traffic noise impacts for a project where there are existing FDOT noise barriers is documented in FDOT’s Methodology to Evaluate Existing Noise Barriers (FDOT 2025). This methodology is used to determine whether to replace, extend, or maintain the existing noise barrier if there are traffic noise impacts from the Build condition. The following summarizes the steps taken for the evaluation of the two existing noise barriers within the project corridor.

- Identify Impacts. Are receptors impacted under the Build condition with the existing noise barrier included in the analysis? If no, the analysis is complete and the existing noise barrier is maintained. If yes, the evaluation proceeds to Step 2.

- Determine if Existing Barrier Meets Current FDOT Requirements. The intent of Step 2 is to determine the abatement merits of the existing noise barrier and if it meets the FDOT acoustic feasibility and reasonableness requirements with the proposed project. If the existing barrier is proven to provide acoustically feasible abatement to all impacted receptors and meets the NRDG, the existing barrier is not a candidate for further abatement strategies.

A comparison is made between the predicted Build condition noise levels without the existing noise barrier and the predicted Build condition with the existing noise barrier. Does the existing noise barrier benefit two impacted receptors AND meet the NRDG? If no, the evaluation proceeds to Step 3. If yes, does the Build condition impact receptors that are not benefited by the existing noise barrier? If no, the analysis is complete and the existing noise barrier is maintained. If yes, the evaluation proceeds to Step 3.

- Compare Build Condition Noise Levels to Existing Noise Levels. The intent of Step 3 is to compare the future Build condition and existing noise levels to determine the extent of the proposed roadway design's noise increase. Is the change in noise levels from the existing condition to the Build Alternative greater than 3 dB(A) at any impacted receptor that is not benefited by the existing noise barrier with the Build Alternative? If not, the analysis is complete. If yes, the evaluation proceeds to Step 4.

- Evaluate Lengthening Existing Barrier. The intent of Step 4 is to determine if the existing noise barrier can be lengthened to provide effective noise reduction to the impacted, but not currently benefited, receptors identified in Step 2. To determine if the "lengthened noise barrier" scenario meets the feasibility and reasonableness criteria, a comparison is made between the lengthened noise barrier scenario and the "without noise barrier" from Step 2.

If the lengthened noise barrier segment(s) is not feasible and reasonable while maintaining the existing barrier's height, the evaluation proceeds to Step 5.

- Is the Barrier a Candidate for Replacement? Once it has been established that lengthening the existing barrier is not feasible or reasonable, it is suggested that replacement options be evaluated. However, it must be possible for a potential replacement noise barrier to be at least 6 ft. taller than the existing noise barrier. According to FHWA guidance, an additional 6 ft. in height equals an approximate 3 dB(A) decrease, assuming the line of sight is blocked. A 3 dB(A) decrease is the lowest perceptible difference in noise reduction unless a change in the line of sight from any receptor to the roadway occurs. If the existing noise barrier is located on a bridge structure or along the shoulder of the roadway, the existing noise barrier is not eligible for replacement unless a change in the line of sight is possible. If the existing barrier is ground-mounted and taller than 16 ft., it is not a candidate for replacement unless the line of sight is altered.

If the line of sight cannot be altered, the analysis is complete; no feasible and reasonable replacement or lengthening options exist, and the existing barrier is maintained. If the line of sight can be altered, the evaluation proceeds to Step 6.

- Evaluate Replacing the Existing Noise Barrier. Following the FDOT Traffic Noise Modeling & Analysis Practitioners' Handbook, the final step evaluates the optimum replacement barrier dimensions (length and height). To calculate a receptor's noise reduction from the replacement barrier, compare the "replacement noise barrier" scenario to a "no noise barrier" scenario (Step 2). This allows for a more achievable noise reduction, as the existing noise barrier already provides a reduction in noise levels. All newly benefitted receptors (those not benefited by the existing noise barrier) are included, as is the entire length/height of the replacement noise barrier in the cost-effectiveness calculation, excluding the cost of removing the existing noise barrier. Does the replacement noise barrier meet FDOT's feasible and reasonable criteria, excluding the cost of removing the existing noise barrier and only counting receptors not already benefited by the existing noise barrier? If not, no feasible and reasonable options exist, and the existing barrier is maintained.

## Noise Model

Noise level predictions were produced using the FHWA’s TNM Version [VERSION NUMBER]. This model calculates estimated noise levels at noise sensitive receptor sites based on highway traffic parameters that are input to the model. Model-predicted noise levels are influenced by several factors including vehicle speed and types, the distance between the noise source and receptor, the effects of intervening structures (buildings, barriers, etc.), ground surface type, and topography.

### Model Validation

If there have not been any significant changes to the noise model (e.g., Changing from STAMINA to TNM, or changing from TNM 1.0 to TNM 2.5) use the following language to reference PD&E Validation.

Noise model validation was conducted during the Project Development and Environment (PD&E) phase in accordance with FDOT and FHWA guidelines. Since there have been no substantial changes to the noise model or surrounding/adjacent land use since the PD&E phase, re-validation of the noise model is not required. The previous validation results, per the [REFERENCE FOR PD&E NSR] and the validated model remains appropriate for use in the current design phase.

If there have been any significant changes to the noise model (e.g., Changing from STAMINA to TNM, or changing from TNM 1.0 to TNM 2.5) or surrounding/adjacent land uses that could affect the local soundscape, use the following language. If no changes have been made, remove the following language and Section 2.5.1.1.

To validate the accuracy of the computer noise model for the project area, field measurements were taken following procedures documented in FHWA’s Noise Measurement Handbook. All monitoring events were a minimum of 30 minutes (3 repetitions of 10 minutes each) in duration, consistent with FDOT procedures. Noise monitoring was performed on [DATE] using a [MAKE/MODEL OF SOUND LEVEL METER] [NUMBER] sound level meter (SLM). Prior to taking noise measurements, the SLM was calibrated using a [MAKE/MODEL OF CALIBRATOR] calibrator. TNM model validation was performed at [NUMBER] sites along the project corridor. The validation sites were located along the ROW of the existing highway. Traffic volumes by vehicle classification were noted during each monitoring event. A  [MODEL] radar gun was used to obtain average operating speeds. Field notes for each monitoring event are provided in Appendix D. Locations of monitoring sites are depicted in aerials provided in Appendix B.

#### Model Validation Results

The results for each monitoring event are provided in Table ‎2-5. The FDOT PD&E manual states that the model is validated if the existing field measured highway traffic noise levels and predicted highway traffic noise levels for the existing condition are within 	+/- 3.0 dB(A). The variance between measured and predicted noise levels was less than 3 dB(A). Therefore, the noise model is predicting within the level of accuracy specified in FDOT’s PD&E Manual, Part 2, Chapter 18.  If the noise measurements at the validation sites are outside the TNM predicted model limits of +/- 3.0 dB(A), additional measurements or project reconnaissance may be necessary to determine why noise measurements did not validate.

Table ‎2-5
Noise Model Validation

| Location | Run # | Date | Start Time | Field Measured dB(A) | TNM Predicted dB(A) | Variance dB(A) |
| --- | --- | --- | --- | --- | --- | --- |
| Validation Site #1 ~XX’ from Edge of Pavement | 1 | [XX/XX/XX] | XX:XX | XX.X | XX.X | XX.X |
|  | 2 |  | XX:XX | XX.X | XX.X | XX.X |
|  | 3 |  | XX:XX | XX.X | XX.X | XX.X |
| Validation Site #2 ~XX’ from Edge of Pavement | 1 | [XX/XX/XX] | XX:XX | XX.X | XX.X | XX.X |
|  | 2 |  | XX:XX | XX.X | XX.X | XX.X |
|  | 3 |  | XX:XX | XX.X | XX.X | XX.X |
| Validation Site #3 ~XX’ from Edge of Pavement | 1 | [XX/XX/XX] | XX:XX | XX.X | XX.X | XX.X |
|  | 2 |  | XX:XX | XX.X | XX.X | XX.X |
|  | 3 |  | XX:XX | XX.X | XX.X | XX.X |

## Noise Sensitive Sites

The receptors representing noise sensitive sites are located in accordance with 23 CFR 772, FDOT’s PD&E Manual, Part 2, Chapter 18 and the FDOT Traffic Noise Modeling & Analysis Practitioners Handbook as follows:

Activity Category B and D receptors were located at the edge of the dwellings/buildings that are closest to [ROAD NAME].

Activity Category E receptors were located in exterior areas where frequent human use may occur.

Ground-floor receptor points were modeled at five ft. above the ground elevation. Each additional floor was modeled at an additional 10 ft. For example, second-story receptors were modeled at 15 ft., and third story receptors were modeled at 25 ft.

The locations of the receptors are depicted on aerials in Appendix B. The alphanumeric identification for each receptor point was formulated as follows:

- Receptor points representing residential land uses (Activity Category B) are specified with an “R”.

- Receptor points representing non-residential land uses (i.e., SLUs) are specified with an “N”.

- The numeric portion of the receptor identification identifies a specific receptor point and generally increase from [SOUTH/WEST] to [NORTH/EAST].

- Multi-story receptors are identified with “a” to represent ground-floor units; “b” to represent 2nd floor units; “c” to represent 3rd floor units, etc.

Highway traffic noise levels were predicted at [NUMBER] receptors representing [NUMBER] residences and [NUMBER] non-residential SLUs potentially affected by highway traffic noise. Receptors representing noise sensitive land uses were established by a field review performed on [DATE], and verified using data from the County Property Appraiser.

If desired, describe the types of noise sensitive sites identified. Optional sentence: Within the project limits, the noise sensitive land uses along [ROAD NAME] for which there is a NAC include:

- Activity Category B (residential areas) – [NUMBER] residences

- Activity Category C –

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

- Activity Category D –

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

- Activity Category E –

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

  - [PROPERTY TYPE]: [NUMBER]

### Common Noise Environments and Noise Analysis Areas

Common Noise Environments (CNEs) are defined by FHWA as a group of receptors representing noise sensitive sites within the same activity category that are exposed to similar noise sources and levels; traffic volumes, traffic mix, and speed; and topographic features, according to 23 CFR Part 772. Generally, CNEs occur between two secondary noise sources, such as interchanges, intersections, cross-roads.

The Noise Analysis Area (NAA) is the area within and beyond the project limits that has Type I project characteristics and that requires a noise analysis. The NAA completely encompasses the area where alterations and construction will occur and may also include areas beyond the construction limits. The NAA is subdivided based on geographic, traffic, and other similar characteristics for grouping of CNEs. CNEs should be grouped into NAAs based on if a single noise barrier (or continuous noise barrier system) could be designed for all CNEs in the NAA based on reasonable assumptions. Each NAA is discussed separately in this report.

# Traffic Noise Analysis Results

The following section provides the results of the traffic noise analysis, including the identification of existing noise levels, future noise levels, and the evaluation of abatement measures at impacted noise sensitive sites.

## Existing Noise Levels

If a substantial increase in traffic noise was not identified during the PD&E phase, Section 3.1 and the reference to “existing noise levels” in the description above can be removed.

Existing noise levels are typically not evaluated during the design phase unless a substantial increase in traffic noise was previously identified during the project’s PD&E phase or a design change has occurred. If a substantial increase in traffic noise was identified during the PD&E phase (or a design change has occurred), existing noise levels, including ambient measurements (if applicable), should be re-referenced, and a recalculation of the predicted noise level change (delta) between existing and future Build conditions should be conducted during the design phase and included in this section

Existing noise level at each receptor can be determined by one of two ways, depending on if traffic noise is the dominant source of noise or not:

- Ambient Measurements – The Noise Analyst obtains ambient measurements near noise sensitive sites in the project area using a sound level meter (SLM) and assigns the resulting noise levels to appropriate nearby receptors. This methodology is used for receptors that are not near existing roadways where traffic noise is not the dominant source of noise (e.g., a new alignment); or

- TNM Modeling – The Noise Analyst models existing conditions using existing traffic values in TNM. Each receptor’s existing noise level is then predicted by TNM. This methodology is used for receptors that are found near existing roadways where traffic noise is the dominant source of noise s.

Note that only one method may be used for each receptor, but the project as a whole may utilize both methods. If both methods are used, Appendix C should notate which receptors were modeled and which were measured.

The following should be used for receptors whose noise level were determined from ambient measurements (otherwise, remove the following statement).

Existing (i.e., ambient) noise monitoring was performed to establish present-day noise levels for some/all receptors, since the [ROAD NAME] is a new alignment and traffic noise is not a prevalent noise source at some NAAs along the entire Preferred Alternative. Because this project is a new alignment, TNM modeling was not performed for Existing and No-Build scenarios where traffic noise was not a dominant source of noise.

Noise monitoring followed the procedures documented in FHWA’s Noise Measurement Handbook, Ambient noise measurements were taken at [NUMBER OF MEASUREMENT SITES] sites using a  [SOUND LEVEL METER NAME] SLM, which was calibrated using a [CALIBRATOR NAME] calibrator. Ambient noise monitoring field sheets and results are provided in Table ‎3-1. Non-highway-traffic noise sources during each monitoring event were noted to ensure a reasonable existing condition was captured (e.g., characteristics of vegetation, presence of wildlife, types of man-made noise sources, etc.). Common natural noise sources included [NOISE SOURCES SUCH AS BIRDS, OTHER WILDLIFE SUCH AS INSECTS, ETC.]. Common man-made noise sources included [MAN-MADE NOISE SOURCES SUCH AS AIRPLANES, DISTANT TRAFFIC, RESIDENTIAL EQUIPMENT SUCH AS AIR CONDITIONERS, ETC.]. Non-highway-traffic noise sources are listed in Table ‎3-1.

The locations of ambient noise monitoring sites are shown in the aerial sheets found in Appendix B. Each receptor was assigned an existing noise level based on the ambient noise level measured at the nearest monitoring location.  An individual receptor’s existing noise level is listed in Appendix C.

The following should be used for receptors whose noise level were determined from TNM (otherwise, remove the following statement).

The FHWA’s TNM was used to predict existing noise levels for receptors located near roadways and where traffic noise is dominant. As noted in Section ‎2.2, existing traffic data for [YEAR] was obtained from [SOURCE]. Appendix C lists the existing noise level for each receptor.

Table ‎3-1
Ambient Noise Monitoring (EXAMPLE)

| Monitoring Site | Event | Duration | Date | Time | Leq(h) | Average Leq(h) dB(A) | Field Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | 1 | 10 mins. | 7/6/2024 | 9:00 AM | 55.5 | 55.6 | Distant air conditioning equipment; birds chirping |
|  | 2 | 10 mins. | 7/6/2024 | 9:12 AM | 55.6 |  | Distant air conditioning equipment; birds chirping |
|  | 3 | 10 mins. | 7/6/2024 | 9:22 AM | 55.7 |  | Distant air conditioning equipment; birds chirping |
| M2 | 4 | 10 mins. | 7/6/2024 | 9:40 AM | 46.3 | 46.7 | Birds chirping |
|  | 5 | 10 mins. | 7/6/2024 | 9:51 AM | 46.7 |  | Birds chirping |
|  | 6 | 10 mins. | 7/6/2024 | 10:02 AM | 47.1 |  | Birds chirping |
| M3 | 7 | 10 mins. | 7/6/2024 | 10:35 AM | 57.2 | 58.0 | Sandhill cranes in distance |
|  | 8 | 10 mins. | 7/6/2024 | 10:45 AM | 58.0 |  | Sandhill cranes in distance |
|  | 9 | 10 mins. | 7/6/2024 | 10:55 AM | 58.8 |  | Sandhill cranes in distance |
| M4 | 10 | 10 mins. | 7/6/2024 | 11:06 AM | 52.2 | 49.9 | Plane overhead at 1 minute in; frogs in pond |
|  | 11 | 10 mins. | 7/6/2024 | 11:17 AM | 49.4 |  | Plane overhead at 5 minutes in; frogs in pond |
|  | 12 | 10 mins. | 7/6/2024 | 11:28 AM | 48.0 |  | Plane overhead at 8 minutes in; frogs in pond |

## Substantial Increase in Noise

If substantial increases are not predicted to occur, the following language, or variation thereof,  should be included.

An evaluation of substantial increases was performed during the PD&E/Design phase analysis completed in [REFERENCE YEAR OF PD&E NSR]. Each noise sensitive site was assigned an existing noise level based on a representative monitoring station’s average Leq(h) (see Section 2.8)/ TNM predicted existing noise levels and is provided in Appendix C of the [REFERENCE FOR PD&E NSR]. As shown in Appendix C of the [REFERENCE FOR PD&E NSR], substantial increases in traffic noise are not expected as a result of this project. Therefore, existing noise levels were not re-evaluated during the design phase of this project.

If substantial increases are predicted to occur, the following language should be included (otherwise, remove the following statement):

An evaluation of substantial increases was performed during the PD&E phase analysis completed in [REFERENCE YEAR OF PD&E NSR] . Each noise sensitive site was assigned an existing noise level based on a representative monitoring station’s average Leq(h) (see Section 2.8)/ TNM predicted existing noise levels and is provided in Appendix C of the [REFERENCE FOR PD&E NSR]. As shown in Appendix C of the [REFERENCE FOR PD&E NSR], substantial increases in traffic noise are expected as a result of this project. As a result, existing and predicted sound levels were re-evaluated during this design phase to verify the predicted noise level impacts and to ensure compliance with applicable noise abatement criteria. Refer to Appendix C for existing and predicted sound levels, outlining the substantial increases within the project corridor.

Receptors which were identified as having a substantial increase were re-evaluated for noise abatement and are discussed in Section ‎3.3.

## Noise Abatement Criteria Evaluation

An evaluation of noise sensitive sites that may be impacted by traffic noise levels that meet or exceed the FDOT NAC was performed. Each noise sensitive site was modeled in TNM and a future noise level was predicted. Noise abatement was considered at all noise sensitive sites found to be impacted.  Each NAA is discussed separately.

### NAA #1

This NAA is located north/south/east/west of [ROAD NAME] (Appendix B, Aerial Sheet [NUMBER]) and is composed of [NUMBER] residences and [NUMBER] SLUs. Each residential area/SLU/residential area and SLU is discussed in the sections below.

#### COMMON NOISE ENVIRONMENTS

Include the following statement (or variation thereof) for residential areas.

##### [RESIDENTIAL AREA/NEIGHBORHOOD NAME]

Residences from [ROAD NAME] to [ROAD NAME] in [SUBDIVISION AND NEIGHBORHOOD NAMES] (Appendix B, Aerial Sheet [NUMBER]) are represented by [NUMBER] receptor points representing [NUMBER] residences ([RECEPTOR IDs]). Exterior traffic noise levels are predicted to range from [NUMBER] to [NUMBER] dB(A) for the design year and do not approach, meet or exceed/approaches, meets or exceeds the FDOT NAC at the residences. Furthermore, the residences are/are not expected to have a substantial increase of 15 dB(A) over existing conditions (see Appendix C). Therefore, a noise barrier for these residences was/was not evaluated further.

Include the following statement (or variation thereof) for SLUs.

##### [SLU NAME]

[SLU NAME] (Appendix B, Aerial Sheet [NUMBER]) is located along [ROAD NAME] and was evaluated as an exterior use area/indoor use area, as no exterior area of frequent human use exists on the property. This [SLU TYPE] has a [NOISE SENSITIVE SITE TYPE; e.g, playground, active sports area, etc.]  that was evaluated for traffic noise impacts as an Activity Category C/D/E and was represented by [NUMBER] receptors ([RECEPTOR IDs]). Predicted exterior/Including a 25 dB(A) insertion loss provided by the building, predicted interior highway traffic noise levels range from [NUMBER] to [NUMBER] dB(A) for the year [YEAR]. Build condition highway traffic noise levels do/do not meet or exceed the FDOT NAC at the [SLU TYPE] exterior/interior use area. Furthermore, the facility is/is not expected to have a substantial increase of 15 dB(A) over existing conditions (see Appendix C). Therefore, a noise barrier was/was not evaluated further.

#### NOISE BARRIER EVALUATION

Consult the following Table, Table Ex. 2, to identify the proper text to include in the report. Remove Table Ex. 2 upon completion of the Report.

Table Ex. 2 - Noise Barrier Evaluation Example Text (table to be deleted)

| Scenario | Example Text |
| --- | --- |
| If there is an isolated impacted residence, include the following statement (or variation thereof) (otherwise, remove the following statement). | Although one residence is predicted to experience noise levels that approach, meet or exceed the NAC, because FDOT policy requires two impacted receptors to be benefited by a 5 dB(A) reduction in order for a barrier to be feasible, a barrier is not considered a feasible abatement measure for the isolated impacted residence. Therefore, a noise barrier for the isolated impacted residence was not evaluated. |
| If an SLU is present in the NAA and it did not pass the Preliminary Screening, the following statement (or variation thereof) should be used (otherwise, remove the following statement). | A noise barrier was evaluated following the FDOT SLU procedures outlined in Section ‎2.4.2. Following the FDOT SLU procedures, a preliminary screening was performed to identify if the person-hour usage of the [SLU TYPE] was high enough to proceed with a noise barrier analysis. The results of the preliminary screening identified that, based upon reasonable assumptions, the [SLU TYPE] does not have enough person-hours of use for a noise barrier to be considered a cost reasonable form of abatement. Therefore, a noise barrier was not evaluated further for the SLU. |
| If an SLU is present in the NAA, the following statement (or variation thereof) should be used to document the Benefited Equivalent Residence calculation for the SLU (otherwise, remove the following statement). | A noise barrier was evaluated following the FDOT SLU procedures outlined in Section ‎2.4.2. The property owner was contacted to obtain usage information and was/but was not able to provide usage information. Based on the information provided by the SLU owner(s)/coordination with the FDOT District Noise Specialist, daily usage of the [SLU TYPE] was assumed to be [NUMBER] people per day for [NUMBER] hours per day. Based on the methodology described in FDOT’s Methodology to Evaluate Highway Traffic Noise at Special Land Uses, the Benefited Equivalent Residences (BER) for this SLU is [NUMBER] residences (see Appendix E). |
| If an isolated SLU is present in the NAA and it has a Benefited Equivalent Residence of the SLU is less than 2, the following statement (or variation thereof) should be used (otherwise, remove the following statement). | Based on the methodology described in FDOT’s Methodology to Evaluate Highway Traffic Noise at Special Land Uses, an SLU is considered ineligible for a barrier if it is an isolated SLU (i.e., not adjacent to another impacted SLU or impacted residential area) with a BER of less than two. This is based upon the FDOT feasibility requirement that two impacted residents must receive a benefit for a noise barrier to be considered feasible. Based upon usage information provided by the SLU owner(s)/identified through coordination with the FDOT District Noise Specialist, this SLU is isolated and has a BER of [NUMBER]. Therefore, the SLU is ineligible for a noise barrier, and a noise barrier was not evaluated further. |
| If a ROW noise barrier was not evaluated, include the following statement (or variation thereof) (otherwise, remove the following statement). | A ROW noise barrier was not evaluated, due to [REASON(S) WHY A ROW MOUNTED NOISE BARRIER WAS NOT EVALUATED.]. |
| If a ROW noise barrier was evaluated, include the following statement (or variation thereof) (otherwise, remove the following statement). | A ROW noise barrier was evaluated at a maximum height of 22 ft., stepping down in 2 ft. increments. The noise barrier evaluation, shown in Table ‎3-2, found that ROW noise barriers ranging from [NUMBER] to [NUMBER] ft. in height would/would not provide a benefit to at least two impacted residences/ERs/residences and ERs, would/would not meet the NRDG, and it is/is not cost reasonable. The reason for not being considered cost reasonable is the low density of homes evaluated relative to the size and cost of a noise barrier. |
| If a shoulder and/or structure mounted barrier was evaluated, include the following statement (or variation thereof) (otherwise, remove the following statement). | A shoulder noise barrier was evaluated at a maximum height of 14 ft., stepping down in 2 ft. increments (and limited to 8 ft. where on structure),. The noise barrier evaluation, shown in Table ‎3-2, found that a shoulder noise barrier ranging from [NUMBER] to [NUMBER] ft. in height would/would not provide a benefit to at least two impacted receptors, would/would not meet the NRDG, and it was/was not cost reasonable. The reason for not being considered cost reasonable is the low density of homes evaluated relative to the size and cost of a noise barrier. |
| If a combination ROW and Shoulder and/or structure mounted noise barrier was evaluated, include the following statement (or variation thereof) (otherwise, remove the following statement). | A combination ROW and shoulder-mounted/ROW, shoulder and structure-mounted/ROW and structure-mounted noise barrier was evaluated at heights ranging from [NUMBER] to [NUMBER] ft. in height. The noise barrier evaluation, shown in Table ‎3-2, found that a shoulder noise barrier ranging from [NUMBER] to [NUMBER] ft. in height would/would not provide a benefit to at least two impacted receptors and would/would not meet the NRDG, and it was/was not cost reasonable. The reason for not being considered cost reasonable is the low density of homes evaluated relative to the size and cost of a noise barrier. |
| The following statement (or variation thereof) should be included when a noise barrier is found to be not reasonable or feasible (otherwise, remove the following statement). | A feasible and reasonable noise barrier was not identified for this NAA. Therefore, a noise barrier for this NAA is not recommended. |
| The following statement (or variation thereof) should be included when a noise barrier is found to be reasonable but not feasible (otherwise remove the following statement) | Noise barriers were evaluated for feasibility and reasonableness in accordance with FDOT and FHWA criteria. While the proposed barriers were determined to be reasonable based off cost-effectiveness, they were found to be not feasible due to [REASONS WHY THE BARRIER WAS NOT FEASIBLE]. |
| The following statement (or variation thereof) should be included when a noise barrier is found to be feasible but not reasonable (otherwise remove the following statement) | Noise barriers were evaluated for feasibility and reasonableness in accordance with FDOT and FHWA criteria. While the proposed barriers were determined to be feasible based off cost-effectiveness and public input, they were found to be not reasonable due to [REASONS WHY THE BARRIER WAS NOT REASONABLE]. |
| The following statement (or variation thereof) should be included when a noise barrier is found feasible and reasonable (otherwise, remove the following statement). | As shown in Table ‎3-2, Barrier ID No. [NUMBER], a [NUMBER] ft. ROW-mounted/shoulder-mounted/ROW and shoulder-mounted noise barrier, is feasible and reasonable and provides the most benefit to the residences/SLU/residences and SLU/residences and SLUs. Therefore, a noise barrier for the residences/SLU/residences and SLU/residences and SLUs is recommended for further evaluation in the design phase. |
| The following statement (or variation thereof) should be included when the engineering review for the proposed barrier passed (otherwise, remove the following statement) | The proposed barrier design for [INSERT BARRIER ID] has been reviewed and meets all applicable engineering standards and performance criteria. It has been deemed structurally sound, effective in noise attenuation, and suitable for implementation of [INSERT NOISE BARRIER LOCATION DESCRIPTIONS]. |
| The following statement (or variation thereof) should be included when the engineering review for the proposed barrier did not pass (otherwise, remove the following statement) | The proposed noise barrier design for [INSERT BARRIER ID] has been reviewed and does not meet the required engineering standards due to [REASONS WHY THE BARRIER WAS NOT FEASIBLE]. Therefore, the noise barrier is not recommended for further evaluation. |

Table ‎3-2
NAA #1: Noise Barrier Evaluation Results

.

| Evaluated Noise Barrier System ID | Barrier Location | Approximate Barrier XY/ Stationing Extent | Barrier Height1 (ft.) | Barrier Length2 (ft.) | Number of Residences3 Receiving a 5-5.9 dB(A) Reduction | Number of Residences3 Receiving a 6-6.9 dB(A) Reduction | Number of Residences3 Receiving a > 7 dB(A) Reduction | Number of Impacted and Benefited Residences3,4 | Total Number of Benefited Residences3 | Average Reduction dB(A) at Residences3 | Maximum Reduction dB(A)4 | Total Estimated Cost5 ($) | Cost per Benefited Residence3 ($)6 | Cost Reasonable? | Result/Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Impacts: | 109 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ROW |  | 8 | 2,000 | 0 | 0 | 0 | 0 | 0 | 0.5 | 0.8 | N/A | N/A | N/A | NRDG not met; Benefit requirement not met |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 2 | ROW |  | 10 | 2,000 | 0 | 0 | 0 | 0 | 0 | 1.5 | 4.5 | N/A | N/A | N/A | NRDG not met; Benefit requirement not met |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | ROW |  | 12 | 2,000 | 6 | 0 | 0 | 6 | 6 | 3.5 | 5.3 | N/A | N/A | N/A | NRDG not met |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 4 | ROW |  | 14 | 2,000 | 18 | 0 | 0 | 16 | 18 | 4.5 | 5.5 | N/A | N/A | N/A | NRDG not met |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 5 | ROW |  | 16 | 2,000 | 50 | 2 | 0 | 41 | 52 | 5.5 | 6.3 | N/A | N/A | N/A | NRDG not met |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 6 | ROW |  | 18 | 2,000 | 60 | 20 | 2 | 61 | 82 | 6.5 | 7.5 | $1,600,000 | $19,512 | Yes | Meets criteria; Not recommended |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 7 | ROW |  | 20 | 2,000 | 61 | 70 | 12 | 102 | 143 | 7.5 | 8.0 | $1,760,000 | $12,308 | Yes | Meets criteria; Not recommended |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |
| 8 | ROW |  | 22 | 2,000 | 62 | 80 | 20 | 111 | 162 | 8.5 | 9.0 | $1,920,000 | $11,852 | Yes | Recommended |
|  | Shoulder |  | 8 | 500 |  |  |  |  |  |  |  |  |  |  |  |

NRDG = Noise Reduction Design Goal

1 Full height is for the length indicated.

2Barrier length refers to the total length at the ROW, Shoulder, or on Structure. If a shoulder noise barrier location is indicated, the length of vertical height tapers at the shoulder barrier’s terminus (See FDOT Standard Plans) would be in addition to the length indicated.

3Residence and/or Equivalent Residence.
4If total Impacted and Benefited Residences/Equivalent Residences is less than 2, the noise barrier is not considered feasible. 
4If Maximum Reduction is less than 7 dB(A), the Noise Reduction Design Goal (NRDG) requirement is not met.

5Assumes $40 per square foot.

6 The FDOT cost-reasonable limit is $64,000 per benefited equivalent residence.

Note: This table can be adjusted for project-specific needs.

# Outdoor Advertising

If noise barriers are not recommended and no field review was required outdoor advertising conflicts, consult with the District if this section should be removed. If this section is desired, include the following statement, or variation thereof.

No noise barriers are not recommended for this project, as the traffic noise analysis did not identify any locations where noise abatement would be feasible and reasonable. As a result, a review of outdoor advertising locations was not conducted for this project.

If noise barriers are recommended and the review of outdoor advertising locations found no conflicts, the following statement (or variation thereof) should be included.

A field review was conducted to determine if any outdoor advertising signs would be in conflict with [INSERT NOISE BARRIER ID(S)]. The field review determined that there are no outdoor advertising signs that would be blocked visually by the barrier.

If noise barriers are recommended and the review of outdoor advertising locations found conflicts, the following statement (or variation thereof) should be included.

A field review was conducted to determine if any outdoor advertising signs would be in conflict with [INSERT NOISE BARRIER ID(S)]. The field review determined that there are outdoor advertising signs that would be blocked visually by the barrier at the following locations:

- [INSERT NOISE BARRIER ID(S)]

- [INSERT NOISE BARRIER ID(S)]

- [INSERT NOISE BARRIER ID(S)]

# Conclusions

## Impacted Noise Sensitive Sites

If there are no impacted receptors within the project, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

Noise levels were predicted at [NUMBER] receptor points representing [NUMBER] residences and [NUMBER] SLUs. For the year [YEAR] Build condition, noise levels are not predicted to meet or exceed the FDOT NAC at any residence or SLUs within the project limits. Based on the noise analyses performed to date, there appear to be no impacted areas within the project that require abatement consideration.

If there are impacted receptors within the project, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

Noise levels were predicted at [NUMBER] receptor points representing [NUMBER] residences and [NUMBER] SLUs. For the year [YEAR] Build condition, noise levels are predicted to meet or exceed the FDOT NAC at [NUMBER] residences and [NUMBER] SLUs within the project limits. These impacted sites were evaluated to determine the feasibility and reasonableness of providing barriers to reduce traffic noise. Additionally, a substantial increase of 15 dB(A) is/is not predicted to occur at any residence or SLU (shown in Appendix C).

## Recommended Noise Barriers

If no impacts were identified in the project, this section can be removed, or the following statement (or variation thereof) should be used (otherwise, remove the following statement):

No impacts were identified as a result of this project. Therefore, noise barrier consideration was not required.

If noise barriers were unable to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The noise barrier evaluation identified that noise barriers cannot meet FDOT's acoustic and/or reasonableness criteria for [NUMBER] impacted residences and [NUMBER] impacted SLUs. Based on the noise analyses performed to date, there are no feasible and reasonable solutions available to mitigate the noise impacts at the locations identified in Section ‎3.3. A list of NAAs not qualifying for noise abatement is found in Table ‎5-2.

If noise barriers were able to meet feasible and reasonable requirements, the following statement (or variation thereof) should be used (otherwise, remove the following statement):

The noise barrier evaluation identified that noise barriers are a feasible and reasonable form of abatement and could potentially meet FDOT’s acoustic and reasonableness criteria at [NUMBER] of the [NUMBER] impacted residences ([NAA NAMES]), and at  [NUMBER] of the [NUMBER] impacted SLUs ([SLU NAMES/DESCRIPTIONS]). Noise barriers for the NAAs listed in Table ‎5-1 and have been included in design plans, shown in [APPENDIX B] and [AERIAL SHEET X]. A list of NAAs not qualifying for noise abatement is found in Table ‎5-2.

Table ‎5-1
Potentially Feasible and Reasonable Noise Barriers

| Barrier ID | Noise Analysis Area | Barrier Location | Approximate Barrier XY/ Stationing Extent | Barrier Height1 (ft.) | Barrier Length2 (ft.) | Total Estimated Cost ($)3 | Impacted Residences4 | Impacted and Benefited Residences4 | Benefited and Not Impacted Residences4 | Total Benefited Residences4 | Maximum Reduction dB(A)5 | Cost per Benefited Residence4 ($)6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NAA # 1: Residences in ABC neighborhood | ROW |  | 22 | 1,500 | $1,512,000 | 45 | 45 | 3 | 48 | 8.3 | $31,500 |
|  |  | Shoulder |  | 8 | 600 |  |  |  |  |  |  |  |
| 2 | NAA #2: ABC Apartments and XYZ Church | ROW |  | 22 | 2,840 | $2,499,000 | 70 | 70 | 5 | 75 | 8.7 | $33,320 |
| 3 | NAA #3: ABC Park | ROW |  | 22 | 1,500 | $1,320,000 | 20 | 20 | 1 | 21 | 7.9 | $62,857 |
| Totals | ------ | ------ | ------ | ------ | ------ | $5,331,200 | ------ | ------ | ------ | ------ | ------ | ------ |

1 Full height is for the length indicated.

2Barrier length refers to the total length at the ROW, Shoulder, or on Structure. If a shoulder noise barrier location is indicated, the length of vertical height tapers at the shoulder barrier’s terminus (See FDOT Standard Plans) would be in addition to the length indicated.

3Assumes $40 per square foot. 
4Residence and/or Equivalent Residence.

5If Maximum Reduction is less than 7 dB(A), the NRDG requirement is not met.

6The FDOT cost-reasonable limit is $64,000 per benefited equivalent residence.
Note: This table can be adjusted for project-specific needs.

Table ‎5-2 Noise Analysis Areas Not Qualifying for Abatement

| Noise Analysis Area | # Impacted Equivalent Residences | Barrier Height1 (ft.) | Barrier Length2 (ft.) | Location | Meets Feasibility Requirement? | Meets the NRDG? | Barrier System Cost-Reasonable? | Probable Cause for Not Meeting Feasibility and Reasonableness Criteria? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAA # 1: Residences in XYZ neighborhood | 3 | 22 | 1,500 | ROW | Yes | Yes | No | Low residential density |
| NAA #2: XYZ Apartments and ABC Church | 7 | 22 | 980 | ROW | No | No | No | Access roads reduced the acoustic effectiveness of barriers evaluated. |
| NAA #3: XYZ Park | 2 | 22 | 1,890 | ROW | Yes | Yes | No | Lack of required person-hour usage |

1 Full height is for the length indicated.

2Barrier length refers to the unoptimized length. 
Note: This table can be adjusted for project-specific needs.

# Construction Noise and Vibration

Based on the existing land use within the limits of this project, construction of the proposed roadway improvements will/will not result in temporary noise and/or vibration impact. If noise sensitive land uses develop adjacent to the roadway prior to construction, additional impacts could result. It is anticipated that the application of the FDOT Standard Specifications for Road and Bridge Construction will minimize or eliminate most of the potential construction noise and vibration impacts. However, should unanticipated noise or vibration issues arise during the construction process, the Project Manager, in concert with the District Noise Specialist and the Contractor, will investigate additional methods of controlling these impacts.

# Community Coordination

## Public Involvement

The FDOT has conducted an extensive public involvement program over the life of the project. This includes the PD&E phase alternatives workshop, public meetings, Request for Proposal phase and the creation of various project advisory groups made up of representatives from the community, businesses, local government agencies and nearby cultural resources. Specific information regarding the dates and purpose of previously held or planned meetings can be obtained from the FDOT District [INSERT DISTRICT] Public Involvement Office. Meeting minutes are also included in the SWEPT project file.

Coordination with local agencies, officials and the general public is ongoing/complete. The public has had the opportunity to comment on the proposed project at public meetings and other outreach efforts. The following outreach efforts have occurred/will occur:

- [PUBLIC OUTREACH EVENT NAME, DATE, TIME AND PLACE]

  - Public comments: [NO COMMENTS RECEIVED/INSERT COMMENTS RECEIVED].

- [PUBLIC OUTREACH EVENT NAME, DATE, TIME AND PLACE]

  - Public comments: [NO COMMENTS RECEIVED/INSERT COMMENTS RECEIVED].

- [PUBLIC OUTREACH EVENT NAME, DATE, TIME AND PLACE]

  - Public comments: [NO COMMENTS RECEIVED/INSERT COMMENTS RECEIVED].

### Noise Barrier Surveys

If barriers are recommended, and survey packages have been mailed to property owners, include the following text. If noise barrier surveys were not performed, Section ‎7.1.1 should be removed.

To gauge and document the impacted public’s support for, or opposition to, the proposed noise barrier, a noise barrier survey was conducted. A noise barrier survey package was mailed via [INSERT MAILING AGENCY] to the registered property owner and for instances where the mailing address for the property differed from the physical address, the survey package was mailed to both the mailing address and the physical address. If there is an associated Homeowners Association (HOA) that was consulted during the public involvement process, please include the following text. Survey packages were also sent to the [ENTER NAME OF COMMUNITY HOA] If there was coordination with property owners for the same barrier structure, but outside of the HOA jurisdiction, use the following text and to the registered owner of the [INSERT PARCEL AND/OR RESIDENCE INFORMATION]. Survey packages consisted of several items; [OUTLINE WHAT WAS INCLUDED IN THE SURVEY PACKAGES]  The survey served to document the official position/viewpoint of the property owner/renter regarding the proposed noise barrier. A self-addressed and stamped envelope was also provided to facilitate the return of the completed surveys.

A total of [NUMBER OF] survey packages were mailed [DATE], and all deliveries were confirmed by [DATE]. A response deadline of [DATE], was stated on the survey cover letter and survey form. A total of [NUMBER OF] surveys were returned, with all [NUMBER OF] in favor of noise barrier construction. The majority of those supporting the noise barrier chose the [INSERT TEXTURE AND COLOR CHOSEN] for the residential side of the noise barrier.

[Appendix X] contains information supporting the noise barrier-specific public involvement for the project, including noise barrier survey package items, community workshop sign-in sheets, barrier survey results, submitted barrier survey forms, and correspondence with [INSERT CORRESPONDING AGENCY, I.E. HILLSBOROUGH COUNTY].

# REFERENCES

23 CFR Part 772, "Procedures for Abatement of Highway Traffic Noise and Construction Noise", Federal Register, Vol. 75, No. 133, Tuesday, July 13, 2010; pages 39834‐39839.

Federal Highway Administration Report FHWA‐HEP‐10‐025, “Highway Traffic Noise: Analysis and Abatement Guidance”, June 2010 (revised December 2011); 76 pages.

Federal Highway Administration, “FHWA Traffic Noise Model User’s Guide (Version 2.5 Addendum)”, April 2004; 44 pages.

Federal Highway Administration Report Number FHWA‐HEP-18-065, “Noise Measurement Handbook”, June 2018; 205 pages.

Federal Highway Administration Report FHWA‐HEP‐06‐015, “FHWA Highway Construction Noise Handbook:  Final Report”.  August 2006; 185 pages.

Federal Highway Administration. “Consideration of Existing Noise Barrier in a Type I Noise Analysis FHWA-HEP-12-051.” https://www.fhwa.dot.gov/ENVIRONMENT/noise/noise_barriers/abatement/existing.cfm

Florida Department of Transportation. "Highway Traffic Noise", Part 2, Chapter 18. Project Development and Environment Manual, Florida Department of Transportation, Tallahassee, May 1, 2026.

Florida Department of Transportation. "Methodology to Evaluate Existing Noise Barriers, Florida Department of Transportation, Tallahassee, February 2025.

Florida Department of Transportation. "Methodology to Evaluate Highway Traffic Noise at Special Land Use Locations, Florida Department of Transportation, Tallahassee, July 2025.

Florida Department of Transportation. "Traffic Noise Modeling and Analysis Practitioners’ Handbook, Florida Department of Transportation, Tallahassee, September 2025.

Florida Department of Transportation Design Manual Volume 1, Chapter 264, “Noise Walls and Perimeter Walls”, January 2025.

Florida Department of Transportation "Standard Specifications for Road and Bridge Construction", January 2025.

Florida Department of Transportation, “2023 FDOT Quality/Level of Service Handbook”; Tallahassee, Florida; January 2023.

APPENDICES

APPENDIX A Traffic Data

APPENDIX B Project Aerials

APPENDIX C Predicted Noise Levels

APPENDIX D Model Validation Field Sheet

APPENDIX E SLU Equivalent Residence Calculation Worksheets

APPENDIX F Engineering Reviews

APPENDIX G Noise Barrier Survey Package

# APPENDIX A Traffic Data

# APPENDIX B Project Aerials

The following Appendix should contain Project Aerials of the corridor. The project aerials are suggested to depict noise-specific features, including but not limited to:

- Noise sensitive sites

  - Impact status

  - Benefit status (if applicable)

- SLU names (or CNE, NAA IDs/names)

- Validation measurement locations

- Ambient measurement locations (if applicable)

- Property acquisitions

- Outdoor advertising locations

- Neighborhood walls

- Existing noise barriers

- Parcel/property lines

- Proposed Design

  - Roadway

  - ROW

  - Stationing

- Recommended Feasible and Reasonable Noise Barriers

Example graphics are shown on the following pages. Note that these examples may be changed according to project and District needs.

EXAMPLE

\

EXAMPLE

EXAMPLE

EXAMPLE

EXAMPLE

# APPENDIX C Predicted Noise Levels

| Common Noise Environment Name | Aerial Sheet Number [OPTIONAL] | Receptor ID | Activity Category | Property Type [OPTIONAL] | Number of Residents Represented | [YEAR] Existing Condition dB(A) [OPTIONAL] | [YEAR] No-Build Condition dB(A) | [YEAR] Build Condition dB(A) | Difference Between Existing and Build dB(A) | Impacted |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Substantial Increase | NAC |
| Example: Residences from ABC Street to XYZ Road | 1 | EB1 | B | Residence | 1 | 62.5 | 64.2 | 66.5 | 4.0 | No | Yes |
| Example: ABC School | 2 | R8 | C | School (exterior) | 0.5 | 63.8 | 64.9 | 68.8 | 5.0 | No | Yes |
|  |  |  |  |  | EXAMPLE | EXAMPLE |  |  |  |  |  |

# APPENDIX D Model Validation Field Sheets

If validation was not performed, this Appendix should be removed.

Table D-1 
Measured Noise Levels and Traffic Data for Validation [EXAMPLE]

| Project |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |  |  |  |
| Name of Observer(s) |  |  |  |  |  |  |  |  |  |  |
| Wind Speed |  |  |  |  |  |  |  |  |  |  |
| Temperature |  |  |  |  |  |  |  |  |  |  |
| Humidity |  |  |  |  |  |  |  |  |  |  |
| Cloud Cover |  |  |  |  |  |  |  |  |  |  |
| Feet from Edge of Pavement |  |  |  |  |  |  |  |  |  |  |
| X, Y, Z |  |  |  |  |  |  |  |  |  |  |
| Sound Level Meter Model/Serial # |  |  |  |  |  |  |  |  |  |  |
| Calibration Performed |  |  |  |  |  |  |  |  |  |  |
| Site # |  |  |  |  |  |  |  |  |  |  |
| Repetition | File Name | Start Time | Roadway Direction | Cars | Medium Trucks | Heavy Trucks | Motor- cycles | Buses | Speed (mph) | Sound Level dB(A) |
| 1 |  |  |  |  | EXAMPLE |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| Repetition | Notes |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |

# APPENDIX E SLU Equivalent Residence Calculation Worksheet

If SLU Equivalent Residence Identification was not performed, this Appendix should be removed.

# APPENDIX F Engineering Review

If an Engineering Review was not performed, this Appendix should be removed.

For each noise barrier that an engineering review was completed, create a separate Appendix with the Engineering Review Results to aid in organization. Each Appendix may contain a copy of the Engineering Review form, design plans, or other details supporting the noise barrier review.

Noise Barrier Engineering Review Form

[PROJECT NAME]  [FM NUMBER]

[INSERT COUNTY] County, Florida

Noise Barrier #:___________________________________________________

Date Reviewed: ___________________________________________________

Reviewed By: _____________________________________________________

| Topic | Details |
| --- | --- |
| Location |  |
| Length |  |
| Height | EXAMPLE |
| Estimated Cost (@ $40/ sq. ft.) |  |
| Design/Constructability Issues |  |
| Drainage Issues |  |
| Utility Issues |  |
| Safety Issues |  |
| Maintenance Issues |  |
| ROW Acquisition Issues |  |
| Legal Issues |  |
| Outdoor Advertising Issues |  |
| Are any of the above issues severe enough so that a noise barrier cannot be constructed at this location? If so, please explain in detail. |  |

# APPENDIX G Noise Barrier Survey Package

If public surveys were not performed, this Appendix should be removed. The following is an example of a noise barrier survey. The package can include the notification letter, barrier survey, and results.

[INSERT DATE]

Dear [INSERT NAME],

The Florida Department of Transportation (FDOT) seeks your input regarding the proposed construction of a noise barrier adjacent to your community.

The Florida Department of Transportation (FDOT) is preparing to make improvements to [INSERT ROADWAY NAME AND PROJECT LIMITS] in [INSERT COUNTY] County. As part of the proposed improvements, a traffic noise evaluation has been performed to identify areas that could benefit from noise abatement. The results of the study found that a noise barrier could potentially provide a reduction to traffic noise levels in the area.

EXAMPLE

The FDOT is considering construction of [INSERT NOISE BARRIER NUMBER FROM THE NOISE STUDY REPORT] documented in [INSERT REFERENCE TO NOISE STUDY REPORT]. This [INSERT NOISE BARRIER TYPE] noise barrier is proposed along [INSERT ROADWAY NAME] of [INSERT HEIGHT] feet in height adjacent to the homes found in [INSERT NEIGHBORHOOD NAME]. A map of this potential noise barrier can be found in [INSERT FIGURE REFERENCE].

The FDOT is seeking input from you, as a benefited property owner/renter, to document your support for, or opposition to, the construction of the noise barrier. This survey form may not be duplicated to share with other property owners to solicit their response.

SURVEY PROCESS

Before giving further consideration to a noise barrier at the designated location, the FDOT requires documented support in favor of the barrier from benefited property owners. The FDOT will commit to the construction of the proposed noise barrier if a numerical majority of survey responses received during the response period indicate support for the barrier. The FDOT may determine that the barrier should not be constructed if the numerical majority of responses indicate a lack of support, or if the overall number of responses received is low. Therefore, your input is an important part of the decision-making process.

Enclosed is a Noise Barrier Survey to document your opinion. Responses to this survey will be used in the decision-making process and all input will be incorporated into the project file.

NOISE BARRIER INFORMATION

A noise barrier is a permanent structure primarily designed to reduce traffic noise from a roadway.  However, there are other advantages and disadvantages to having a noise barrier adjacent to your property.

Advantages of Noise Barriers

- Noise barriers, when designed at the proper height and length, can provide an audible reduction in highway traffic noise to residences directly behind the barrier.

- Noise barriers provide privacy from passing motorists.

- Noise barriers can affect sunlight by casting a fairly large shadow or permanently shade some areas, which may be desired to provide shelter from summer heat.

- Noise barriers act as visual barriers, potentially blocking undesirable views for residents.

EXAMPLE

Noise barriers can be designed to be aesthetically pleasing from both the highway and property owner side of the barrier.

Disadvantages of Noise Barriers

- Noise barriers can affect sunlight by casting a fairly large shadow or permanently shade some areas, which may adversely affect or cause the decline of nearby ornamental vegetation, trees, and shrubs

- Noise barriers act as visual barriers –

  - May block desirable views for residents.

  - May block the view of commercial or landmark signs from passerby users of the roadway, limiting advertisement.

- Noise barriers may present a public safety concern because the view of police and other safety patrols traversing the highway will be blocked, and noise barriers can create nighttime shadow areas.

- Noise barriers can obstruct breezes and other air flow.

- Noise barriers can attract graffiti.

If you have any questions or would like more information, please contact [INSERT DISTRICT CONTACT NAME] at (###)###-####, or by email at [INSERT NAME]@dot.state.fl.us.

Sincerely,

[INSERT DISTRICT CONTACT NAME]

# [INSERT NEIGHBORHOOD] Noise Barrier Survey Form

[INSERT PROJECT NAME]

[INSERT COUNTY] County, FL

Financial Project Number: [INSERT FPID]

PROPERTY OWNER

[INSERT PROPERTY OWNER NAME]

[INSERT PROPERTY OWNER ADDRESS]

EXAMPLE

[INSERT PROPERTY OWNER PARCEL NO.]

Please mark the box next to your desired response:

☐ Yes: I am in favor of the FDOT building [INSERT NOISE BARRIER #] to reduce traffic noise from the proposed project

☐ No: I am not in favor of the FDOT building [INSERT NOISE BARRIER #] and acknowledge that I may be affected by future traffic noise levels because of the proposed project.

Additional Comments: __________________________________________________________________

SURVEY PARTICIPANT INFORMATION

Name of person completing this Survey: ____________________________________________________

Status of person completing this Survey (check one):

☐ Property Owner                ☐ Legal Representative                  ☐ Renter

Mailing Address: ______________________________________________________________________

Telephone number:_____________________________Email: __________________________________

Signature: ____________________________________

Please complete and return this survey form to the Florida Department of Transportation no later than [INSERT DATE] in the enclosed stamped envelope or electronically to:

[INSERT NAME]@dot.state.fl.us