Preconstruction Notes

Broward County		Page 2

Palm Beach County		Page 3

Martin County		Page 3

St. Lucie County		Page 4

Indian River County		Page 4

City of Boca Raton		Page 4

City of Fort Pierce		Page 5

City of Port St. Lucie	Page 6

Town of Palm Beach	Page 7

Broward County Pre-Construction Requirements

Thirty (30) days prior to the beginning of construction notify the Special Projects Coordinator at BCTED to arrange a Pre-Construction/School Safety meeting at 954-847-2671

Deliver all removed traffic signal equipment, except concrete strain poles, to BCTED located at:

2300 West Commercial Blvd, Fort Lauderdale, FL 33309

Contact BCTED Seven (7) full working days prior to delivery at 954-847-2600

If there are Copper Interconnect Cables within project limits or within 1500 feet of project limits, contact telecommunications@broward.org or 954-847-2761

When communications to an intersection must be disrupted to perform work, provide two day advance notice to BCTED via email to telecommunications@broward.org including contact person, telephone number, purpose, location and duration. The disruption shall last for no more than 3 consecutive business days.  Where possible, the disruption shall be during off peak hours beginning at 9:00am and ending at 3:00pm.

If there are Fiber Optic Cables within project limits or within 1500 feet of project limits, contact telecommunications@browar.org or 954-847-2745

When communications to an intersection must be disrupted to perform work, provide two day advance notice to BCTED via email to telecommunications@broward.org including contact person, telephone number, purpose, location and duration. The disruption shall last for no more than 3 consecutive business days.  Where possible, the disruption shall be during off peak hours beginning at 9:00am and ending at 3:00pm.

Controller cabinet shall be oriented such that cabinet door opens away from intersection.

Palm Beach County – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT THE PBC TRAFFIC OPERATIONS SUPERINTENDENT, (561) 233-3900, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE PALM BEACH COUNTY TRAFFIC OPERATIONS LOCATED AT: 2555 VISTA PARKWAY, WEST PALM BEACH, FL. 33411, (561) 233-3900.  THE CONTRACTOR SHALL CONTACT PBCTED (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, AT THE TIME OF SIGNAL CONDITIONAL ACCEPTANCE INSPECTION BY THE MAINTAINING AGENCY.

Martin County – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT MARTIN COUNTY TRAFFIC ENGINEERING DIVISION LOCATED AT:

2151 SE AVIATION WAY, STUART, FL. 34996

(772) 288-5528, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, AT THE TIME OF SIGNAL CONDITIONAL ACCEPTANCE INSPECTION BY THE MAINTAINING AGENCY.

THE CONTRACTOR SHALL DELIVER REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE MARTIN COUNTY TRAFFIC ENGINEERING DIVISION LOCATED AT: 2151 SE AVIATION WAY , STUART, FL. 34996 (772) 288-5528.  THE CONTRACTOR SHALL CONTACT MCTED (7) FULL WORKING DAYS PRIOR TO DELIVERY FOR DIRECTION AS TO WHAT EQUIPMENT IS TO BE DELIVERED.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL.

Indian River County – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT INDIAN RIVER COUNTY TRAFFIC ENGINEERING DIVISION LOCATED AT: 4548 41TH STREET, VERO BEACH, FL. 32967 (772) 226-3461, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE INDIAN RIVER COUNTY TRAFFIC ENGINEERING DIVISION LOCATED AT: 4548 41TH STREET, VERO BEACH, FL. 32967 (772) 226-3461.  THE CONTRACTOR SHALL CONTACT IRCTED (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, AT THE TIME OF SIGNAL CONDITIONAL ACCEPTANCE INSPECTION BY THE MAINTAINING AGENCY.

City of Boca Raton – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT THE CITY OF BOCA RATON TRAFFIC OPERATIONS ENGINEER, (561) 416-3374, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE CITY OF BOCA RATON TRAFFIC DIVISION LOCATED AT: 2500 NW 1St AVENUE, BOCA RATON, FL. 33431 (561) 416-3364.  THE CONTRACTOR SHALL CONTACT THE CITY (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL. ALSO INCLUDES THE COST OF REMOVING.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, SEVEN (7) DAYS PRIOR TO THE SIGNAL INSPECTION BY THE MAINTAINING AGENCY.  THE CONTACTOR SHALL BE REQUIRED TO BECOME FAMILIAR WITH AND COMPLY WITH CITY’S INSPECTION PROCEDURE.

City of Fort Pierce – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT THE CITY OF FT. PIERCE TRAFFIC OPERATIONS ENGINEER, (772) 467-3774, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE CITY OF FT. PIERCE PUBLIC WORKS COMPOUND LOCATED AT: 52 SAVANNAH ROAD, FT. PIERCE, FL. 34950 (772) 467-3774.  THE CONTRACTOR SHALL CONTACT THE CITY (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL.  ALSO INCLUDES THE COST OF REMOVING, CAREFULLY STOCKPILING, AND RE-INSTALLING GPS PRE-EMPTION EQUIPMENT ONTO NEW SIGNAL SYSTEM.  CONTRACTOR SHALL ENSURE GPS PRE-EMPTION IS FULLY FUNCTIONING AS REQUIRED BY THE CITY.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, SEVEN (7) DAYS PRIOR TO THE SIGNAL INSPECTION BY THE MAINTAINING AGENCY.  THE CONTRACTOR SHALL BE REQUIRED TO BECOME FAMILIAR WITH AND COMPLY WITH CITY OF FORT PIERCE INSPECTION PROCEDURE.

City of Port Saint Lucie Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT CITY OF PORT ST. LUCIE TRAFFIC OPERATIONS MANAGER AT (772) 871-5182, TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL MAKE ALL VIDEO DETECTORS INSTALLED AS PART OF THE PROJECT FULLY OPERATIONAL IN ACCORDANCE WITH THEIR ASSOCIATED ISOLATED INTERSECTION SIGNAL TIMING CHART WITHIN 24 HOURS OF THEIR INSTALLATION.

THE CONTRACTOR SHALL NOTIFY THE APPROPRIATE UTILITY COMPANY TWO (2) FULL WORKING DAYS IN ADVANCE OF ANY EXCAVATION INVOLVING ITS UTILITIES SO THAT A COMPANY REPRESENTATIVE CAN BE PRESENT.

ALL STANDARD PULL BOXES SHALL BE STAMPED "TRAFFIC SIGNAL" ON THE COVER.  ALL LARGE PULL BOXES SHALL BE STAMPED “FIBER OPTICS” ON THE COVER.

THE CONTRACTOR SHALL PROVIDE ALL NECESSARY EQUIPMENT INCLUDING A TWO-MAN BUCKET OR PLATFORM LIFT TRUCK FOR THE USE BY THE MAINTAINING AGENCY DURING INSPECTION OF TRAFFIC SIGNALS.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE CITY OF PORT ST. LUCIE TRAFFIC ENGINEERING LOCATED AT: 821 SW DWYER AVE, PORT ST. LUCIE, FL. 34983 (772) 344-4360.  THE CONTRACTOR SHALL CONTACT THE CITY (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, AT THE TIME OF SIGNAL CONDITIONAL ACCEPTANCE INSPECTION BY THE MAINTAINING AGENCY.

Town of Palm Beach – Signalization Requirements

PRIOR TO BEGINNING CONSTRUCTION, CONTRACTOR SHALL CONTACT TOWN OF PALM BEACH TRAFFIC SIGNALS SECTION AT (561) 838-5440 TO INFORM THEM OF CONSTRUCTION OPERATIONS.

THE CONTRACTOR SHALL NOTIFY THE TOWN TWO (2) FULL WORKING DAYS IN ADVANCE OF ANY EXCAVATION INVOLVING ITS UTILITIES SO THAT A TOWN REPRESENTATIVE CAN BE PRESENT. THE LOCATIONS OF THE UTILITIES SHOWN IN THE PLANS ARE APPROXIMATE ONLY. THE EXACT LOCATIONS SHALL BE DETERMINED BY THE CONTRACTOR DURING CONSTRUCTION.

THE CONTRACTOR SHALL PROVIDE FOUR COPIES OF MARKED-UP (AS-BUILT) CONSTRUCTION PLANS, AND ONE CADD FILE OF SUCH, AT THE TIME OF SIGNAL CONDITIONAL ACCEPTANCE INSPECTION BY THE MAINTAINING AGENCY.

THE CONTRACTOR SHALL DELIVER ALL REMOVED TRAFFIC SIGNAL EQUIPMENT, EXCEPT CONCRETE STRAIN POLES, TO THE TOWN OF PALM BEACH TRAFFIC SIGNALS SECTION LOCATED AT: 951 OLD OKEECHOBEE ROAD, WEST PALM BEACH, FL. 32401 (561) 838-5440.  THE CONTRACTOR SHALL CONTACT THE CITY (7) FULL WORKING DAYS PRIOR TO DELIVERY.  REMOVED CONCRETE STRAIN POLES SHALL BE DISPOSED OF AT THE CONTRACTOR'S EXPENSE IN A MANNER AND LOCATION APPROVED BY THE ENGINEER PRIOR TO REMOVAL.