Survey Report Checklist

This checklist is not intended to be all inclusive.  There may be other requirements set forth by the Standards of Practice found in Rule Chapter 5J-17 of the Florida Administrative Code or the Department that must be adhered to.

General Report Contents

Every survey report must include:

- The type of survey it depicts, pursuant to 5J-17.050(1)(a)-(k).

- Project information, i.e. project number, project name, county, and a description of the work performed.

- The name of the company, certificate of authorization number and mailing address of the business entity, along with the name and license number of the surveyor and mapper in responsible charge.

- The survey date range (start date to end date), which is the date of acquisition. When the graphics of a map are revised, but the survey date stays the same, the map must list the dates for all revisions.

- A statement indicating that “the survey map and report or the copies thereof are not valid without the signature and original raised seal of a Florida licensed surveyor and mapper”, unless the map is signed electronically.

- A reference to the map by title, date and subject.  Likewise the map should reference the report title, date and subject.

- A statement on the report and map that neither is full and complete without the other.

- A reference to all bearings shown shall be stated.

- A north arrow and either a stated or graphic scale.

- A legend containing all abbreviations, symbols and line types.

- The horizontal and vertical datum used.

- A list of all published control points (horizontal and vertical) that were used, if any.

- If digital coordinate files are delivered, a statement to the effect of: “This map is intended to be displayed at a scale of 1/_ or smaller”.

- A list of all included deliverables, e.g. .dgn files, .dwg files, .gpk files, .obs files, field books.

Specific Report Contents

Boundary Survey reports must include:

- The legal description.

- The methodology used for boundary location, e.g. conventional traverse, RTK GPS, static GPS.

- The horizontal and vertical values and the description of all control points found or set.

- An explanation of any boundary inconsistencies that the survey did not attempt to detect.

Control Survey reports must include:

- The methodology used for the location and/or setting of control points, e.g. conventional traverse, RTK GPS, static GPS.

- The horizontal and vertical values and the description of all control points found or set.

Topographic Survey reports must include:

- The methodology used for the location of topographic features e.g. conventional total station, RTK GPS, photogrammetry.

- The horizontal and vertical values and the description of the site control used to collect topographic data.

- If boundary lines are shown, a statement as to their source.

Specific Purpose Survey reports must include:

- A statement as to the purpose and conditions of the survey

  - Surveys performed for the purpose of monumenting, referencing, describing, and mapping centerline or baseline, as well as surveys performed for the purpose of monumenting official right of way lines may be performed as Specific Purpose Surveys.