Narrative

HISTORY.  In 1972, Proj. 50001-3401 constructed 305’ Culvert No. CBC123 to carry I-10 over Example Creek.  In 2001, Project 50001-3422 (FIN 222534-1) added riprap to the north end, and improved the geometry for the eastbound on-ramp to I-10, but 50001-3422 did not extend the culvert.  This load rating is an update that includes Emergency Vehicles.

CONDITION.  01-08-2018, David Stump inspected, finding good condition overall (NBI Item Nos. 61 and 62 were both 7-Good).  Among all walls, Stump noted “Vertical crack with efflorescence, corrosion bleed out and active water leakage in all sidewalls.”  However, the walls do not govern this structure, and the deterioration does not affect the carrying capacity at this time.  Therefore, this analysis applies a condition factor of 1.00 and uses full sections.

DISCUSSION.  This analysis was used to revise the “Hinged-End Culvert Example” published at https://www.fdot.gov/maintenance/LoadRating.shtm.  Code citations were updated to LRFD 8th Ed., and FAST Act emergency vehicles were added.

CONCLUSION.  Culvert No. 500112 is sufficient for all legal loads.