Methodologies for

THE FDOT SOURCE BOOK

A Technical Report - 2021

produced by

Florida Department of Transportation Forecasting and Trends Office

Methodologies for

produced by

Florida Department of Transportation Forecasting and Trends Office

Published

February 2022

# THE FDOT SOURCE BOOK

A Technical Report - 2021

Table of Contents

1.0	Executive Summary	1

2.0	Fast Facts 2021	2-1

3.0	Factors Affecting Mobility, Infrastructure, and Accountability	3-1

3.1	Factors Affecting Mobility	3-1

3.2	Accountability	3-2

4.0	Use of Probe Data in the 2021 Source Book	4-1

5.0	Source Book Segmentation	5-1

5.1	Segment Consolidation	5-1

5.2	Segments Updated Based on FHWA Urbanized and MPO Planning Boundaries	5-1

6.0	Intermediate Calculation Routines	6-1

6.1	D Factors	6-1

6.2	Hourly Factors	6-2

6.3	Day of Week Variance	6-3

6.4	Vehicle Occupancies	6-4

6.5	Travel Speeds	6-4

6.5.1	Field Mobility Measures Calculations: Field-Measured Speeds	6-4

6.5.2	Modeled Mobility Measures Calculations: Speed-Volume Functions	6-6

6.6	Peak Hour vs Peak Period	6-10

7.0	People-Related Mobility Measures: Auto/Truck	7-1

7.1	Vehicle Miles Traveled	7-1

7.2	Person Miles Traveled	7-1

7.3	Travel Time Reliability: Planning Time Index (Auto and Combination Truck)	7-2

7.4	Average Travel Speed – Auto	7-3

7.5	Average Speed vs. Posted Speed	7-4

7.6	Vehicles per Lane Mile	7-4

7.7	% Travel by Congestion Type	7-5

7.8	% Miles by Congestion Type	7-5

7.9	% Time by Congestion Type	7-6

7.10	Duration of Congestion	7-7

7.11	Hours of Delay	7-7

7.12	Job Accessibility by Auto	7-10

8.0	Safety Measures: Auto/Truck	8-1

8.1	Number of Fatalities	8-1

8.2	Number of Serious Injuries	8-1

8.3	Rate of Fatalities	8-1

8.4	Rate of Serious Injuries	8-2

8.5	Motorcycle Fatalities and Serious Injuries	8-2

8.6	Pedestrian Fatalities and Serious Injuries	8-2

8.7	Bicycle Fatalities and Serious Injuries	8-3

8.8	Safety Belt Use	8-3

9.0	People-Related Mobility Measures: Transit	9-1

9.1	Transit Revenue Miles	9-1

9.2	Transit Passenger Trips	9-1

9.3	Transit Revenue Miles between Failures	9-1

9.4	Transit Weekday Span of Service	9-2

9.5	Resident Access to Transit	9-2

9.6	Job Accessibility by Transit	9-2

9.7	Transit Passenger Trips per Revenue Mile	9-3

10.0	People-Related Mobility Measures: Pedestrian/Bicycle	10-1

10.1	% Pedestrian Facility Coverage	10-1

10.2	% Bicycle Facility Coverage	10-1

11.0	People-Related Mobility Measures: Aviation	11-1

11.1	Aviation Passenger Boardings	11-1

11.2	Aviation Departure Reliability	11-2

12.0	People-Related Mobility Measures: Rail	12-1

12.1	Rail Passengers	12-1

12.2	Passenger Rail On-time arrival	12-1

13.0	People-Related Mobility Measures: Seaport	13-1

13.1	Seaport Passenger Movements	13-1

14.0	Freight-Related Mobility Measures: Truck	14-1

14.1	Average Travel Speed – Combination Truck	14-1

14.2	Combination Truck Cost of Delay	14-1

15.0	Freight-Related Mobility Measures: Aviation	15-1

15.1	Aviation Tonnage	15-1

16.0	Freight-Related Mobility Measures: Space Measures	16-1

16.1	Space Launches and Sites	16-1

16.2	Space Payloads	16-2

17.0	Freight-Related Mobility Measures: Seaport	A-1

17.1	Seaport Tonnage	A-1

17.2	Seaport Twenty-Foot Equivalent Units	A-1

Appendix

Appendix A.	Hourly Factors	A-1

Appendix B.	Day of Week Calculation	B-1

Appendix C.	Freeway Speed Model	C-1

Appendix D.	Arterial Speed Model	D-1

Appendix E.	Two-Lane Highway Speed Model	E-1

Appendix F.	2012 Generalized Service Volume Tables	F-1

List of Tables

Table ES.1	Multimodal Mobility Measures	2

Table ES.2	Summary of Multimodal Mobility Measures	3

Table 2.1	Fast Facts 2021 Data Sources	2-1

Table 5.1	Segment Consolidation Criteria	5-1

Table 6.1	D Factors	6-1

Table 6.2	Day of Week Variance Factors	6-3

Table 7.1	Recommended Average Vehicle Occupancies	7-2

Table 7.3	Summary of Delay Threshold Speeds	7-7

Table D.1	BPR Parameters	D-1

Table D.2	Interpolated Speed	D-2

List of Figures

Figure 6.1	Queue Spillover Algorithm	6-7

Figure 6.2	Estimation of Average Travel Speed on Freeways	6-8

Figure 6.3	Estimation of Average Travel Speed on Arterials	6-9

Figure 6.4	Estimation of Average Travel Speed on Two-Lane Highways	6-10

Figure E.1	Two-Lane Highway Classification	E-1

Figure E.2	Base (Ideal) Speed-Flow Relationship in Two-Lane Highways	E-2

# Executive Summary

The FDOT Source Book (Source Book) is an easy-to-use compendium of current and historical safety and mobility data to measure Florida's transportation system performance and of factors affecting the system. The objective of this document is to detail the methodologies used to develop the measures and factors presented in the Source Book.

Identifying the right measures assists in portraying an accurate account of the current state of Florida’s transportation system. Good performance measures provide the consumer with information that allows them to draw an understanding of the situation. These measures can be used to gauge the quality, quantity, accessibility and utilization of Florida’s roadway network. They can either emphasize the travelers’, shippers’ or the agency’s perspective (e.g. delay, reliability, combination truck tonnage, etc.). Performance measures are critical for evaluating transportation system conditions and congestion. By reporting on a robust set of multimodal performance measures, the Florida Department of Transportation (FDOT) captures mobility and safety conditions, and analyzes and addresses any factors that could have impacted or would impact system performance.

Calculation of mobility measures in the Source Book is based on a combination of measured (archived field-measured travel speed data) and modeled results. Most of the data are obtained from existing sources such as the Department’s Roadway Characteristics Inventory (RCI), Traffic Characteristics Inventory (TCI), Weigh-in-Motion (WIM), and the archived field-measured travel probe speed data provided by HERE through Regional Integrated Transportation Information System (RITIS). These data sources provide information on the roadway geometry (such as the number of lanes, posted speed limit, and functional classification), demand for the roadway (traffic volume and vehicle classification) and field-measured roadway travel speed information. The full list of multimodal mobility performance measures is shown in Table ES.1. Most of these are calculated from 2016 to 2021 using the methodologies discussed in this document. Table ES.2 presents a summary of the calculation routines, methodology, sources and reporting periods for all the measures.

In addition to the multimodal mobility and safety measures, the Source Book also provides a context to many of the measures by linking them and other factors to present a comprehensive picture. This analysis aims to understand the relationship between various external factors that are beyond the control of FDOT, and the mobility measures.

List of Factors Affecting Mobility

- Hourly Traffic Volume

- Fuel Consumption

- Employment in e-Commerce and Department Stores

- Commuting and Remote Work

- Electric Vehicles

- Demographic Trends

Table ES.1	Multimodal Mobility Measures

Table ES.2	Summary of Multimodal Mobility Measures

| Measure | Description | Calculation | Reporting Period | Sources |
| --- | --- | --- | --- | --- |
| Auto Mobility Measures |  |  |  |  |
| Vehicle Miles Traveled | Vehicle miles traveled (VMT) is determined using vehicle traffic volume and segment length. | Segment Length ×Segment Volume | Peak Hour Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) |
| Person Miles Traveled | Person Miles Traveled (PMT) is determined by multiplying VMT by the average vehicle occupancy. | Segment Length×Volume×Average Vehicle Occupancy | Peak Hour Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) U.S. DOT – National Household Travel Survey 2009 Florida Add-On |
| Planning Time Index | The Planning Time Index (PTI) is the ratio of the 95th percentile travel time to the travel time at reference speed. This measure represents the additional time that a traveler should budget to ensure on-time arrival 95 percent of the time. | 95th Percentile Travel TimeRefernce Speed Travel Time | Peak Hour Peak Period Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Average Travel Speed | Average travel speed is the average of all hourly segment travel speeds captured by probe data or modeled through speed-volume functions, weighted by the segment’s VMT. | Segment VMT ×Average Travel Speed on SegmentVMT | Peak Hour Peak Period | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Average Speed vs. Posted Speed | This measure is expressed as the ratio of average peak hour speed against the posted speed limit. The output represents the percent of the posted speed limit achieved during the peak hour (5:00 to 6:00 pm on a weekday). | Segemnt VMT ×Peak Hour SpeedPosted Speed LimitVMT | Peak Hour | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 311 (Speed Limits) HERE Technologies – Travel Time Data |
| Vehicles Per Lane Mile | The vehicles per lane mile is calculated as the summation of each roadway segment’s peak hour vehicle miles traveled divided by the number of lane miles. | Segment VolumeNumber of Lanes×Lane MilesLane Miles | Peak Hour | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) and Feature 212 (Through Lanes) |
| % Travel by Congestion Level | Percent travel by congestion level is determined using VMT when speeds are at certain threshold levels for heavy congestion, mild congestion, or uncongested. Each congestion category is based on average travel speeds meeting the corresponding congestion threshold. | VMT when travel speeds meet congestion thresholdVMT×100 | Peak Hour Peak Period Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) and Feature 311 (Speed Limits) HERE Technologies – Travel Time Data |
| % Miles by Congestion Level | This measure reports the percent miles of roadway categorized as heavily congested, mildly congested, or uncongested. Each congestion category is based on average travel speeds meeting the corresponding congestion threshold. This measure estimates the percent miles by the three levels of congestion for the reporting periods of peak hour and peak period. | Segment length when travel speeds meet congestion thresholdSegment Length×100 | Peak Hour Peak Period | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) and Feature 311 (Speed Limits) HERE Technologies – Travel Time Data |
| % Time by Congestion Level | This measure estimates the percent of hours spent by travelers during three different levels of congestion: heavy, mild, or uncongested. For each roadway segment, congestion is categorized as heavy, mild, or uncongested based on whether the average travel speed meets the corresponding congestion threshold. | Hours when travel speeds meet congestion thresholdSegment Length×100 | Daily Yearly | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) and Feature 311 (Speed Limits) HERE Technologies – Travel Time Data |
| Vehicle Hours of Delay | Vehicle hours of delay was estimated hourly by determining the difference between actual travel time and the delay threshold travel time along a facility. Delay threshold travel time is the travel time for a motorist during uncongested conditions. The definition of uncongested conditions is level of service (LOS) B. | Vehicle Volume ×Travel Time-Threshold Travel Time | Peak Hour Daily Yearly | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Person Hours of Delay | Person hours of delay was estimated by multiplying the average vehicle occupancy by the vehicle hours of delay. Average vehicle occupancies were developed by county using the 2009 National Household Travel Survey (NHTS) Florida add-on dataset | Average Vehicle Occupancy ×Vehicle Volume ×Travel Time-Threshold Travel Time | Peak Hour Daily Yearly | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) U.S. DOT – National Household Travel Survey 2009 Florida Add-On HERE Technologies – Travel Time Data |
| Job Accessibility by Auto | Job accessibility is a “cumulative opportunity” metric — it reflects the total number of jobs reachable by auto within a travel time threshold. Travel times are calculated using a detailed road network and speed data that reflect typical conditions for an 8 a.m. Wednesday morning departure. Additionally, the accessibility results for 8 a.m. are compared with the maximum accessibility results across the 24-hour period to estimate the impact of road and highway congestion on job accessibility. | BlockLevel Number of Jobs ×Number of WorkersNumberof Workers | Yearly | University of Minnesota Accessibility Observatory – National Accessibility Evaluation Florida Report |
| - Safety-Related Mobility Measures |  |  |  |  |
| Number of Fatalities | Total number of fatalities on Florida’s roadways as a direct result of a traffic crash within thirty days of the crash occurrence. | People Killed Due to Traffic Crashes | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Number of Serious Injuries | Number of disabling/incapacitating injuries from traffic crashes that prevent the injured individuals from engaging in normal activities and which usually require transport to a medical facility and hospitalization. | People Seriously Injuried Due to Traffic Crashes | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Rate of Fatalities | Total number of fatalities on Florida’s roadways per 100 million VMT. | FatalitiesAnnual VMT x 100 million | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Rate of Serious Injuries | Total number of serious injuries (non-fatal) on Florida’s roadways per 100 million VMT. | Serious InjuriesAnnual VMT x 100 million | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Motorcycle Fatalities and Serious Injuries | These safety measures capture the number of fatalities and incapacitating injuries of both motorcyclists and their passengers. | Motocyclist and Passenger Fatalities Motocyclist and Passenger Serious Injuries | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Pedestrian Fatalities and Serious Injuries | This measure is the total number of pedestrian fatalities and serious injuries (incapacitating) that are reported on Florida’s public roads. | Pedestrian Fatalities Pedestrian Serious Injuries | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Bicycle Fatalities and Serious Injuries | This is the total number of bicyclist and passenger fatalities and serious injuries (incapacitating) on all of Florida’s roadways. | Bicyclist and passenger Fatalities Bicyclist and passenger Serious Injuries | Yearly | Florida Department of Highway Safety and Motor Vehicles – Traffic Crash Facts Annual Report |
| Safety Belt Usage | A field survey of a one-hour observation at each of the 165 selected sites across 15 counties. | Observed Safety-Belt-Using Drivers and PassengersTotal Observations x 100 | Yearly | FDOT Safety Office – Florida Safety Belt Use Reports |
| Transit Mobility Measures |  |  |  |  |
| Transit Revenue Miles | Number of annual miles a transit vehicle travels while in active service (i.e. available to pick up revenue passengers). | Revenue miles of vehicle operation while in active service | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Transit Passenger Trips | Annual number of passengers boarding on transit vehicles. A trip is counted each time a passenger boards a transit vehicle. Thus, if a passenger must transfer between buses or trains to reach a destination, the passenger is counted as making two passenger trips. | Passengers Boading Transit Vehicles | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Transit Revenue Miles Between Failures | Number of total annual revenue miles divided by the number of revenue vehicle system failures, an indicator of the average frequency of delays because of a problem with the equipment. A failure is classified as the breakdown of either a major or minor element of the revenue vehicle’s mechanical system. | Total Annual Revenue MilesNumber of Revenue Vehicle System Failures | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Transit Weekday Span of Service | The number of hours that transit service is provided on a representative weekday in the operation of the transit agency. This indicator is determined by computing the number of hours between the time service begins and the time service ends for an average weekday. | Service SpansTransit Agencies | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Resident Access to Transit | The percentage of the population within a half-mile of fixed-route transit. | Population within a half mile of TransitTotal Population | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Job Accessibility by Transit | Job accessibility is a “cumulative opportunity” metric — it reflects the total amount of jobs reachable by transit within a travel time threshold. It is calculated for each census block, and the results are aggregated to provide a statewide average. This analysis uses the 7:00 a.m. - 9:00 a.m. time period. The data are averaged across that window from minute-by-minute departures. | Block Level Number of Jobs ×Number of WorkersNumber of Workers | Yearly | University of Minnesota Accessibility Observatory – National Accessibility Evaluation Florida Report |
| Transit Passenger Trips Per Revenue Mile | The ratio of annual transit passenger trips to total annual transit revenue miles of service; a key indicator of service effectiveness that is influenced by the levels of demand and the supply of service provided. | Annual Transit Passenger TripsAnnual Transit Revenue Miles | Yearly | FDOT – Florida Transit Information and Performance Handbook |
| Pedestrian/Bicyclists Mobility Measures |  |  |  |  |
| % Pedestrian Facility Coverage | The pedestrian facilities include non-limited State Highway System (SHS) roadways and a limited number of non-SHS miles deemed of interest to FDOT with a sidewalk or a shared-use path on either side of a roadway. | Urban Pedestrian Facility Miles Urban SHS Non-Limited Access Centerline Miles ×100 | Yearly | FDOT – Roadway Characteristics Inventory Feature 216 (Bike Lanes/Pedestrian Facilities) |
| % Bicycle Facility Coverage | The bicycle facility miles include non-limited SHS roadway miles and a limited number of non-SHS miles deemed of interest to FDOT, with a Paved Shoulder ≥ 4' and < 7' or a Shared Path on either side. | Miles of Bicycle FacilitiesSHS Non-Limited Access Centerline Miles ×100 | Yearly | FDOT – Roadway Characteristics Inventory Feature 214 (Outside Shoulders) and Feature 216 (Bike Lanes/Pedestrian Facilities) |
| Aviation Mobility Measures |  |  |  |  |
| Aviation Passenger Boardings | The total number of revenue passengers who board an aircraft at a Florida airport. These airports include all commercial service airports. If a passenger has to transfer between planes to reach a destination, the passenger is counted as making two passenger boardings. | Revenue Passengers Boarding Aircraft | Yearly | U.S. Federal Aviation Administration, Passenger Boarding (Enplanement) and All-Cargo Data for U.S. Airports |
| Aviation Departure Reliability | Departure is deemed reliable if the flight departs within 15 minutes after the scheduled time shown in the carrier’s Computerized Reservations Systems (CRS). In the aviation industry, this is considered an on-time departure. Departure reliability is based on departure from the gate and can be influenced by various factors such as heavy traffic volume, weather, and mechanical reasons beyond the control of the airport or the air carrier. Aviation departure reliability is reported for all commercial service airports. | Departures≤15 Minutes of ScheduleDepartures ×100 | Yearly | U.S. Department of Transportation, Bureau of Transportation Statistics, On-Time Departure T-100 Segments |
| People-Related Mobility Measures: Rail |  |  |  |  |
| Rail Passengers | Rail passengers refer to the annual number of revenue-paying rail passengers. For the purpose of this performance measure, rail passengers include those riding on Amtrak as well as inter-regional transit services, such as SunRail and Tri-Rail. | Rail Passengers on SunRail, Tri-Rail and Amtrak | Yearly | Amtrak – Amtrak Fact Sheet Federal Transit Administration – National Transit Database (NTD) |
| Passenger Rail On-time arrival | Rail on-time arrival (Amtrak, SunRail and Tri-Rail) captures the on-time performance of a passenger rail system operating within Florida. A train is considered on-time if arrival is within a specified threshold time frame of scheduled arrival time. The threshold time frame varies based on the trip length. For long distance trips, the time frame is wide (e.g., 30 minutes). For commuter trains and local heavy rail systems, the time frame is narrow (e.g., 15 minutes). | On Time TrainsTrains ×100 | Yearly | Amtrak – Amtrak Fact Sheet South Florida Regional Transportation Agency’s Transit Development Plan (TDP) FDOT Freight & Multimodal Operations Office |
| Seaport Mobility Measures |  |  |  |  |
| Seaport Passenger Movements | Seaport passenger movements refer to passengers embarking and disembarking a cruise ship at a Florida seaport. For example, when a cruise passenger departs on a cruise ship from PortMiami, they are counted. When the same cruise passenger arrives back at PortMiami, they are counted again. | Cruise Passenger Movements | Yearly | Florida Seaport Transportation and Economic Development (FSTED) Council – Five-Year Florida Seaport Mission Plan |
| Freight-Related Mobility Measures: Truck |  |  |  |  |
| Combination Truck Planning Time Index | Combination Truck Planning Time Index is the same as Planning Time Index (PTI), defined as a ratio of the 95th percentile travel time to the free flow travel time. This measure represents the additional time that a shipper should budget to ensure on-time arrival 95% of the time. | 95th Percentile Travel TimeRefernce Speed Travel Time | Peak Hour Peak Period Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Combination Truck Hours of Delay | Combination Truck (CT) Hours of Delay is estimated hourly by determining the difference between actual travel time and the delay threshold travel time along a facility. Delay threshold travel time is the travel time experienced during uncongested conditions. | CT Volume ×CT Travel Time-Threshold Travel Time | Daily | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Combination Truck Average Travel Speed | Combination truck (CT) average travel speed is the average of all hourly segment travel speeds captured by probe data or modeled through speed-volume functions. The CT average speed is estimated assuming that the free flow speed for automobile is set as the posted speed limit plus 5 mph, and the free flow speed for combination trucks is the posted speed limit. | CTMT ×CT Average Travel SpeedCTMT | Peak Hour | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data |
| Combination Truck Cost of Delay | Combination truck cost of delay is estimated by calculating the average marginal cost of labor per hour and multiplying that by the number of hours of delay experienced by combination trucks (Classes 8-13). | Combination Truck Hours of Delay ×Average Marginal Cost of Labor per Hour | Yearly | FDOT – Traffic Characteristics Inventory FDOT – Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System) HERE Technologies – Travel Time Data American Transportation Research Institute (ATRI) – An Analysis of the Operational Costs of Trucking |
| Freight-Related Mobility Measures: Aviation |  |  |  |  |
| Aviation Tonnage | Aviation tonnage is the weight of all reported air cargo and mail handled at Florida airports which includes commercial service, general aviation, and military airports. | Weight of Air Cargo Handled at Florida Airports | Yearly | U.S. Bureau of Transportation Statistics - T100 Market (all Carriers) |
| Freight-Related Mobility Measures: Space Launches and Payloads |  |  |  |  |
| Space Launches | Space launches account for all the successful mission launches from Cape Canaveral Spaceport, other than any test flights. | Successful Orbital Launches | Yearly | Space Florida |
| Active Launch Sites in Florida | Launch sites include active launch sites at Florida's civilian and military spaceports. | Launch Sites | Yearly | Space Florida |
| Space Payloads | Payloads describes the cargo aboard spacecraft launches, identifying the military, civil, and commercial benefits of aerospace transportation. | Number of civil, commercial and military payloads | Yearly | Space Florida |
| Payload Weight in Orbit | The weight of cargo in pounds measured at sea level on Earth. Payload weight accounts for only the revenue-generating portion of the load that is carried by a spacecraft exclusive of what is necessary for operation and flight. | Weight of Payloads | Yearly | Space Florida |
| Freight-Related Mobility Measures: Seaport |  |  |  |  |
| Seaport Tonnage | International and domestic waterborne tons of cargo handled at Florida’s public seaports. | Seaport Tonnage | Yearly | FSTED Council – Five-Year Florida Seaport Mission Plan |
| Seaport Twenty-Foot Equivalent Units | Twenty-foot equivalent unit (TEU) represents the cargo capacity of a standard intermodal container, twenty feet long and eight feet wide. It is the standardized unit of the capacity of a container ship, or a container terminal. | Twenty-Foot Equivalent Units | Yearly | FSTED Council – Five-Year Florida Seaport Mission Plan |

# Fast Facts 2021

This section describes the data sources used to summarize the Florida FAST Facts table.

Table 2.1	Fast Facts 2021 Data Sources

| Measure | Source |
| --- | --- |
| General |  |
| People | U.S. Census Bureau, 2020 Census |
| Licensed Drivers | Florida Department of Highway Safety and Motor Vehicles, 2021 Annual License Driver Report |
| Square Miles of Land | Florida Department of State, Quick Facts / Geographical Facts |
| Highway & Bridge |  |
| Centerline miles of public roads | FDOT Transportation Data and Analytics Office, Public Road Mileage and Travel Report |
| Centerline miles of State Highway System (SHS) | FDOT Transportation Data and Analytics Office, State Highway System Mileage and Travel (DVMT) Report |
| Centerline miles of Strategic Intermodal System (SIS) | FDOT Transportation Data and Analytics Office, Strategic Intermodal System Only (SIS) |
| Bridges maintained by FDOT | FDOT Office of Maintenance, Florida Bridge Inventory |
| Percent of bridges with a National Bridge Inventory Rating of at least 6 | FDOT Office of Maintenance |
| Percent of pavement conditions on SHS exceeding FDOT standards | FDOT State Materials Office, Pavement Condition Survey Program |
| Registered motor vehicles | Florida Department of Highway Safety and Motor Vehicles, Traffic Crash Facts Annual Report 2020 |
| Daily vehicle miles traveled on the SHS | FDOT Transportation Data and Analytics Office, State Highway System Mileage and Travel (DVMT) Report |
| Percent of SHS centerline miles heavily congested during peak hour | FDOT Transportation Data and Analytics Office, Traffic Characteristics Inventory FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory HERE Technologies, Travel Time Data |
| Transit |  |
| Urban transit systems | FDOT Transit Office |
| Rural transit systems | FDOT Transit Office |
| Annual transit passenger trips | FDOT Transit Office, Florida Transit Information and Performance Handbook |
| Percent of Florida's population living within a half-mile of fixed route transit | FDOT Transit Office, Florida Transit Information and Performance Handbook |
| Pedestrian & Bicycles |  |
| Miles of bicycle facilities on non-freeway SHS | FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory, Feature 214 (Outside Shoulders) and Feature 216 (Bike Lanes/Pedestrian Facilities), |
| Miles of pedestrian facilities on urban non-freeway SHS | FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory, Feature 214 (Outside Shoulders) and Feature 216 (Bike Lanes/Pedestrian Facilities) |
| Percent of non-freeway SHS have bicycle facilities | FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory, Feature 214 (Outside Shoulders) and Feature 216 (Bike Lanes/Pedestrian Facilities) |
| Percent of non-freeway SHS in urban areas have pedestrian facilities | FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory Feature 214 (Outside Shoulders) and Feature 216 (Bike Lanes/Pedestrian Facilities) |
| Ports (Sea/Air/Space) |  |
| Number of commercial airports | FDOT Aviation Office |
| Number of deep water seaports | Florida Seaport Transportation and Economic Development Council, Five-Year Florida Seaport Mission Plan |
| Number of active space launch sites | Space Florida |
| Number of space launches | Space Florida |
| Annual aircraft passenger boardings | Federal Aviation Administration, Air Carrier Activity Information System (ACAIS) Passenger Boarding and All-Cargo Data |
| Percent of aircraft trips departing on time | U.S. Department of Transportation Bureau of Transportation Statistics, Percent On-Time Departure |
| Number of annual cruise passenger movements | Florida Seaport Transportation and Economic Development Council, Five-Year Florida Seaport Mission Plan |
| Rail |  |
| Miles of mainline railroad track | FDOT Freight and Multimodal Operations Office |
| Annual rail passengers (Tri-Rail, Amtrak, SunRail) | Federal Transit Administration; National Transit Database, Monthly Ridership Database, September 2021 Adjusted (Tri-Rail and SunRail) Amtrak, State Fact Sheet (Amtrak) |
| Emerging Mobility |  |
| Electric vehicle registrations in Florida (as of July 2020) | Florida Department of Highway Safety and Motor Vehicles |
| Publicly accessible electric vehicle charging stations in Florida | U.S. Department of Energy Alternative Fuels Data Center, Florida Transportation Data for Alternative Fuels and Vehicles |
| Economy |  |
| Florida's Gross Domestic Product | U.S. Bureau of Economic Analysis, GDP and Personal Income |
| Visitors to Florida | Visit Florida, Research |
| Safety |  |
| Serious Injuries | Florida Department of Highway Safety and Motor Vehicles, Traffic Crash Facts Annual Report 2020 |
| Fatalities | Florida Department of Highway Safety and Motor Vehicles, Traffic Crash Facts Annual Report 2020 |

# Factors Affecting Mobility, Infrastructure, and Accountability

## Factors Affecting Mobility

This section of the Source Book explores the impact of various external factors on mobility in Florida. The process for analyzing each factor/trend is as follows:

- Gather relevant data from the appropriate Florida specific data source,

- Process and analyze the data, if applicable, and

- Visualize the Florida trends.

In this section, the data sources used for the factors affecting mobility are provided below:

#### Hourly Traffic Volume

- FDOT Transportation Data and Analytics Office. Telemetered Traffic Monitoring Sites (TTMS) on Freeways

#### Fuel Consumption

- Florida Department of Revenue. “Certified Taxable Gallons of Motor Fuel and Diesel.” Tax Collections and Distributions, General Tax Statistics: Download Fuel Tax Distribution.

#### Employment in E-Commerce and Department Stores

- U.S. Bureau of Labor Statistics. “BLS Data Finder”. Search SMU12000004245410001 for Electronic Shopping. Search SMU12000004245220001 for Department Stores.

#### Commuting and Remote Work

- U.S. Census Bureau. “American Community Survey.”  Data Releases. https://www.census.gov/programs-surveys/acs/news/data-releases.2020.html

- FDOT Forecasting and Trends Office. “2021 Florida Resident Satisfaction Survey.” Statewide Surveys. https://www.fdot.gov/planning/survey/default.shtm

#### Electric Vehicles and Charging Stations

- FDOT Forecasting and Trends Office. “2021 Florida Resident Satisfaction Survey.” Statewide Surveys. https://www.fdot.gov/planning/survey/default.shtm

- U.S. Department of Energy, Office of Energy Efficiency and Renewable Energy Alternative Fuel Data Center. “Alternative Fueling Station Locator.” https://afdc.energy.gov/stations/#/find/nearest

#### Demographic Trends

- Florida Department of Highway Safety and Motor Vehicles. “Licensed Drivers by Age, Sex and County.” Driver and Vehicle Reports & Statistics. https://www.flhsmv.gov/resources/driver-and-vehicle-reports/

- US Census Bureau. “2020 Decennial Census by Race (P1) – Florida Total Population.” Decennial Census P.L. 94-171 Redistricting Data. : https://data.census.gov/cedsci/table?g=0400000US12,12%240500000&y=2020&d=DEC%20Redistricting%20Data%20%28PL%2094-171%29  Infrastructure

#### Pavement Conditions

The FDOT State Materials Office’s Pavement Condition Unit conducts annual surveys of the entire State Highway System (SHS) in support of the Department's Pavement Management Program. The data collected (in terms of crack, ride, and rut measurements) is used to assess the condition and performance of the State’s roadways as well as to predict future rehabilitation needs.

FDOT State Materials Office. Pavement Condition Survey program

#### Bridge Conditions

FDOT’s core bridge measure is determined as the ratio of bridges maintained by the Department with a National Bridge Inventory (NBI) rating of 6 or higher to the total FDOT-maintained bridges.

- FDOT Office of Maintenance -

#### Maintenance Conditions

Maintenance condition is determined by evaluating each of the following five highway components: roadway, roadside, traffic services, drainage and vegetation/aesthetics.

FDOT Office of Maintenance

## Accountability

#### Construction Contracts Completed On-Time

FDOT Office of Construction. Quarterly Performance Measures Reports

#### Construction Contracts Completed Within Budget

FDOT Office of Construction, Quarterly Performance Measures Reports

.

# Use of Probe Data in the 2021 Source Book

The FDOT Source Book (Source Book) uses field-measured travel speed data, provided by HERE Technologies through the Regional Integrated Transportation Information System (RITIS) at the University of Maryland’s Center for Advanced Transportation Technology Laboratory (CATT Lab), for all its speed-based measures. The field-measured travel speed data covers most of the State Highway System (SHS) and National Highway System (NHS) roadways. Where the speed data does not cover the SHS or NHS, the reported mobility measures for the Source Book are calculated based on speeds estimated from speed-volume functions. The modeled and field-measured travel speeds as described below:

Modeled Mobility Measures Calculations: Mobility measures for the years of 2016 to 2020 are first calculated without using field-measured travel speed data. Speeds for each roadway segment are estimated using the traffic volume data and speed-volume functions discussed in Section 6.5.2.

Field Mobility Measures Calculations: Mobility measures for years of 2016 to 2020 are calculated using field-measured travel speed data and traffic volume data.

Since some measures are not dependent on travel speeds, the method for applying modeled and field-measured travel speeds is only applicable to the following measures:

- Travel Time Reliability: Planning Time Index

  - Auto

  - Combination truck

- % Travel by Congestion Type

- % Miles by Congestion Type

- % Time by Congestion Type

- Duration of Congestion

- Vehicle Hours of Delay

  - Auto

  - Combination truck

- Person Hours of Delay

- Average Travel Speed

  - Auto

  - Combination truck

# Source Book Segmentation

Roadway segmentation is developed to create appropriate segment lengths for analysis purposes. The 2021 Source Book continues in the same fashion and uses the similar segmentation as in the 2020 Source Book. The 2021 Source Book segments are updated since some new segments were removed from, updated or added to the 2020 State Highway System compared to previous years. Details for the segment consolidation are discussed below. The 2021 Source Book uses a uniform approach to segmenting arterial, highway, and freeway facilities for both calculation of non-reliability and reliability related measures.

## Segment Consolidation

For all 5 years of data from 2016 through 2020, segments are consolidated to produce longer segments than the RCI Top 30 table. This table is a “query for the most requested information” from the RCI database. Consolidation is based on criteria in Table 5.1. The segment consolidation is only completed on adjacent segments meeting these criteria. The resulting data set has 30% to 40% fewer segments than the RCI Top 30 table.

Table 5.1	Segment Consolidation Criteria

| Criteria | Nearby Roadway Characteristics |
| --- | --- |
| Consolidation | Roadway ID Federal Functional Classification Number of Lanes Speed Annual Average Daily Traffic (AADT) Level-of-Service Database Area Type Level-of-Service Database Facility Type |

## Segments Updated Based on FHWA Urbanized and MPO Planning Boundaries

The consolidated segmentation is further updated to support the easy extraction of performance measures for different geographic boundaries, such as FHWA urbanized boundaries, Metropolitan Planning Organization (MPO) planning boundaries and county boundaries.

GIS geographic analysis tools are used at the MPO, county, and urbanized area boundary levels to update the consolidated segmentations by splitting them and joining information. Each updated segment has the information to show whether it is covered by a county, MPO or an urbanized area. Using this information, the data and performance measures could be easily extracted for the specific county, MPO or urbanized area.

# Intermediate Calculation Routines

There are several intermediate calculation routines required by multiple measures in the 2021 Source Book. They are discussed in the sections below.

## D Factors

The D factor, or the directional factor, is the percentage of the hourly volume in the predominant direction. For the 2016-2020 mobility measures calculations, directional Annual Average Daily Traffic (AADT), when available, are used to estimate traffic volume in both the peak (with higher directional AADT) and off-peak directions. When the directional AADTs are not available, D factors are used to factor roadway total volume.

For those roadway segments without directional AADTs in the 2016-2020 calculations, peak and off-peak directional volumes are estimated with the assigned D factors based on lane equality, time of day and area type. Specifically, D factors are assigned based on the criteria summarized in Table 6.1.

Table 6.1	D Factors

| Lane Equality | Area Type | Time Period | D Factor |
| --- | --- | --- | --- |
| One-Way | All | All | 100% |
| Equal number of lanes in both travel directions | Urbanized | 6 - 9 a.m. | 54% |
|  |  | 4 - 5 p.m. | 54% |
|  |  | 5 - 6 p.m. | 55% |
|  |  | 6 - 7 p.m. | 54% |
|  |  | All others | 52% |
|  | Transitioning and Rural | 3 - 6 p.m. | 54% |
|  |  | All others | 52% |
| Unequal number of lanes in both travel directions | All | All | Max (0.55, D factor from TCI) |

Measures that use a D factor are identified below:

- % Travel by Congestion Type

- Vehicle Hours of Delay

  - Auto

  - Combination truck

- Person Hours of Delay

- Average travel speed

  - Auto

  - Combination truck

## Hourly Factors

The FDOT Transportation Data and Analytics Office compiled 2012 Florida continuous traffic data for many roadway facilities and area types. These data are categorized and hourly factors (i.e., the ratio of hourly volumes to daily volumes) are calculated for the eight facility types below:

- Freeway core (FRC)

Freeway urbanized (FRU)

Freeway transitioning (FRT)

Freeway rural (FRR)

Arterial urbanized (AU)

Arterial non-urbanized (ANU)

Two lane highway rural (2HY)

Multilane highway rural (MHY)

The results for the Source Book are affected by the hourly factors used. Comparisons and smoothing are performed with the hourly factors used in previous versions and with hourly factors derived from national research projects.

In addition, a set of weekend hourly factors are used to better represent traffic volumes on Saturdays and Sundays (see section below). Appendix A contains the weekday hourly factors and the weekend factors used in the 2021 Source Book.

Performance measures that use an hourly factor are identified below:

- Vehicle Miles Traveled

  - Auto

  - Trucks

  - Combination Trucks

- Person Miles Traveled

- % Travel by Congestion Type

- Vehicle Hours of Delay

  - Auto

  - Combination truck

- Person Hours of Delay

- Travel time reliability – planning time index

  - Auto

  - Combination truck

- Average travel speed

  - Auto

  - Combination truck

## Day of Week Variance

The continuous traffic data also enables a comparison of daily traffic between weekdays, Saturdays, and Sundays. It is expected that roadways would show higher daily traffic on weekdays, lower traffic on Saturdays, and much lower traffic on Sundays. While some variations from this pattern are observed (e.g., high traffic on rural roads on Fridays), the hypothesis is generally supported by the data. Therefore, day of week variance factors are developed for weekdays, Saturdays, and Sundays, allowing for a more precise representation of overall traffic conditions. Table 6.2 summarizes the day of week variance for daily traffic used in the 2021 Source Book.

Table 6.2	Day of Week Variance Factors

| Weekday | Saturday | Sunday |
| --- | --- | --- |
| 105% | 90% | 85% |

The application of day of week variance factors does not change the annual average daily traffic (i.e., the weighted-average of factors is equal to 100%).

For peak hour and peak period performance measures, only the weekday traffic volumes and results are used. For measures reported on a daily basis, a seven-day average was calculated by weighting the weekday (5x), Saturday (1x), and Sunday (1x) results. Measures that are reported daily and weighted by vehicle miles traveled (VMT) (e.g., average travel speed) give additional importance to weekday results and less importance to weekend ones, as there are not only more weekdays than weekend days but traffic volumes (and thus, VMT) are higher on weekdays. Appendix B presents a flowchart of the day of week variance calculation routine used in the 2021 Source Book calculations.

Measures that use day of week variance are identified below:

- Vehicle Miles Traveled

  - Auto

  - Trucks

  - Combination Trucks

- Person Miles Traveled

- Vehicles Per Lane Mile

- % Travel by Congestion Type

- % Time by Congestion Type

- Duration of Congestion

- Vehicle Hours of Delay

  - Auto

  - Combination truck

- Person Hours of Delay

- Travel time reliability – planning time index

  - Auto

  - Combination Truck

- Average travel speed

  - Auto

  - Combination truck

## Vehicle Occupancies

Vehicle occupancies are used to estimate the number of person miles traveled (PMT). In the 2021 Source Book, the vehicle occupancies are taken from 2009 National Household Travel Survey (NHTS) Florida Add-on dataset. The NHTS add-on dataset contains data from completed household surveys in the sample including household, person, vehicle and daily (travel day) trip level data. In addition to the households selected as part of the national survey, this dataset includes the data collected from the twenty add-on program regions, including the State of Florida. Data extracted from the NHTS are only for the trips where it is indicated that the respondent drove the vehicle. In other words, passenger trips by the respondent are excluded. Occupancies are calculated from driver trip data and reported occupancies. Occupancy averages are weighted by trip lengths to develop factors that can directly convert VMT to PMT.

Measures that use vehicle occupancies are identified below:

- Person Miles Traveled

- Person Hours of Delay

## Travel Speeds

Field-measured HERE data are used to estimate speeds in the 2021 Source Book. Model calculations are used to estimate speeds for segments that had no HERE coverage.

### Field Mobility Measures Calculations: Field-Measured Speeds

Travel speeds for 2016-2020 are obtained from field-measured data provided by HERE rather than the speed-volume functions. For each Source Book roadway segment, field-measured travel speeds are obtained on 15-minute increments. from the archive of the real time HERE speeds between January 1, 2020 and December 31, 2020. HERE removed outliers and provided imputation where there were not enough observations.

The following processes are conducted to conflate HERE speeds to the RCI segmentation:

- A data repository and analytical tool, developed by the Regional Integrated Transportation Information System (RITIS) at the University of Maryland, receives data feeds through an XML file from FDOT.

- These “uncapped speeds”, from the real-time dataset, are averaged to create 15-minute travel speeds per Traffic Message Channel (TMC) and downloaded as a text file

- TMC segments conflated to Source Book segmentation along FDOT’s Linear Referencing System using conflation process built upon the one developed by FDOT Safety Office.

- Source Book segments’ speeds are calculated.

The resulting dataset is one containing directional speeds in 15-minute increments for the Source Book segmentation. This dataset is used to compute Planning Time Index, while the rest of the speed-based measures use an aggregated dataset that summarizes speeds by hour, direction, day type, and congestion threshold. For segments where field-measured speeds are not available, speeds calculated from the speed-volume functions were used.

The day type criteria classify days by four types: regular weekdays, Saturdays, Sundays, and holidays. These holidays are excluded from speed calculations:

- January 1 – New Year

- January 20 – Martin Luther King

- May 25 – Memorial Day

- July 3 – Independence Day

- September 7 – Labor Day

- October 12 – Columbus Day

- November 11 – Veterans Day

- November 26 – Thanksgiving

- December 25 – Christmas Day

The congestion threshold criteria identifies speed ranges applicable for Heavily Congested (HC), Mildly Congested (MC), and Uncongested (UC) travel conditions based on the segment’s area and roadway type, as shown in the table below.

Table 6.3. Speed Ranges for Congestion Threshold

| Area Type | Roadway Type | Heavily Congested | Mildly Congested | Uncongested |
| --- | --- | --- | --- | --- |
| Urbanized Areas | Arterial PSL ≤ 35 mph | ≤ 13 mph | > 13 and < 22 mph | > 22 mph |
|  | Arterial PSL ≥ 40 mph | ≤ 18 mph | > 18 and < 31 mph | > 31 mph |
|  | Two-Lane Highway | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Multilane Highway | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Freeway (PSL <=50 mph) | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Freeway (PSL 55 mph) | ≤ 45 mph | > 45 and < 50 mph | > 50 mph |
|  | Freeway (PSL 60 mph) | ≤ 45 mph | > 45 and < 55 mph | > 55 mph |
|  | Freeway (PSL 65 mph) | ≤ 45 mph | > 45 and < 60 mph | > 60 mph |
|  | Freeway (PSL 70 mph) | ≤ 45 mph | > 45 and < 65 mph | > 65 mph |
| Non-Urbanized Areas | Arterial PSL ≤ 35 mph | ≤ 17 mph | > 17 and < 22 mph | > 22 mph |
|  | Arterial PSL ≥ 40 mph | ≤ 23 mph | > 23 and < 31 mph | > 31 mph |
|  | Two-Lane Highway | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Multilane Highway | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Freeway (PSL <=55 mph) | ≤ 66.7% of FFS | > 66.7% and < 83.3% of FFS | > 83.3% of FFS |
|  | Freeway (PSL 60 mph) | ≤ 50 mph | > 50 and < 55 mph | > 55 mph |
|  | Freeway (PSL 65 mph) | ≤ 50 mph | > 50 and < 60 mph | > 60 mph |
|  | Freeway (PSL 70 mph) | ≤ 50 mph | > 50 and < 65 mph | > 65 mph |
|  | Freeway (PSL 75 mph) | ≤ 50 mph | > 50 and < 70 mph | > 70 mph |

Note: PSL = Posted Speed Limit

Free Flow Speed (FFS) is defined as the 85th percentile of speeds during overnight hours (10 pm to 5 am) on all days excluding holidays.

### Modeled Mobility Measures Calculations: Speed-Volume Functions

As mentioned previously, speed-volume functions are only used in calculating the modeled mobility measures. The speed-volume functions are applied as follows:

- Freeway travel speeds are calculated using the Modified Davidson curve for both under-saturated and over-saturated conditions (see Appendix C). A comparison of the performance of different speed-flow functions in the Orlando Urban Area Transportation Study (OUATS) travel demand model suggests that the modified Davidson equation has the highest average coefficient of determination (R2) out of the study functions. The minimum speed is set at 10 mph.

- Arterial travel speeds are calculated using different methodologies for under-saturated and over-saturated conditions (see Appendix D). For under-saturated conditions (demand less than capacity), operating speeds are calculated based on the latest set of Generalized Service Volume Tables.

For over-saturated conditions (demand greater than capacity), a Bureau of Public Roads (BPR) curve (with Greater Treasure Coast Regional Planning Model parameters) is used to determine average travel speed given vehicle volume and segment capacity. The BPR curve is calibrated such that the V/C = 1.0 speed matches the Generalized Service Volume Tables LOS E speed. The v/c = 2.0 speed is set at a minimum of 7 mph.

- Two-lane highway travel speeds are calculated assuming a linear relationship between speed and flow according to the Highway Capacity Manual 6th Edition (HCM). Chapter 15: Two-Lane Highways of the HCM provides guidance on estimating average speeds on two-lane highways. The minimum speed is set at 10 mph (see Appendix E).

The following description summarizes the speed estimation technique.

#### Step 1: Obtain Hourly Directional Vehicle Volume

Daily volume is obtained directly from AADT. For purposes of estimating speed, AADT is first converted to hourly directional volumes, based on the day of week variance factor, updated hourly factor and directional factor presented in the appendices.

#### Step 2: Adjust for Spillback

Hourly vehicle volumes calculated from Step 1 are adjusted to consider queuing that occurs under saturated flow conditions. For each hour between 7:00 a.m. and 10:00 a.m. and between 4:00 p.m. and 7:00 p.m. where saturated conditions exist, it is assumed that the unserved part of the queue will spill back to the next hour. This unserved demand is added to the volume of the next hour and this total volume is used to calculate speed. The process is repeated until demand (including that from previous hour(s)) is less than the capacity. Figure 6.1 presents the algorithm in a flowchart graphical format.

Figure 6.1	Queue Spillover Algorithm

Hourly Directional Volume 
= AADT x The Day of Week Variance Factor x Hourly Factor x Directional Factor + Spillover Volume

Generalized Service Volume Table LOS E Threshold

Is Hourly Directional Volume > LOS E Service Volume?

Within Capacity: 
Spillover Volume = 0

Spillover Volume = Hourly Volume – LOS E Service Volume

Exceeding Capacity: 
Spillover Volume transferred to next hour for time periods between 7-10 a.m. and between 4-7 p.m.

YES

NO

#### Step 3: Determine whether Undersaturated or Oversaturated Conditions Prevail

The adjusted hourly volumes computed in Step 2 are compared with capacities obtained from the Generalized Service Volume Tables (Appendix F). If volume is greater than the capacity, oversaturated condition is assumed.

#### Step 4: Select Appropriate Travel Speed Estimation Technique

A speed estimation technique is then chosen based on the facility type and whether the facility is operating within capacity or over capacity. Figures 6.2, 6.3 and 6.4 provide a summary of the estimation techniques that are used in the 2021 Source Book for the modeled mobility measures calculations.

Figure 6.2	Estimation of Average Travel Speed on Freeways

Use modified Davidson equation documented in Appendix C to calculate hourly speeds for cars and trucks

Calculate Average Travel Speed by using weighted average (by volume) of speeds for cars and trucks

Hourly Directional Volume 
= AADT x The Day of Week Variance Factor x Hourly Factor x Directional Factor

AADT

Hourly Factor and Directional Factor

The Day of Week Variance Factor

Figure 6.3	Estimation of Average Travel Speed on Arterials

Hourly Volume 
= AADT x The Day of Week Variance Factor x Hourly Factor x Directional Factor

Is Hourly Volume ≥ LOS E Service Volume?

Within Capacity: 
Determine Auto LOS per Generalized Service Volume Tables

Exceeding Capacity: 
Use BPR Equation with customized parameters, documented in detail in Appendix D

YES

NO

Calculate Travel Speed for cars and trucks using linear interpolation

Calculate Average Travel Speed by using weighted average (by volume) of speeds for cars and trucks

AADT

Hourly Factor and Directional Factor

The Day of Week Variance Factor

Figure 6.4	Estimation of Average Travel Speed on Two-Lane Highways

Hourly Volume 
= AADT x The Day of Week Variance Factor  x  Hourly Factor x  Directional Factor

Use HCM 6th Edition Model, documented in detail in Appendix E

Calculate Average Travel Speed by using weighted average (by volume) of speeds for cars and trucks

Calculate Travel Speed for cars and trucks using linear interpolation

AADT

Hourly Factor and Directional Factor

The Day of Week Variance Factor

## Peak Hour vs Peak Period

Peak Hour is defined based on the highest weekday hourly observed volume (Appendix A) for different facility and area types. The hour with the highest hourly volume factor is 5-6 PM for most facility types and 4-5 PM for two facility types. Reporting for planning time indices uses the 5 – 6 pm peak hour; these are the peak hours by facility type:

  - Freeway Core: 5 – 6 pm

  - Freeway Urbanized: 5 – 6 pm

  - Freeway Transitioning: 5 – 6 pm

  - Freeway Rural: 4 – 5 pm

  - Arterial Urbanized: 5 – 6 pm

  - Arterial Non-urbanized: 5 – 6 pm

  - Two-lane Highway: 4 – 5 pm

  - Multilane Highway: 5 – 6 pm

Peak Period is defined as 4 – 6 pm for all facility types.

# People-Related Mobility Measures: Auto/Truck

The calculation methodology for each individual Auto/Truck mobility measure is discussed in this section.

## Vehicle Miles Traveled

The following steps describe the process used to obtain Vehicle Miles Traveled (VMT) on a single segment:

#### Step 1: Obtain Daily and Peak Hour Vehicle Volume

Daily vehicle volume was directly obtained from Annual Average Daily Traffic (AADT), while the peak hour volume was the product of the AADT and the highest hourly factor.

#### Step 2: Obtain Vehicle Miles Traveled (VMT)

VMT is determined using vehicle traffic volume and segment length. The number of vehicle-miles traveled is based on data obtained from FDOT’s traffic monitoring sites and Roadway Characteristics Inventory (RCI) Feature 111 data. Vehicle Miles Traveled is equal to the product of the daily or hourly volume and the roadway’s length in miles as shown in Equation :

Equation 1

VMT= Segment Length ×Segment Volume

#### Data Sources

- FDOT Transportation Data and Analytics Office, Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System)

## Person Miles Traveled

Person Miles Traveled (PMT) is computed by multiplying VMT by the average vehicle occupancy. Average vehicle occupancies in the Source Book are provided on a county-by-county basis (see Table 7.1). PMT was computed using Equation :

Equation 2

PMT=Segment Length×Segment Volume×Average Vehicle Occupancy

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- U.S. Department of Transportation (USDOT). National Household Travel Survey, Florida Add-On, 2009

Table 7.1	Recommended Average Vehicle Occupancies

| County | Occupancy | County | Occupancy | County | Occupancy |
| --- | --- | --- | --- | --- | --- |
| Alachua | 1.77 | Hardee | 1.81 | Okeechobee | 1.79 |
| Baker | 1.72 | Hendry | 1.81 | Orange | 1.70 |
| Bay | 1.52 | Hernando | 1.62 | Osceola | 1.60 |
| Bradford | 1.72 | Highlands | 1.81 | Palm Beach | 1.55 |
| Brevard | 1.58 | Hillsborough | 1.69 | Pasco | 1.57 |
| Broward | 1.55 | Holmes | 1.77 | Pinellas | 1.44 |
| Calhoun | 1.53 | Indian River | 1.79 | Polk | 1.73 |
| Charlotte | 1.55 | Jackson | 1.77 | Putnam | 1.71 |
| Citrus | 1.63 | Jefferson | 1.53 | Saint Johns | 1.84 |
| Clay | 1.27 | Lafayette | 1.68 | Saint Lucie | 1.47 |
| Collier | 1.59 | Lake | 1.48 | Santa Rosa | 1.51 |
| Columbia | 1.72 | Lee | 1.48 | Sarasota | 1.46 |
| DeSoto | 1.81 | Leon | 1.57 | Seminole | 1.64 |
| Dixie | 1.68 | Levy | 1.68 | Sumter | 1.63 |
| Duval | 1.61 | Liberty | 1.53 | Suwannee | 1.68 |
| Escambia | 1.90 | Madison | 1.53 | Taylor | 1.53 |
| Flagler | 1.71 | Manatee | 1.73 | Union | 1.72 |
| Franklin | 1.53 | Marion | 1.75 | Volusia | 1.66 |
| Gadsden | 1.29 | Martin | 1.92 | Wakulla | 1.53 |
| Gilchrist | 1.68 | Miami-Dade | 1.76 | Walton | 1.77 |
| Glades | 1.81 | Monroe | 1.76 | Washington | 1.77 |
| Gulf | 1.53 | Nassau | 1.78 | FLORIDA | 1.63 |
| Hamilton | 1.68 | Okaloosa | 1.60 |  |  |

Source: U.S. DOT - National Household Travel Survey (NHTS) 2009 Florida Add-On

## Travel Time Reliability: Planning Time Index (Auto and Combination Truck)

Unlike the other speed-based measures, the computation of Planning Time Index (PTI) is based upon the granular speed dataset comprised of 15-minute epochs for each Source Book segment. While this dataset is unmodified for all-vehicle PTI, it is modified for combination truck PTI, as described below.

If the field-measured speed is at or above the posted speed limit plus 5 mph, then the combination truck travel speed was assumed to be 5 mph below field-measured speed. If the field-measured speed is at or below 60 mph, then the combination truck travel speed is assumed to be the same as the field-measured speed. Linear interpolation is used to estimate the combination truck travel speed for field-measured speed between 60 mph and the posted speed limit plus 5 mph. For example, if posted speed limit is 70 mph and the field-measured speed is 75 mph, then the combination truck speed is 70 mph (75 mph - 5 mph). If it is the same posted speed limit and the field-measured speed is 55 mph, then the combination truck speed is 55 mph (since it is below 60 mph).

The Planning Time Index (PTI) is the ratio of the 95th percentile travel time to the travel time at reference speed. The reference speed is defined as the 85th percentile unmodified speed during weekday off-peak hours (9 am to 4 pm and 7 pm to 10 pm). This applies to all vehicles and combination trucks. The following equation is used to calculate the PTI:

Equation 3

Planning Time Index= Travel Time95th percentileTravel Time@ reference speed

Weekend PTI is calculated using the same methodology and formula as indicated above, with the exception that the calculations are made for the weekends. The 95th percentile travel time is calculated during weekend peak period (12 – 2 PM) and for daily conditions. Reference speed is calculated as the 85th percentile unmodified speed during weekend off-peak hours (9 am to 4 pm and 7 pm to 10 pm).

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- HERE Technologies. Travel Time Data

## Average Travel Speed – Auto

Field-measured data are used to capture average speed for most segments for the 2021 Source Book. However, speed-volume functions are used to capture speeds where the probe data network has no coverage. Field-measured travel speeds are obtained from HERE, downloaded at the granular level. The granular level field-measured speeds are aggregated for each Source Book segment by hour, direction, day type, and congestion thresholds (i.e., heavily congested, mildly congested, and uncongested – see Error! Reference source not found.3).

The HERE dataset does not span all the State Highway System (SHS) and National Highway System (NHS) network, leaving several road segments without field-measured speeds. For segments where field-measured speeds are not available, speeds calculated from the speed-volume functions are used. The process for calculating speeds is described in Appendix C for freeways and multilane highways, Appendix D for arterials, and Appendix E for 2 lane highways.

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- HERE Technologies. Travel Time Data

## Average Speed vs. Posted Speed

This measure is calculated for arterials and highways by comparing the average peak hour speed against the posted speed limit. The output represents the overall percentage of the speed limit achieved in the peak hour. Average travel speed is the average of all peak hour segment travel speeds captured by probe data or modeled through speed-volume functions.

Equation 4

Average Speed vs. Posted Speed =VMT ×Peak Hour SpeedPosted Speed LimitVMT

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- HERE Technologies. Travel Time Data

## Vehicles per Lane Mile

The most commonly used mobility performance measure to describe utilization is vehicles per lane mile, which represents the average density on a roadway. It is calculated as the roadway segment’s peak hour volume divided by the number of lane miles weighted by the aggregate lane miles of each segment. Vehicle miles traveled is used in the aggregation of this measure rather than average annual daily traffic, since vehicle miles traveled describes the volume across a segment and is more suitable to the segmentation used for Florida’s highway systems. Vehicles per lane mile is reported only for peak hour.

#### Step 1: Obtain Peak Hour Vehicle Volume and Number of Lanes

Peak hour volume is the product of the AADT, the day of week variance factor and the highest hourly factor. Total number of lanes (in both directions) is obtained by summing the number of lanes on the right and left sides of the roadway segment, which are obtained directly from the RCI database.

#### Step 2: Obtain Vehicles per Lane and Vehicle per Lane Mile

Vehicles per lane mile is calculated as:

Equation 1

Vehicles per Lane = VolumeNumber of Lanes

Equation 5

Vehicles per Lane Mile= Volume Number of Lanes×Lane MilesLane Miles

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System) and Feature 212 (Through lanes)

## % Travel by Congestion Type

The percent of travel by congestion type is determined by summing the vehicle-miles traveled on roadways operating within each congestion category The congestion categories are defined in Table 6.3.

This measure is calculated as:

Equation 6

% Travel Heavily Congested= VMT when travel speeds are at or below heavily congested thresholdsVMT ×100

% Travel Mildly Congested= VMT when travel speeds are between uncongested and heavily congested thresholdsVMT ×100

% Travel Uncongested= VMT when travel speeds meet uncongested thresholdsVMT ×100

Use speed thresholds for congestion provided in Table 7. For each segment, calculate the percentage of observations in each speed bin (i.e. heavily congested, mildly congested, uncongested) and the respective average per bin.

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System) and Feature 311 (Speed limits)

- HERE Technologies. Travel Time Data

## % Miles by Congestion Type

The % of Miles by Congestion Type for all vehicles is determined by summing the miles of roadway in the congestion category and then dividing it by the total system miles.

Equation  computes the percent of miles heavily, mildly, and uncongested.

Equation 7

% Miles Heavily Congested= Segment length whose travel speeds are at or below heavily congested thresholdsSegment length ×100

% Miles Mildly Congested= Segment length whose travel speeds  are between uncongested and heavily congested thresholdsSegment length ×100

% Miles Uncongested= Segment length whose travel speeds are at or below uncongested thresholdsSegment length ×100

Use speed thresholds for congestion provided in Table 7.

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System) and Feature 311 (Speed limits)

- HERE Technologies. Travel Time Data

## % Time by Congestion Type

Percent time by congestion type accounts for the amount of time the facility is operating within the congested category. It is the percent of time in which segments operate at uncongested, mildly congested, and heavily congested conditions.

This measure uses the same speed thresholds by congestion type indicated in Table 7. To report a single duration of congestion metric, congestion durations from multiple roadways are weighted by the roadways’ lane miles, using Error! Not a valid bookmark self-reference..

Equation 8

For each roadway segment, the measure is computed as follows:

% Time Heavily Congested= Hours in a day or yearwhen travel speeds meet heavily congested threshold24 for daily measure or 8,760 for yearly measure ×100

% Time Mildly Congested= Hours in a day or year when travel speeds meet mildly congested threshold24 for daily measure or 8,760 for yearly measure ×100

% Time Uncongested= Hours in a day or yearwhen travel speeds meet uncongested threshold24 for daily measure or 8,760 for yearly measure ×100

For area-wide aggregation, the measure is computed as follows:

Epochs in a day or yearfor each congestion threshold*Miles for each congestion thresholdTotal Epochs in a day or year*Total Miles

Use speed thresholds for congestion provided in Table 6.3.

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System) and Feature 311 (Speed limits)

- HERE Technologies. Travel Time Data

## Duration of Congestion

In the 2021 Source Book, Duration of Congestion is a new measure to replace the Hours Heavily Congested (Daily and Annual) performance measure. The duration of congestion is taken as the number of hours categorized as heavily congested. Segments with at least 15-minutes of daily heavy congestion are selected as the portion of the system susceptible to heavy congestion. This measure is weighted by lane-miles, making the measure independent of segmentation. To account for directional travel, the direction of travel with more heavy congestion is chosen for the total duration measure.

Equation 9

Duration of Congestion=i=1# SegmentsLane Milesi×hours in a day experienceing heavy congestioni=1# SegmentsLane Milesi 
given that Segment i has at least 15 minutes of heavy congestion

#### Data Sources

- FDOT Transportation Data and Analytics Office, Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System)

- HERE Technologies, Travel Time Data

## Hours of Delay

In the 2021 Source Book, Vehicle Hours of Delay, Person Hours of Delay, and Combination Truck Hours of Delay are estimated on an hourly basis by determining the difference between delay threshold travel time and actual travel time along a facility. Delay threshold travel time/speed is considered the additional travel time experienced by a motorist beyond what would be experienced under uncongested conditions. The delay threshold speeds for the 2021 Source Book are provided in Table 7.3 below.

Table 7.2	Summary of Delay Threshold Speeds

| Area Type | Roadway Type | Uncongested |
| --- | --- | --- |
| Urbanized Areas | Arterial PSL ≤ 35 mph | > 22 mph |
|  | Arterial PSL ≥ 40 mph | > 31 mph |
|  | Two-Lane Highway | > 83.3% of FFS |
|  | Multilane Highway | > 83.3% of FFS |
|  | Freeway (PSL <50 mph) | > 83.3% of FFS |
|  | Freeway (PSL 55 mph) | > 50 mph |
|  | Freeway (PSL 60 mph) | > 55 mph |
|  | Freeway (PSL 65 mph) | > 60 mph |
|  | Freeway (PSL 70 mph) | > 65 mph |
| Non-Urbanized Areas | Arterial PSL ≤ 35 mph | > 22 mph |
|  | Arterial PSL ≥ 40 mph | > 31 mph |
|  | Two-Lane Highway | > 83.3% of FFS |
|  | Multilane Highway | > 83.3% of FFS |
|  | Freeway (PSL <50 mph) | > 83.3% of FFS |
|  | Freeway (PSL 60 mph) | > 55 mph |
|  | Freeway (PSL 65 mph) | > 60 mph |
|  | Freeway (PSL 70 mph) | > 65 mph |
|  | Freeway (PSL 75 mph) | > 70 mph |

Delay estimation considers unserved demand from the preceding hours for the time periods between 7:00 a.m. and 10:00 a.m. and between 4:00 p.m. and 7:00 p.m. During oversaturation, the undischarged queue—the difference between demand and capacity—from an oversaturated hour is added to the next hour. The process is repeated until demand (including that from previous hour(s)) is less than the capacity.

The following description summarizes the technique to calculate the Vehicle Hours of Delay. The calculations for obtaining Person Hours of Delay and Combination Truck Hours of Delay are similar, with the exception of multiplying by the average vehicle occupancy rate in the former and using the combination truck average travel speed in the latter.

#### Step 1: Obtain Hourly Directional Vehicle Volumes

AADT is first converted to hourly directional volumes, based on the day of week variance factor, hourly factor and directional factor (if directional AADT is not available). Hourly directional volumes used for delay calculation are adjusted for queue spill-back under over-saturation conditions for the time periods between 7:00 a.m. and 10:00 a.m. and between 4:00 p.m. and 7:00 p.m.

#### Step 2: Obtain Delay Threshold Travel Time

A segment’s delay threshold travel time is obtained by dividing the segment length by the threshold speeds presented above.

#### Step 3: Compute Hourly Directional Total Vehicle Delay

Average vehicle delay (in time units per vehicle) is computed as the difference between the actual travel time and the delay threshold travel time. It should be noted that when the actual travel time is less than the delay threshold travel time, there is no delay on the roadway segment. Total vehicle hours of delay (in vehicle-time units) are the product of average vehicle delay and the vehicle volume. The actual travel time is computed based on the average travel speed calculations described in the following section. On a single segment and for a single hour, the delay calculation is presented in the following equations:

Equation 10

Vehicle Hours of Delay =Vehicle Volume ×Travel Time-Travel Time at Uncongested Speeds

Equation 2

Person Hours of Delay =Average Vehicle Occupancy ×Vehicle Hours of Delay

Equation 3

Combination Truck Hours of Delay =Combination Truck Volume ×Combination Truck Travel Time-Travel Time at Uncongested Speeds

- Average vehicle occupancies are discussed in Section 7.2.

- Combination truck speed calculations are provided in Section 14.1.

#### Step 4: Obtain Daily Total Delay and Hours of Delay per Mile

The daily total hours of delay are the sum of hourly totals. The daily vehicle hours of delay are calculated as shown below:

Equation 13

Daily Vehicle Hours of Delay =t=124Total Hourly Vehicle Delayt

* t=a single hour, 24=24 hours

Equation 4

Daily Vehicle Hours of Delay per Mile= t=124Total Hourly Vehicle DelaytSegment Length ×100

#### Data Sources

- FDOT Transportation Data and Analytics Office, Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office, Roadway Characteristics Inventory Feature 147 (Strategic Intermodal System)

- HERE Technologies, Travel Time Data

- U.S. Department of Transportation (USDOT), National Household Travel Survey, Florida Add-On, 2009

## Job Accessibility by Auto

Accessibility is the ease of reaching valued destinations. It can be measured across different times of day (accessibility in the morning rush might be lower than the less-congested midday period). It can be measured for each mode (accessibility by walking is usually lower than accessibility by transit, which is usually lower than accessibility by car). There are a variety of ways to measure accessibility, but the number of destinations reachable within a given travel time is the most comprehensible and transparent as well as the most directly comparable across cities.

This measure uses the University of Minnesota Accessibility Observatory’s Access Across America, which estimates the accessibility to jobs by auto for each of the 11 million U.S. census blocks. Travel times are calculated using a detailed road network and speed data that reflect typical conditions for an 8 a.m. Wednesday morning departure.

Equation 14

Job AccessibilityAuto= Block Level Number of Jobs ×Number of WorkersNumber of Workers

#### Data Sources

- University of Minnesota Accessibility Observatory. National Accessibility Evaluation

# Safety Measures: Auto/Truck

The calculation methodology for each safety measure is discussed in this section.

## Number of Fatalities

This measure is the total number of fatalities on Florida's roadways that are a direct result of a traffic crash within thirty days of the crash occurrence.

Equation 15

Number of Fatalities= People killed from traffic crashes

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

- FDOT State Safety Office. Crash Analysis Reporting System (CARS) database as of December 13, 2021

## Number of Serious Injuries

This measure looks at the number of disabling/incapacitating injuries from traffic crashes that prevent the injured individuals from engaging in normal activities and usually requiring transport to a medical facility and hospitalization.

Equation 5

Number of Serious Injuries= people seriously injured or incapacitated from traffic crashes

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

- FDOT State Safety Office. Crash Analysis Reporting System (CARS) database as of December 13, 2021

## Rate of Fatalities

This measure is the total number of fatalities on Florida’s roadways per 100 million VMT.

Equation 17

Rate of Fatalities= FatalitiesAnnual VMT x 100 million

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

- FDOT State Safety Office. Crash Analysis Reporting System (CARS) database as of December 13, 2021

- FDOT Transportation Data and Analytics Office. Public Road Mileage and Travel Report, State Highway System Mileage and Travel Report

## Rate of Serious Injuries

This measure is the total number of serious injuries on Florida’s roadways per 100 million VMT.

Equation 18

Rate of Serious Injuries= Serious InjuriesAnnual VMT x 100 million

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles, Traffic Crash Facts Annual Report

- FDOT State Safety Office, Crash Analysis Reporting System (CARS) database as of December 13, 2021

- FDOT Transportation Data and Analytics Office, Public Road Mileage and Travel Report, State Highway System Mileage and Travel Report

## Motorcycle Fatalities and Serious Injuries

These safety measures capture the number of incapacitating injuries and fatalities of both motorcyclists and their passengers within thirty days of the crash occurrence.

Equation 19

Motorcycle Fatalities = Motocyclist and Passenger Fatalities

Motorcycle Serious Injuries = Motocyclist and passenger Serious Injuries

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

## Pedestrian Fatalities and Serious Injuries

This measure is the total number of pedestrian serious (incapacitating) injuries and fatalities that are reported on Florida’s public roads within thirty days of a crash.

Equation 6

Pedestrian Fatalities=Pedestrians killed from traffic crashes

Pedestrian Serious Injuries = Pedestrians Seriously Injuried incapacitatedfrom traffic crashes

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

## Bicycle Fatalities and Serious Injuries

This is the total number of bicyclist serious (incapacitating) injuries and fatalities on all of Florida's roadways within thirty days of the crash occurrence.

Equation 7

Bicycle Fatalities=Bicyclist and passenger Fatalities

Bicycle Serious  Injuries= Bicyclist and Passenger Serious Injuries

#### Data Sources

- Florida Department of Highway Safety and Motor Vehicles. Traffic Crash Facts Annual Report

## Safety Belt Use

FDOT conducts Florida’s annual statewide safety belt use survey. The statewide survey follows National Highway Traffic Safety Administration (NHTSA) procedures in determining the outboard, front-seat occupant belt use rate. Every five years, NHTSA requires that statewide surveys include newly sampled survey sites based on the most recent traffic fatality counts. These sites were selected randomly to represent all the traffic on various roadway types around the State of Florida. This is the percentage of driver and front seat passengers observed to be using a safety belt. Observations were recorded at 165 sites across 15 counties and occurred at random one-hour periods on all days of the week during daylight hours.

Equation 82

Percent Safety Belt Use= Number of Driver and Front Seat Passengers Using Safety BeltNumber of Observed Drivers and Front Seat Passengers

#### Data Sources

- FDOT State Safety Office

# People-Related Mobility Measures: Transit

The calculation methodology for each people-related transit mobility measure is discussed in this section.

## Transit Revenue Miles

Transit revenue miles is the number of annual miles of vehicle operation while in active service (available to pick up revenue passengers). This number is smaller than vehicle miles because of the exclusion of deadhead miles such as vehicle miles from the garage to the start of service, vehicle miles from the end of service to the garage, driver training, and other miscellaneous miles that are not considered to be in direct revenue service.

Equation 9

Transit Revenue Miles= Revenue miles of vehicle operation while in active service

#### Data Sources

- FDOT Public Transit Office, Florida Transit Information and Performance Handbook

## Transit Passenger Trips

Transit passenger trips are the annual number of passenger trips on the transit vehicles. A trip is counted each time a passenger boards a transit vehicle. Thus, if a passenger makes a transfer between buses to reach a destination, the passenger is counted as making two passenger trips.

Equation 10

Transit passenger Trips= Passengers Boarding Transit Vehicles

#### Data Sources

- FDOT Public Transit Office. Florida Transit Information and Performance Handbook

## Transit Revenue Miles between Failures

Transit revenue miles between failures is the number of total annual revenue miles divided by the number of revenue vehicle system failures, an indicator of the average frequency of delays caused by a problem with the equipment.

Equation 115

Transit Revenue Miles between Failures= Total Annual Revenue MilesNumber of Revenue Vehicle System Failures

#### Data Sources

- FDOT Public Transit Office. Florida Transit Information and Performance Handbook

## Transit Weekday Span of Service

Transit weekday span of service is the number of hours that transit service is provided on a representative weekday in the operation of the transit agency. This indicator is determined by computing the number of hours between the time service begins and the time service ends for an average weekday.

Equation 126

Transit Weekday Span of Service= Service SpansTransit Agencies

#### Data Sources

- FDOT Public Transit Office. Florida Transit Information and Performance Handbook

## Resident Access to Transit

This measure refers to the percentage of the population within a half-mile of fixed route transit.

Equation 27

Resident Access to Transit= Population within a half mile of TransitTotal Population

#### Data Sources

- FDOT Public Transit Office. Florida Transit Information and Performance Handbook

## Job Accessibility by Transit

Job accessibility is a “cumulative opportunity” metric — it reflects the total number of jobs reachable by transit within a travel time threshold. It is calculated for each census block, and the results are aggregated to provide a statewide average. Local accessibility levels depend not just on the availability and routes of transit service but also on their frequency. A traveler using transit service that runs more frequently will spend less time waiting. This analysis uses the 7:00 a.m. - 9:00 a.m. time period. The data are averaged across that window from minute-by-minute departures.

Equation 28

Job AccessibilityTransit= Block Level Number of Jobs ×Number of WorkersNumber of Workers

#### Data Sources

- University of Minnesota Accessibility Observatory. National Accessibility Evaluation

## Transit Passenger Trips per Revenue Mile

This measure is the ratio of annual passenger trips to total annual transit revenue miles of service, a key indicator of service effectiveness that is influenced by the levels of demand and the supply of service provided.

Equation 29

Transit Passenger Trips per Revenue Mile= Annual Transit Passenger TripsAnnual Transit Revenue Miles

#### Data Sources

- FDOT Public Transit Office. Florida Transit Information and Performance Handbook

# People-Related Mobility Measures: Pedestrian/Bicycle

This section discusses the calculation methodology for each pedestrian and bicycle mobility measure.

## % Pedestrian Facility Coverage

This measure reports the percentage of centerline miles of non-limited access SHS facilities in urban areas (5,000+ population) that have sidewalks and/or shared-use paths available to pedestrians. The pedestrian facilities include urban non-limited SHS roadways and a limited number of non-SHS roadways deemed of interest to FDOT with a sidewalk or a shared path on either side. In the formula below, urban non-limited access SHS centerline miles include those non-SHS roadway miles with pedestrian facilities.

Equation 13

% Pedestrian Facility Coverage= Pedestrian Facility Miles in Urban AreasUrban Non-Limited Access SHS Centerline Miles  ×100

#### Data Sources

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 216 (Bike Lanes/Pedestrian Facilities)

## % Bicycle Facility Coverage

This measure reports the percentage of centerline miles of non-limited access SHS facilities that have paved shoulders, or shared pathways available to bicyclists. The bicycle facilities include non-limited SHS roadways and a limited number of non-SHS roadways deemed of interest to FDOT, with a paved shoulder ≥ 4' and < 7' or a shared path on either side. In the formula below, non-limited access SHS centerline miles include those non-SHS roadway miles with bicycle facilities.

Equation 14

% Bicycle Facility Coverage= Miles of Bike FacilitiesNon-Limited Access SHS Centerline Miles ×100

#### Data Sources

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 216 (Bike Lanes/Pedestrian Facilities) and Feature 214 (Outside shoulder)

# People-Related Mobility Measures: Aviation

## Aviation Passenger Boardings

The total number of revenue passengers who board an aircraft at a Florida airport. If a passenger makes a transfer between planes to reach a destination within Florida, the passenger is counted as making two passenger boardings.

Equation 15

Aviation Passenger Boardings= Revenue Passengers Boarding Aircraft

#### Data Sources

- U.S. Federal Aviation Administration. Enplanements at All Airports (Primary, Non-primary Commercial Service, and General Aviation) by State and Airport

## Aviation Departure Reliability

Departure reliability is based on a flight’s take-off time from the gate. A departure is deemed reliable if the flight departs within 15 minutes of the scheduled time shown in the carrier’s Computerized Reservations Systems. In the aviation industry, this is commonly known as on-time departure. Departure reliability can be influenced by various factors such as heavy traffic volume, weather, and mechanical reasons beyond the control of an airport or an airline.

Equation 16

Aviation Departure Reliability= Departures<15 Minutes of ScheduleDepartures ×100

#### Data Sources

- U.S. Bureau of Transportation Statistics, On-Time Departure T-100 Segment

# People-Related Mobility Measures: Rail

## Rail Passengers

This measure provides the annual number of revenue-paying rail passengers. For the purpose of this performance measure, rail passengers include those riding on Amtrak, SunRail, and Tri-Rail.

Equation 17

Rail Passengers= Rail Passengers of Amtrak, SunRail and Tri-Rail

#### Data Sources

- Amtrak. Amtrak Fact Sheet

- Federal Transit Administration. National Transit Database (NTD) September 2021 Adjusted Monthly Ridership Database

## Passenger Rail On-time arrival

Rail on-time arrival captures the on-time performance of a passenger rail system operating within Florida. It represents percent of Amtrak, SunRail, and Tri-Rail trains that arrive on time. A train is considered on-time if arrival at endpoint is within a specified threshold timeframe of scheduled arrival time. The threshold timeframe varies based on the trip length. For long distance trips, the timeframe is wide (e.g. 30 minutes). For commuter trains and local heavy rail systems, the time frame is narrow (e.g. 15 minutes).

Equation 18

Rail On-Time Arrival= On-Time TrainsTrains ×100

#### Data Sources

- Amtrak. Amtrak Fact Sheet

- South Florida Regional Transportation Authority. South Florida Regional Transportation Agency’s Transit Development Plan (TDP)

- FDOT Freight & Multimodal Operations Office

# People-Related Mobility Measures: Seaport

## Seaport Passenger Movements

Seaport passenger movements refer to passengers embarking and disembarking at a Florida seaport. For example, when a cruise passenger departs on a cruise ship from PortMiami, they are counted. When the same cruise passenger arrives back at PortMiami, they are counted again.

Equation 19

Seaport Passenger Movements= Cruise Passengers embarking and disembarking cruise ships

#### Data Sources

- Florida Seaport Transportation and Economic Development (FSTED) Council. Five-Year Florida Seaport Mission Plan

# Freight-Related Mobility Measures: Truck

## Average Travel Speed – Combination Truck

The combination truck travel speed is estimated using field measured speed. The methodologies are different for arterials, freeways, and highways.

For arterials, if the HERE speed for all vehicles is at least 5 mph over the posted speed limit (PSL), then the combination truck travel speed is assumed to be the same as the PSL, which means combination truck travel speed is capped at the PSL. If the field-measured speed for all vehicles is 7 mph below the PSL or lower, then the combination truck travel speed is assumed to be the same as the field-measured speed. Linear interpolation is used to estimate the combination truck travel speed for field-measured speed between 7 mph below the PSL and the speed limit plus 5 mph.

For freeways or highways, if the field-measured speed for all vehicles is at the speed limit plus 5 mph, then the combination truck travel speed is assumed to be at the speed limit. Linear interpolation is used to estimate the combination truck travel speed when the field-measured speed is between 45 mph and the speed limit plus 5 mph. If the field-measured speed is at or below 45 mph, the combination truck travel speed is assumed to be the same as the field-measured speed.

The resulting adjusted speed is weighted by combination truck miles traveled (CTMT) for each roadway segment and the average travel speed for combination trucks is determined using the following equation:

Equation 37

Average Travel Speed Combination Trucks=CTMT ×Combination Truck Average Travel SpeedCTMT

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- HERE Technologies. Travel Time Data

## Combination Truck Cost of Delay

This measure monetizes the cost of delay experienced by the freight industry by (1) calculating the average marginal cost of labor per hour and (2) multiplying that by the number of hours of delay for combination trucks. The average marginal cost of labor per hour is the sum of marginal driver wages and driver benefits, obtained from the annual report titled An Analysis of the Operational Cost of Trucking, from 2019 American Transportation Research Institute (ATRI) data. The number of hours of delay for combination trucks is taken from the process described in Section 7.11.

Equation 38

Combination Truck Cost of Delay =Combination Truck Hours of Delay ×Average Marginal Cost of Labor per Hour

#### Data Sources

- FDOT Transportation Data and Analytics Office. Traffic Characteristics Inventory

- FDOT Transportation Data and Analytics Office. Roadway Characteristics Inventory, Feature 147 (Strategic Intermodal System)

- HERE Technologies. Travel Time Data

- American Transportation Research Institute. An Analysis of the Operational Costs of Trucking

# Freight-Related Mobility Measures: Aviation

The calculation methodology for each freight-related aviation mobility measure is discussed in this section.

## Aviation Tonnage

Aviation tonnage comes from the BTS T-100 Market report. This is the weight of all air cargo handled at Florida airports. This measure includes mail and freight cargo originated and terminated at Florida airports.

Equation 39

Aviation Tonnage= Tons Handled at All Florida Airports

#### Data Sources

- U.S. Bureau of Transportation Statistics (BTS). T100 Market (All Carriers)

# Freight-Related Mobility Measures: Space Measures

The calculation methodology for each space measure is discussed in this section.

## Space Launches and Sites

Space launches account for all the successful mission launches from Cape Canaveral Spaceport, excluding any test flights:

Equation 40

Space Launches = Orbital Launches

Space launch sites are all active launch sites in the state of Florida providing access to Earth’s orbit.

Number of Sites = Active Launch Sites

#### Data Sources

- Space Florida

## Space Payloads

Number of payloads describes the cargo and weight onboard spacecraft launches. A payload is the carrying capacity of a launch vehicle and may include cargo, passengers, a flight crew, scientific experiments, munitions, or other equipment:

Equation 41

Number of Payload = Civil, Commercial, and DoD paylaods

Payload Weight = Pounds in Orbit

#### Data Sources

- Space Florida

# Freight-Related Mobility Measures: Seaport

The calculation methodology for each freight-related seaport mobility measure is discussed in this section.

## Seaport Tonnage

Seaport tonnage is the international and domestic waterborne tons of cargo handled at both public and private terminals in port areas of Florida.

Equation 42

Seaport Tonnage= waterborne tons of cargo handled at Florida seaports

#### Data Sources

- FSTED Council. Five-Year Florida Seaport Mission Plan

## Seaport Twenty-Foot Equivalent Units

Twenty-Foot Equivalent Unit (TEU) represents the cargo capacity of a standard intermodal container, twenty feet long, eight feet tall, and eight feet wide. It is the standardized unit of the capacity of a container ship, or a container terminal.

Equation 20

Seaport TEUs= Twenty-Foot Equivalent Units

#### Data Sources

- FSTED Council. Five-Year Florida Seaport Mission Plan

###### Hourly Factors

| Time Period | Freeway Core |  | Freeway Urbanized |  | Freeway |  | Freeway Rural |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday | Weekend | Weekday | Weekend | Weekday | Weekend | Weekday | Weekend |
| 12 a.m. – 1 a.m. | 0.95% | 2.42% | 0.95% | 1.47% | 0.93% | 1.41% | 1.36% | 1.33% |
| 1 a.m. – 2 a.m. | 0.57% | 1.67% | 0.65% | 1.02% | 0.65% | 0.93% | 1.07% | 1.03% |
| 2 a.m. – 3 a.m. | 0.44% | 1.28% | 0.55% | 0.80% | 0.57% | 0.71% | 0.94% | 0.85% |
| 3 a.m. – 4 a.m. | 0.48% | 1.21% | 0.61% | 0.71% | 0.70% | 0.68% | 0.97% | 0.81% |
| 4 a.m. – 5 a.m. | 0.74% | 1.06% | 0.89% | 0.83% | 1.16% | 0.82% | 1.22% | 0.92% |
| 5 a.m. – 6 a.m. | 1.90% | 1.34% | 1.92% | 1.23% | 2.49% | 1.36% | 2.04% | 1.26% |
| 6 a.m. – 7 a.m. | 5.16% | 2.16% | 4.60% | 2.01% | 5.17% | 2.34% | 3.76% | 1.93% |
| 7 a.m. – 8 a.m. | 7.00% | 2.91% | 6.97% | 3.01% | 6.98% | 3.32% | 5.04% | 2.96% |
| 8 a.m. – 9 a.m. | 6.65% | 3.73% | 6.64% | 4.17% | 6.00% | 4.34% | 5.26% | 4.31% |
| 9 a.m. – 10 a.m. | 5.54% | 4.69% | 5.57% | 5.51% | 5.29% | 5.42% | 5.58% | 5.82% |
| 10 a.m. – 11 a.m. | 5.10% | 5.53% | 5.39% | 6.63% | 5.48% | 6.56% | 5.98% | 7.06% |
| 11 a.m. – 12 p.m. | 5.06% | 6.05% | 5.47% | 7.18% | 5.57% | 7.13% | 6.16% | 7.61% |
| 12 p.m. – 1 p.m. | 5.17% | 6.49% | 5.53% | 7.47% | 5.55% | 7.11% | 6.15% | 7.71% |
| 1 p.m. – 2 p.m. | 5.32% | 6.65% | 5.65% | 7.52% | 5.75% | 7.09% | 6.29% | 7.83% |
| 2 p.m. – 3 p.m. | 5.74% | 6.69% | 6.05% | 7.54% | 6.24% | 7.11% | 6.58% | 7.95% |
| 3 p.m. – 4 p.m. | 6.66% | 6.63% | 6.85% | 7.43% | 6.88% | 7.14% | 6.93% | 7.66% |
| 4 p.m. – 5 p.m. | 7.53% | 6.55% | 7.62% | 7.10% | 7.58% | 7.16% | 7.14% | 7.06% |
| 5 p.m. – 6 p.m. | 8.12% | 6.41% | 7.98% | 6.55% | 7.97% | 6.71% | 6.88% | 6.27% |
| 6 p.m. – 7 p.m. | 6.64% | 6.16% | 6.06% | 5.67% | 5.74% | 5.91% | 5.56% | 5.31% |
| 7 p.m. – 8 p.m. | 4.67% | 5.38% | 4.25% | 4.70% | 4.04% | 4.95% | 4.32% | 4.33% |
| 8 p.m. – 9 p.m. | 3.48% | 4.62% | 3.27% | 3.88% | 3.17% | 4.10% | 3.53% | 3.51% |
| 9 p.m. – 10 p.m. | 2.98% | 4.03% | 2.77% | 3.22% | 2.60% | 3.29% | 2.97% | 2.77% |
| 10 p.m. – 11 p.m. | 2.40% | 3.53% | 2.18% | 2.51% | 2.01% | 2.58% | 2.40% | 2.15% |
| 11 p.m. – 12 a.m. | 1.70% | 2.81% | 1.57% | 1.83% | 1.47% | 1.85% | 1.86% | 1.57% |

| Time Period | Arterial Urbanized (AU) |  | Arterial Non-Urbanized (ANU) |  | Two Lane Rural Highway (2HY) |  | Multilane Rural Highway (MHY) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weekday | Weekend | Weekday | Weekend | Weekday | Weekend | Weekday | Weekend |
| 12 a.m. – 1 a.m. | 0.66% | 1.25% | 0.66% | 1.12% | 0.50% | 1.14% | 0.74% | 1.23% |
| 1 a.m. – 2 a.m. | 0.43% | 0.85% | 0.43% | 0.70% | 0.33% | 0.62% | 0.55% | 0.85% |
| 2 a.m. – 3 a.m. | 0.36% | 0.59% | 0.36% | 0.56% | 0.29% | 0.43% | 0.53% | 0.69% |
| 3 a.m. – 4 a.m. | 0.39% | 0.46% | 0.39% | 0.40% | 0.45% | 0.36% | 0.61% | 0.60% |
| 4 a.m. – 5 a.m. | 0.69% | 0.47% | 0.69% | 0.42% | 0.96% | 0.55% | 1.06% | 0.69% |
| 5 a.m. – 6 a.m. | 1.76% | 0.86% | 1.76% | 0.89% | 2.18% | 1.10% | 2.54% | 1.27% |
| 6 a.m. – 7 a.m. | 4.27% | 1.68% | 4.27% | 1.57% | 4.40% | 1.99% | 5.28% | 2.14% |
| 7 a.m. – 8 a.m. | 6.42% | 2.88% | 6.42% | 2.59% | 6.46% | 3.14% | 6.48% | 2.94% |
| 8 a.m. – 9 a.m. | 5.92% | 4.63% | 5.92% | 3.78% | 5.58% | 4.67% | 5.67% | 4.19% |
| 9 a.m. – 10 a.m. | 5.51% | 5.85% | 5.51% | 5.44% | 5.67% | 6.55% | 5.45% | 5.63% |
| 10 a.m. – 11 a.m. | 5.70% | 7.20% | 5.70% | 6.59% | 6.06% | 7.46% | 5.64% | 6.72% |
| 11 a.m. – 12 p.m. | 6.10% | 7.99% | 6.10% | 6.96% | 6.25% | 7.69% | 5.89% | 7.23% |
| 12 p.m. – 1 p.m. | 6.41% | 8.71% | 6.41% | 8.03% | 6.33% | 7.93% | 6.07% | 7.61% |
| 1 p.m. – 2 p.m. | 6.38% | 7.97% | 6.38% | 7.73% | 6.46% | 7.69% | 6.29% | 7.74% |
| 2 p.m. – 3 p.m. | 6.72% | 7.63% | 6.72% | 7.65% | 7.00% | 7.51% | 6.60% | 7.70% |
| 3 p.m. – 4 p.m. | 7.41% | 7.52% | 7.41% | 7.58% | 7.43% | 7.49% | 7.22% | 7.76% |
| 4 p.m. – 5 p.m. | 7.77% | 7.05% | 7.77% | 7.40% | 7.81% | 7.20% | 7.76% | 7.34% |
| 5 p.m. – 6 p.m. | 7.92% | 6.59% | 7.92% | 7.20% | 7.76% | 6.78% | 7.96% | 6.81% |
| 6 p.m. – 7 p.m. | 5.90% | 5.44% | 5.90% | 6.60% | 5.80% | 5.66% | 5.74% | 5.85% |
| 7 p.m. – 8 p.m. | 4.23% | 4.65% | 4.23% | 5.42% | 4.03% | 4.58% | 3.87% | 4.74% |
| 8 p.m. – 9 p.m. | 3.27% | 3.43% | 3.27% | 4.41% | 3.10% | 3.45% | 2.89% | 3.74% |
| 9 p.m. – 10 p.m. | 2.63% | 2.76% | 2.63% | 3.27% | 2.33% | 2.61% | 2.27% | 2.88% |
| 10 p.m. – 11 p.m. | 1.90% | 2.05% | 1.90% | 2.23% | 1.64% | 1.90% | 1.68% | 2.14% |
| 11 p.m. – 12 a.m. | 1.27% | 1.45% | 1.27% | 1.48% | 1.17% | 1.48% | 1.23% | 1.51% |

###### Day of Week Calculation

Update Section Average Daily Traffic inputs to reflect DOW

Update K Factor* inputs to reflect weekday vs. weekend

Store results table

Run Source Book calculations

Aggregate results

Last day of week?

Move to next day of week

No

Yes

Start with day of week (DOW) = Monday

*Refer to Appendix A for the specific K-factors by time-of-day, facility type, and weekday/weekend day.

###### Freeway Speed Model

This Appendix discusses the modified Davidson Equation used in developing modeled speeds in the Source Book for freeway facilities.

- Modified Davidson Equation

The travel time function proposed by Davidson (1966, 1978) for transport planning purposes has been subject to much discussion and efforts of calibration and improvement including some controversy over the meaning of its parameters. In a presentation on the Development of Speed Models for Improving Travel Forecasting and Highway Performance Evaluation, Moses et al. (December 5, 2012) presented a piecewise modified Davidson volume-delay function for use in a study of SR 9/I-95 data in Pompano Beach, Florida. Table C.1 contains modified Davidson parameters applicable to freeways in Florida.

Equation 44

S= S01+JDVC1-VC      forVC≤μS01+JD×μ1-μ+JDVC-μ1-μ2   forVC>μ

Where:

| S | = predicted travel speed (mph) |
| --- | --- |
| So | = free flow speed (mph) |
| JD | = a delay parameter, |
| V | = volume (veh/h) |
| C | = capacity (veh/h) |
| µ | = saturation threshold parameter |

Table C.1	FDOT Parameter Values for Fitted Speed-Volume Functions

| Function | Parameter | Urban Speed Limit (mph) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Speed Limit ≤ 60 | 60 < Speed Limit ≤ 65 | 65 < Speed Limit ≤ 70 |
| Modified Davidson | JD | 0.0090 | 0.0092 | 0.0099 |
|  | µ | 0.9500 | 0.9490 | 0.9510 |

###### Arterial Speed Model

This Appendix discusses the methods used to calculated modeled speeds on arterial facilities at various degrees of saturation.

- Speed calculation when v/c < 1

For v/c < 1.0, speed calculations will be based on speeds given in the 2012 Generalized Level of Service Volume Tables. See Appendix F.

- Speed calculation when v/c ≥ 1

The Bureau of Public Roads (BPR) speed-flow curve is used to estimate speeds at various v/c ratios. The standard BPR curve generally drops too soon before capacity is reached, and then drops too slowly when demand exceeds capacity. For the Source Book, alpha and beta parameters developed in Florida from the Greater Treasure Coast Regional Planning Model (GTCRPM III) were used, and then interpolated to force predicted speeds equal to 15 mph when v/c = 1 and 7 mph when v/c =2.

The BPR equation is shown below:

Equation 45

S=Sf1+aVCb

Where:

| S | = predicted mean speed |
| --- | --- |
| Sf | = free flow speed |
| V | = volume |
| C | = capacity |
| b, a | = parameters |

Table D.1	BPR Parameters

| Source | High Speed Arterial (Speed Limit ≥ 40 mph) |  | Low Speed Arterial (Speed Limit ≤ 35 mph) |  |
| --- | --- | --- | --- | --- |
|  | Alpha | Beta | Alpha | Beta |
| Greater Treasure Coast Regional Planning Model (GTCRPM III) | 0.71 | 2.1 | 0.83 | 5.5 |

Table D.2	Interpolated Speed

| v/c | Predicted Speed by BPR | Interpolated Speed |
| --- | --- | --- |
| 1.0 | 26.32 | 15.00 |
| 1.1 | 24.10 | 13.83 |
| 1.2 | 22.05 | 12.75 |
| 1.3 | 20.16 | 11.76 |
| 1.4 | 18.45 | 10.86 |
| 1.5 | 16.89 | 10.04 |
| 1.6 | 15.49 | 9.30 |
| 1.7 | 14.22 | 8.63 |
| 1.8 | 13.08 | 8.03 |
| 1.9 | 12.05 | 7.49 |
| 2.0 | 11.13 | 7.00 |

###### Two-Lane Highway Speed Model

Chapter 15: Two-Lane Highways of the Highway Capacity Manual 6th Edition (HCM) provides guidance on estimating average speeds on two-lane highways. This memorandum summarizes the HCM methodology and provides advice relevant to the Source Book project.

The HCM methodology considers several characteristics of a two-lane highway, including lane width, percentage of combination trucks, and no-passing zones, etc. However, the HCM’s methodology is limited in its coverage of oversaturated conditions, with the HCM noting that it is rare to see two-lane highways operating at or near capacity (because most will have been expanded before demand reaches capacity), and thus there is a corresponding lack of data on oversaturated operations.

- Classification of Two-Lane Highways

Because two-lane highways are versatile roadways that can serve a multitude of purposes, the HCM has subdivided this type of facility into three distinct classes: Class I, Class II, and Class III (Figure E.1). A brief discussion of the different classes is relevant to the Source Book project, as they have different methodologies for calculating performance measures.

Figure E.1	Two-Lane Highway Classification

Source: Derived from HCM: Chapter 15: Two-Lane Highways. Percent Time Spent Following (PTSF) is the average percent of total travel time that vehicles must travel in platoons behind slower vehicles due to inability to pass on a two-lane highway. It therefore represents the freedom to maneuver and convenience of travel.

- Basic Relationship

Figure E.2 presents the basic relationship between flow, average travel speed, and percent time spent following another vehicle or trailer for two-lane highways in “base conditions”. Base conditions are defined by the HCM as the absence of restrictive geometric, traffic, or environmental factors. Therefore, base conditions are not to be confused with typical or default conditions. They more closely resemble ideal conditions.

For the 2021 Source Book, the two-lane highway speed calculations are based on speed-flow relationships with base conditions (Exhibit 15-2 of HCM 6th Edition).

Figure E.2	Base (Ideal) Speed-Flow Relationship in Two-Lane Highways

Source: HCM 6th Edition, Chapter 15

###### 2012 Generalized Service Volume Tables

Although the Generalized Service Volume tables were updated in 2021, the Source Book used the 2012 version, which was the current version available at the time of publication. The 2012 Generalized Service Volume tables were also used for consistency in computation methodology with historical results.

(Footnote continued on next page...)

For more information, see section 8.3.5 of RCI Handbook at https://ftp.fdot.gov/public/file/i9i09RG8GkK4hWk_IQSCiA/FDOT_RCI_Handbook_Chapter_8.pdf

If D factor is not available in the RCI database, then D factor = 0.55 is used