## INSPECTION.

(REV 2-10-94) (7-16)

SUBARTICLE 5-9.1 is expanded by the following:

Provide and maintain throughout the life of the Contract, two utility boats meeting or exceeding the following minimum requirements:

Utility Boat

(A) Size

(a) Length						19 feet

(b) Width (Beam)					7 feet

(c) Load Capacity					2,400 pounds

(d) Cabin

(1) Length					None

(2) Width					None

(3) Height	None

(B) Construction

(a) Bottom Plating					3/16 inch

(b) Side and Cabin Plating				1/8 inch

(c) Deck and Cockpit

Tread Plating					1/8 inch

(d) Plating Material					Marine Aluminum

(e) Seams and Joints					Welded

(C) Drive Power

(a) Outboard						two each, 75 HP,

total 150 HP

(b) Inboard-Outboard					None

(c) Fuel Tank						Required

(D) Equipment

(a) Mechanical Steering Control

Speed Control and Light

Control - Systems-Located

Forward					Required

(b) Electric Start Mechanism

Located Forward				Required

(c) Anchor and 300 foot Rope			20 pound

(d) Navigation Lights					Required

(e) Width of Shear Line

Rubber Bumper				3 inches

(f) Heavy Duty Seating

Accommodations				two each

Furnish fuel and docking facilities for these boats. The docking facilities’ location and adequacy are subject to the approval of the Engineer.

The Engineer and their staff will be permitted to travel on any or all floating equipment and boats used in the work performed.

Include the costs of furnishing and maintaining the boats, including fuel and docking facilities, in the Contract unit prices for other items of the Contract.