337
ASPHALT CONCRETE FRICTION COURSES.


(REV 5-6-02) (FA 1-4-02) (7-02)

SECTION 337 (Pages 309-317) is deleted and the following substituted:

SECTION 337
ASPHALT CONCRETE FRICTION COURSES

337-1 Description.


Construct an asphalt concrete friction course using the QC 2000 acceptance system as defined in these Specifications. This Section specifies mixes designated as FC-2, FC-3, FC-5, FC-9.5, and FC-12.5 (FC-6).


Meet the plant and equipment requirements of Section 320, as modified herein. Meet the general construction requirements of Section 330, as modified herein.

337-2 Materials.


337-2.1 General Requirements: Meet the requirements specified in Division III as modified herein. The Engineer will base continuing approval of material sources on field performance.


337-2.2 Asphalt Rubber Binder: Meet the requirements of Section 336, and any additional requirements or modifications specified herein for the various mixtures. For projects with a total quantity of FC-2, FC-3, FC-5, FC-9.5, or FC-12.5 (FC-6) less than 500 tons [450 metric tons], the Contractor may elect to substitute a PG 76-22 for the ARB-12 or ARB-5, meeting the requirements of 916-1.


337-2.3 Coarse Aggregate: Meet the requirements of Section 901, and any additional requirements or modifications specified herein for the various mixtures.


337-2.4 Fine Aggregate: Meet the requirements of Section 902, and any additional requirements or modifications specified herein for the various mixtures.


337-2.5 Hydrated Lime: Meet the requirements of AASHTO M303 Type 1.



Provide certified test results for each shipment of hydrated lime indicating compliance with the specifications.


337-2.6 Fiber Stabilizing Additive (Required for FC-5 only): Use either a mineral or cellulose fiber stabilizing additive. Meet the following requirements:



337-2.6.1 Mineral Fibers: Use mineral fibers made from virgin basalt, diabase, or slag treated with a cationic sizing agent to enhance the disbursement of the fiber, as well as to increase adhesion of the fiber surface to the bitumen. Meet the following requirements for physical properties:




1. Size Analysis

Average fiber length
0.25 inch [6.0 mm] (maximum)

Average fiber thickness
0.0002 inch [0.005 mm] (maximum)




2. Shot Content (ASTM C612)

Percent passing No. 60 [250 μm] Sieve
90 - 100

Percent passing No.230 [63 μm] Sieve
65 - 100




Provide certified test results for each batch of fiber material indicating compliance with the above tests.




337-2.6.1.1 Notice of Patented Process: Take notice that the use of mineral fibers treated with cationic sizing agent and the size analysis range for average fiber thickness are subject to U.S. Patent No. 4,613,376, held by Fiberand Corporation, 7150 Southwest 62nd Avenue, South Miami, Fl. 33143. Obtain all mineral fibers required to meet the FC-5 requirements of this Contract only from Fiberand Corporation or a duly authorized licensee of Fiberand. Assume responsibility, pursuant to 7-3, for obtaining any and all necessary rights to use such processes and pay any and all royalties, license fees or other costs incurred in order to meet the FC-5 requirements of this Contract. Include any and all royalties, license fees and other costs arising due to the existence of U.S. Patent No. 4,613,376 in the bid unit price for friction course FC-5.



337-2.6.2 Cellulose Fibers: Use cellulose fibers meeting the following requirements:

1. Fiber length
0.25 inch [6.0 mm] (maximum)




2. Sieve Analysis





a. Alpine Sieve Method

Percent passing No. 100 [150 μm] sieve
60-80





b. Ro-Tap Sieve Method

Percent passing No. 20 [850μm] sieve
80-95

Percent passing No. 40 [425μm] sieve
45-85

Percent passing No. 100 [150μm sieve
5-40

3. Ash Content:
18% non-volatiles (±5%)

4. pH:
7.5 (±1.0)

5. Oil Absorption:
5.0 (±1.0) (times fiber weight)

6. Moisture Content:
5.0 (maximum)




Provide certified test results for each batch of fiber material indicating compliance with the above tests.

337-3 General Composition of Mixes.


337-3.1 General: Use a bituminous mixture composed of aggregate (coarse, fine, or a mixture thereof), asphalt rubber binder, and in some cases, fibers and/or hydrated lime. Size, uniformly grade and combine the aggregate fractions in such proportions that the resulting mix meets the requirements of this Section. The use of RAP material will not be permitted.

337-3.2 Specific Component Requirements by Mix:

337-3.2.1 FC-2:



337-3.2.1.1 Aggregates: In addition to the requirements of Section 901, meet the following coarse aggregate requirements. Use either crushed granite, crushed slag, or lightweight aggregates approved by the Engineer. Crushed limestone from the Oolitic formation may be used if it contains a minimum of 12 % non-carbonate material as determined by FM 5-510 and the Engineer grants approval of the source prior to its use. Aggregates other than those listed above may be used if approved by the Engineer.




337-3.2.1.2 Asphalt Binder: Use an ARB-12 asphalt rubber.

337-3.2.2 FC-3:




337-3.2.2.1 Aggregates: In addition to the requirements of Section 901, meet the following coarse aggregate requirements. Use either crushed gravel, crushed granite, crushed slag, or crushed limestone from the Oolitic formation as specified for use in FC-2. Aggregates other than those listed above may be used if approved by the Engineer.





In addition to the requirements of Section 902, meet the following fine aggregate requirements. Use crushed screenings or a combination of crushed screenings and local materials. Use crushed screenings composed of hard, durable particles resulting from crushing or processing the coarse aggregate as specified above. Screenings from other approved sources may be used provided that the total of these screenings along with silica sand or local materials does not exceed 40%. Ensure that not more than 20% by weight of the total aggregate used is silica sand or local materials defined in Section 902.




337-3.2.2.2 Asphalt Binder: Use an ARB-5 asphalt rubber.

337-3.2.3 FC-5:




337-3.2.3.1 Aggregates: Use an aggregate blend which consists of either 100% crushed granite or 100% crushed Oolitic limestone.





In addition to the requirements of Section 901, meet the following coarse aggregate requirements. Use either crushed granite or crushed limestone. Use crushed limestone from the Oolitic formation, which contains a minimum of 12% non-carbonate material (as determined by FM 5-510), and has been approved for this use.





In addition to the requirements of Section 902, meet the following fine aggregate requirements. Use either crushed granite screenings, or crushed Oolitic limestone screenings for the fine aggregate.




337-3.2.3.2 Asphalt Binder: Use an ARB-12 asphalt rubber.




337-3.2.3.3 Hydrated Lime: Add the lime at a dosage rate of 1.0% by weight of the total dry aggregate to mixes containing granite.




337-3.2.3.4 Fiber Stabilizing Additive: Add either mineral fibers at a dosage rate of 0.4% by weight of the total mix, or cellulose fibers at a dosage rate of 0.3% by weight of total mix.

337-3.2.4 FC-9.5 and FC-12.5 (FC-6):




337-3.2.4.1: Aggregates: In addition to the requirements of Sections 901 and 902, use coarse and fine aggregate components which also meet the aggregate requirements for an SP-9.5 or SP-12.5 Superpave mix, respectively, as specified in Section 334.





Use an aggregate blend that consists of crushed granite, crushed Oolitic limestone, or a combination of the two. (Aggregates other than those listed above may be used if approved by the Engineer for use in friction courses.) Crushed limestone from the Oolitic formation may be used if it contains a minimum of 12% non-carbonate material as determined by FM 5-510 and the Engineer grants approval of the source prior to its use. As an exception, mixes that contain a minimum of 60% crushed granite may contain up to 40% fine aggregate from other approved sources.




337-3.2.4.2: Asphalt Binder: Use an ARB-5 asphalt rubber. If called for in the Contract, use a PG 76-22 meeting the requirements of 916-1.

337-3.3 Grading Requirements:



337-3.3.1 FC-2 and FC-3: Use a mixture with a gradation within the design range specified in Table 331-1.



337-3.3.2 FC-5: Use a mixture having a gradation at design within the ranges shown in Table 337-1.

	Table 337-1

FC-5 Gradation Design Range

	3/4 inch

[19.0 mm]
	1/2 inch [12.50 mm]
	3/8 inch [9.50 mm]
	No. 4 [4.75 mm]
	No. 8

[2.36 mm]
	No. 16

[1.18 mm]
	No. 30

[600 (m]
	No. 50

[300 (m]
	No.  100 [150 (m]
	No. 200 [75 (m]

	100
	85-100
	55-75
	15-25
	5-10
	--
	--
	--
	--
	2-4


Note: The No. 10 [2.00 mm] sieve may be used in lieu of the No. 8 sieve, with the same design range.



337-3.3.3 FC-9.5: Meet the design gradation requirements for a SP-9.5 Superpave mix passing above the restricted zone, as specified in Section 334.



337-3.3.4 FC-12.5 (FC-6): Meet the design gradation requirements for a SP-12.5 Superpave mix passing above the restricted zone, as specified in Section 334.

337-4 Mix Design.


337-4.1 FC-2 and FC-5: The Department will design the FC-2 and FC-5 mixtures. Furnish materials and the appropriate information (source, gradation, etc.) as specified in 331-4.3. The Department will have two weeks to design the mix.



The Department will establish the design binder content for FC-2 within the following ranges based on aggregate type:

	Aggregate Type
	Binder Content

% by weight of total mix

	Crushed Granite
	5.5-7.0

	Crushed Slag
	6.0-8.0

	Crushed Limestone (Oolitic)
	6.5-7.5

	Lightweight
	12.5-15.0


The Department will establish the design binder content for FC-5 within the following ranges based on aggregate type:

	Aggregate Type
	Binder Content

	Crushed Granite
	5.5 - 7.0

	Crushed Limestone (Oolitic)
	6.5 - 8.0



337-4.2 FC-3: Provide a mix design conforming to the requirements of 331-4.3. Submit data showing that the mix design meets the requirements of Table 331-2 using an ARB-5 meeting the requirements of Section 336.


337-4.3 FC-9.5 and FC-12.5 (FC-6): Provide a mix design conforming to the requirements of 334-3.2 for Traffic Level C unless otherwise designated in the plans. Develop the mix design using an ARB-5 meeting the requirements of Section 336.


337-4.4 Revision of Mix Design: For FC-5, FC-9.5 and FC-12.5 (FC-6), meet the requirements of 334-3.3. For FC-5, all revisions must fall within the gradation limits defined in Table 337-1.

337-5 Contractor’s Process Control.


Provide the necessary process control of the friction course mix and construction in accordance with the applicable provisions of 331-4.4 and 331-5.2 for FC-2 and FC-3, 330-2 and 334-6 for FC-5, and 330-2 and 334-6 for FC-9.5 and FC-12.5 (FC-6).


The Engineer will monitor the spread rate periodically to ensure uniform thickness. Provide quality control procedures for daily monitoring and control of spread rate variability. If the spread rate varies by more than 5% of the spread rate set by the Engineer in accordance with 337-8, immediately make all corrections necessary to bring the spread rate into the acceptable range.

337-6 Acceptance of the Mixture At the Plant.


337-6.1 FC-2 and FC-3: The FC-2 and FC-3 mixtures will be accepted in accordance with 331-6.

337-6.2 FC-9.5 and FC-12.5 (FC-6): The FC-9.5 and FC-12.5 (FC-6) mixtures will be accepted at the plant in accordance with 334-4. In addition, meet the requirements of 334-7.

337-6.3 FC-5: The FC-5 mixture will be accepted at the plant with respect to 334-4 with the following exceptions:



1. The mixture will be accepted with respect to gradation (P-3/8, P-4, and P-8), and asphalt binder content (Pb) only.



2. Testing in accordance with AASHTO TP4-00 and FM 1-T 209 (and conditioning prior to testing) will not be required as part of 334-4.1.



3. The standard lot size of FC-5 will be 2,000 tons [2,000 metric tons], with each lot subdivided into four equal sublots of 500 tons [500 metric tons] each.



4. Initial production requirements of 334-4.3 do not apply.



5. The Between-Laboratory Precision Values described in Table 334-4 are modified to include (P-3/8, P-4, and P-8) with a maximum difference per FM 1-T 030 (Figure 2).



6. Table 334-5 (Master Production Range) is replaced by Table 337-2.

	Table 337-2

FC-5 Master Production Range

	Characteristic
	Tolerance (1)

	Asphalt Binder Content (%)
	Target ± 0.60

	Passing 3/8 inch [9.50 mm] Sieve (%)
	Target ± 7.50

	Passing No. 4 [4.75 mm] Sieve (%)
	Target ± 6.00

	Passing No. 8 [2.36 mm] Sieve (%)
	Target ± 3.50


(1) Tolerances for sample size of n = 1 from the verified mix design



337-6.3.1 Individual Test Tolerances for FC-5 Production: In the event that an individual Quality Control test result of a sublot for gradation (P-3/8, P-4, and P-8), or asphalt binder content does not meet the requirements of Table 337-2, steps shall be taken to correct the situation and actions taken shall be reported to the Engineer.




In the event that two consecutive individual Quality Control test results for gradation (P-3/8, P-4, and P-8) or asphalt binder content do not meet the requirements of Table 337-2, the LOT will be automatically terminated and production of the mixture stopped until the problem is adequately resolved (to the satisfaction of the Engineer), unless it can be demonstrated to the satisfaction of the Engineer that the problem can immediately be (or already has been) resolved. Evaluate any material represented by the failing test result in accordance with 334-9.4.

337-7 Acceptance of the Mixture At the Roadway.


337-7.1 FC-2 and FC-5: The FC-2 and FC-5 mixtures will be accepted on the roadway with respect to surface tolerance in accordance with the applicable requirements of 330-12. No density testing will be required for these mixtures.


337-7.2 FC-3: The FC-3 mixture will be accepted on the roadway with respect to density and surface tolerance in accordance with 331-7.


337-7.3 FC-9.5 and FC-12.5 (FC-6): The FC-9.5 and FC-12.5 (FC-6) mixtures will be accepted on the roadway in accordance with 334-5.

337-8 Special Construction Requirements.


337-8.1 Hot Storage of FC-2 and FC-5 Mixtures: When using surge or storage bins in the normal production of FC-2 and FC-5, do not leave the mixture in the surge or storage bin for more than one hour.


337-8.2 Longitudinal Grade Controls for Open-Graded Friction Courses: On FC-2, do not use a longitudinal grade control (skid, ski, or traveling stringline). Use a joint matcher. On FC-5, use either longitudinal grade control (skid, ski or traveling stringline) or a joint matcher.

337-8.3 Temperature Requirements for FC-2:



337-8.3.1 Air Temperature at Laydown: Spread the mixture only when the air temperature, taken as the temperature in the shade away from artificial heat, is at or above 60ºF [15ºC].



337-8.3.2 Temperature of the Mixture: Heat and combine the asphalt rubber binder and aggregate in a manner which will produce a mixture having a temperature, when discharged from the plant, meeting the requirements of 330-6.3. Meet all requirements of 330-9.1.2 at the roadway.


337-8.4 Compaction of FC-2: Perform only seal rolling using a tandem steel-wheel roller. Do not allow the weight of the steel-wheel roller to exceed 135 lb/in (PLI) [2.4 kg/mm] of drum width.

Non SI Units

[image: image1.wmf](inches)

 

Drums

 

of

 Width 

Total

(pounds)

Roller 

 

of

 Weight 

Total

PLI

=


SI Units


[image: image2.wmf] 

rs)

 

(millimete

 

 

 

Drums

 

 

 

of

 

 Width 

 

Total

 

)

 

(kilograms

 

Roller 

 

 

 

of

 

 Weight 

 

Total

 

mm

 

kg

 

=

 




Perform seal rolling with a single coverage and with a nominal amount of overlap. Where the lane being placed is adjacent to a previously laid mat, do not pinch the longitudinal joint with the roller on the cold mat. Pinch the longitudinal joint with the roller on the mat being rolled, overlapping onto the cold mat by no more than 3 inches [75 mm]. Never allow a roller on the mat after completing the seal rolling.

337-8.5 Temperature Requirements for FC-3:



337-8.5.1 Air Temperature at Laydown: Spread the mixture only when the air temperature, taken in the shade away from artificial heat, is at or above 45ºF [7ºC].



337-8.5.2 Temperature of the Mixture: Heat and combine the asphalt rubber binder and aggregate in a manner which will produce a mixture having a temperature, when discharged from the plant, meeting the requirements of 330-6.3. Meet all requirements of 330-9.1.2 at the roadway.

337-8.6 Temperature Requirements for FC-5:



337-8.6.1 Air Temperature at Laydown: Spread the mixture only when the air temperature (the temperature in the shade away from artificial heat) is at or above 65ºF [18ºC]. As an exception, place the mixture at temperatures lower than 65ºF [18ºC], only when approved by the Engineer based on the Contractor’s demonstrated ability to achieve a satisfactory surface texture and appearance of the finished surface. In no case shall the mixture be placed at temperatures lower than 60ºF [16ºC].



337-8.6.2 Temperature of the Mix: Heat and combine the asphalt rubber binder and aggregate in a manner to produce a mix having a temperature, when discharged from the plant, meeting the requirements of 330-6.3. Meet all requirements of 330-9.1.2 at the roadway. The target mixing temperature shall be established at 320(F [160(C].


337-8.7 Compaction of FC-5: Provide two, static steel-wheeled rollers, with an effective compactive weight in the range of 135 to 200 PLI [2.4 to 3.6 kg/mm]as determined in 337-8.4. (Any variation of this equipment requirement must be approved by the Engineer.) Establish an appropriate rolling pattern for the pavement in order to effectively seat the mixture without crushing the aggregate. In the event that the roller begins to crush the aggregate, reduce the number of coverages or the PLI of the rollers. If the rollers continue to crush the aggregate, use a tandem steel-wheel roller weighing not more than 135 lb/in (PLI) [2.4 kg/mm] of drum width as determined in accordance with 337-8.4.

337-8.8. Temperature Requirements for FC-9.5 and FC-12.5 (FC-6):



337-8.8.1 Air Temperature at Laydown: Spread the mixture only when the air temperature (the temperature in the shade away from artificial heat) is at or above 45ºF [7ºC].



337-8.8.2 Temperature of the mix: Heat and combine the asphalt rubber binder and aggregate in a manner to produce a mix having a temperature, when discharged from the plant, meeting the requirements of 330-6.3. Meet all requirements of 330-9.1.2 at the roadway.


337-8.9 Prevention of Adhesion: To minimize adhesion to the drum during the rolling operations, the Contractor may add a small amount of liquid detergent to the water in the roller.



At intersections and in other areas where the pavement may be subjected to cross-traffic before it has cooled, spray the approaches with water to wet the tires of the approaching vehicles before they cross the pavement.


337-8.10 Transportation Requirements of Friction Course Mixtures: Cover all loads of friction course mixtures with a tarpaulin.

337-9 Thickness of Friction Courses.


The thickness of the friction courses will be based on the spread rate set by the Engineer. Plan quantities are based on the maximum spread rate within the ranges shown below. Pay quantities may be less, based on the spread rate set by the Engineer.


337-9.1 Spread Rate for FC-2: For FC-2 with granite, oolitic limestone, or other conventional aggregate, the Engineer will set the spread rate within the range of 50 - 60 lb/yd2 [27 - 34 kg/m2]. For FC-2 with lightweight aggregate, the Engineer will set the spread rate within the range of 28 - 35 lb/yd2 [15 - 19 kg/m2].


337-9.2 Spread rate for FC-3: The Engineer will set the spread rate within the range of 100 - 110 lb/yd2 [54 - 60 kg/m2].


337-9.3 Spread Rate of FC-5: The Engineer will set the spread rate within the range of 70 - 80 lb/yd2 [38 - 44 kg/m2].


337-9.4 Spread Rate of FC-9.5: The Engineer will set the spread rate within the range of 105 - 110 lb/yd2 [54 - 60 kg/m2].


337-9.5 Spread Rate of FC-12.5 (FC-6): The Engineer will set the spread rate within the range of 150 - 160 lb/yd2 [80 - 88 kg/m2].

337-10 Special Equipment Requirements for FC-5.


337-10.1 Fiber Supply System: Use a separate feed system to accurately proportion the required quantity of mineral fibers into the mixture in such a manner that uniform distribution is obtained. Interlock the proportioning device with the aggregate feed or weigh system to maintain the correct proportions for all rates of production and batch sizes. Control the proportion of fibers to within plus or minus 10% of the amount of fibers required. Provide flow indicators or sensing devices for the fiber system, interlocked with plant controls so that the mixture production will be interrupted if introduction of the fiber fails.



When a batch plant is used, add the fiber to the aggregate in the weigh hopper or as approved and directed by the Engineer. Increase the batch dry mixing time by 8 to 12 seconds, or as directed by the Engineer, from the time the aggregate is completely emptied into the pugmill. Ensure that the fibers are uniformly distributed prior to the addition of asphalt rubber into the pugmill.



When a drum-mix plant is used, add and uniformly disperse the fiber with the aggregate prior to the addition of the asphalt rubber. Add the fiber in such a manner that it will not become entrained in the exhaust system of the drier or plant.


337-10.2 Hydrated Lime Supply System: For FC-5 mixes containing granite, use a separate feed system to accurately proportion the required quantity of hydrated lime into the mixture in such a manner that uniform coating of the aggregate is obtained prior to the addition of the asphalt rubber. Add the hydrated lime in such a manner that it will not become entrained in the exhaust system of the drier or plant. Interlock the proportioning device with the aggregate feed or weigh system to maintain the correct proportions for all rates of production and batch sizes and to ensure that all mixture produced is properly treated with hydrated lime. Control the proportion of hydrated lime to within plus or minus 10% of the amount of hydrated lime required. Provide and interlock flow indicators or sensing devices for the hydrated lime system with plant controls so that the mixture production will be interrupted if introduction of the hydrated lime fails. The addition of the hydrated lime to the aggregate may be accomplished by Method (A) or (B) as follows:



337-10.2.1 Method (A) - Dry Form: Add hydrated lime in a dry form to the mixture according to the type of asphalt plant being used.




When a batch plant is used, add the hydrated lime to the aggregate in the weigh hopper or as approved and directed by the Engineer. Increase the batch dry mixing time by eight to twelve seconds, or as directed by the Engineer, from the time the aggregate is completely emptied into the pugmill. Uniformly distribute the hydrated lime prior to the addition of asphalt rubber into the pugmill.




When a drum-mix plant is used, add and uniformly disperse the hydrated lime to the aggregate prior to the addition of the asphalt rubber. Add the hydrated lime in such a manner that it will not become entrained in the exhaust system of the drier or plant.



337-10.2.2 Method (B) - Hydrated Lime/Water Slurry: Add the required quantity of hydrated lime (based on dry weight) in a hydrated lime/water slurry form to the aggregate. Provide a solution consisting of hydrated lime and water in concentrations as directed by the Engineer. Use a plant equipped to blend and maintain the hydrated lime in suspension and to mix it with the aggregates uniformly in the proportions specified.

337-11 Compensation (FC-5, FC-9.5, and FC-12.5 (FC-6) only).


337-11.1 FC-9.5 and FC-12.5 (FC-6): Meet the requirements of 334-8.


337-11.2 FC-5: Meet the requirements of 334-8 with the following exceptions:



1. Pay factors will be calculated for asphalt binder content and the percentages passing the 3/8 inch [9.50 mm], the No. 4 [4.75 mm], and the No. 8 [2.36 mm] sieves only.



2. Table 337-4 replaces Table 334-6.



3. Table 337-5 replaces Table 334-7.



4. The Composite Pay Factor in 334-8.3 is replaced with the following:

CPF = 
[(0.20 x PF 3/8 inch [9.50 mm]) + (0.30 x PF No. 4 [4.75 mm]) + (0.10 x PF No. 8 [2.36 mm]) + (0.40 x PF AC)]

	Table 337-4

Small Quantity Pay Table for FC-5

	Pay Factor
	1-Test Deviation
	2-Test Average Deviation

	Asphalt Binder Content (%)

	1.00
	0.00-0.50
	0.00-0.35

	0.90
	0.51-0.60
	0.36-0.42

	0.80
	>0.60
	>0.42

	3/8 inch [9.50 mm] Sieve (%)

	1.00
	0.00-6.50
	0.00-4.60

	0.90
	6.51-7.50
	4.61-5.30

	0.80
	>7.50
	>5.30

	No. 4 [4.75 mm] Sieve (%)

	1.00
	0.00-5.00
	0.00-3.54

	0.90
	5.01-6.00
	3.55-4.24

	0.80
	>6.00
	>4.24

	No. 8 [2.36 mm] Sieve (%)

	1.00
	0.00-3.00
	0.00-2.12

	0.90
	3.01-3.50
	2.13-2.47

	0.80
	>3.50
	>2.47


	Table 337-5

Specification Limits for FC-5

	Quality Characteristic
	Specification Limits

	Asphalt Binder Content (%)
	Target ± 0.45

	Passing 3/8 inch [9.50 mm] sieve (%)
	Target ( 6.00

	Passing No. 4 [4.75 mm] sieve (%)
	Target ( 4.50

	Passing No. 8 [2.36] sieve (%)
	Target ( 2.50


337-12 Low Quality Material (FC-5, FC-9.5, and FC-12.5 (FC-6) only).


Meet the requirements of 334-9. For FC-5, use the Master Production Range defined in Table 337-2 in lieu of Table 334-5.

337-13 Method of Measurement.


For the work specified under this Section (including the pertinent provisions of Sections 320 and 330), the quantity to be paid for will be the weight of the mixture, in tons [metric tons]. The pay quantity will be based on the average spread rate for the project, limited to a maximum of 105% of the spread rate set by the Engineer in accordance with 337-9.


The bid price for the asphalt mix will include the cost of the asphalt binder (asphalt rubber (or polymer), asphalt cement, ground tire rubber, anti-stripping agent, blending and handling), as well as fiber stabilizing additive and hydrated lime (if required). There will be no separate payment or unit price adjustment for the asphalt binder material in the asphalt mix. The weight will be determined as provided in 320-2 (including the provisions for the automatic recordation system).

337-14 Basis of Payment.


Price and payment will be full compensation for all the work specified under this Section (including the applicable requirements of Sections 320 and 330).


Payment will be made under:

Item No.   337-  7-
Asphaltic Concrete Friction Course -per ton.

Item No. 2337-  7-
Asphaltic Concrete Friction Course -per metric ton.

_1072757234.doc


rs)







(millimete







 







Drums







 







of







 Width 







Total







)







(kilograms







Roller 







 







of







 Weight 







Total







mm







kg




















