PREVENTION, CONTROL AND ABATEMENT OF EROSION AND WATER POLLUTION.

(REV 6-10-03) (7-03)

ARTICLE 104-5 (Pages 118 and 119) is deleted and the following substituted:

104-5 Preconstruction Requirements.


At the Preconstruction Conference, provide to the Department an Erosion Control Plan meeting the requirements or special conditions of all permits authorizing project construction. If no permits are required or the approved permits do not contain special conditions or specifically address erosion and water pollution, the project Erosion Control Plan will be governed by 7‑1.1, 7‑2.2, 7‑8.1, 7‑8.2, and Section 104.


When a DEP generic permit is issued, the Contractor’s Erosion Control Plan shall be prepared to accompany the Department’s Stormwater Pollution Prevention Plan (SWPPP). Ensure the Erosion Control Plan includes procedures to control off-site tracking of soil by vehicles and construction equipment and a procedure for cleanup and reporting of non‑storm water discharges, such as contaminated groundwater or accidental spills. Do not begin any soil disturbing activities until Department approval of the Contractor’s Erosion Control Plan, including required signed certification statements.


Failure to sign any required documents or certification statements will be considered a default of the Contract. Any soil disturbing activities performed without the required signed documents or certification statements may be considered a violation of the DEP Generic Permit.


When the SWPPP is required, prepare the Erosion Control Plan in accordance with the planned sequence of operations and present in a format acceptable to the Department. The Erosion Control Plan shall describe, but not be limited to, the following items or activities:



(1) For each phase of construction operations or activities, supply the following information:




(a) Locations of all erosion control devices




(b) Types of all erosion control devices




(c) Estimated time erosion control devices will be in operation




(d) Monitoring schedules for maintenance of erosion control devices




(e) Methods of maintaining erosion control devices




(f) Containment or removal methods for pollutants or hazardous wastes



(2) The name and telephone number of the person responsible for monitoring and maintaining the erosion control devices.



(3) Submit for approval the Erosion Control Plans meeting paragraphs 3a, 3b, or 3c below:




(a) Projects permitted by the Southwest Florida Water Management District (SWFWMD), require the following:





Submit a copy of the Erosion Control Plan to the Engineer for review and to the appropriate SWFWMD Office for review and approval. Include the SWFWMD permit number on all submitted data or correspondence.





The Contractor may schedule a meeting with the appropriate SWFWMD Office to discuss his Erosion Control Plan in detail, to expedite the review and approval process. Advise the Engineer of the time and place of any meetings scheduled with SWFWMD.





Do not begin construction activities until the Erosion Control Plan receives written approval from both SWFWMD and the Engineer.




(b) Projects permitted by the South Florida Water Management District or the St. Johns River Water Management District, require the following:





Obtain the Engineer’s approval of the Erosion Control Plan.





Do not begin construction activities until the Erosion Control Plan receives written approval from the Engineer.




(c) Projects authorized by permitting agencies other than the Water Management Districts or projects for which no permits are required require the following:





The Engineer will review and approve the Contractor’s Erosion Control Plan.





Do not begin construction activities until the Erosion Control Plan receives written approval from the Engineer.





Comply with the approved Erosion Control Plan.

SUBARTICLE 104-7.1 (Page 123) is deleted and the following substituted:


104-7.1 General: Provide routine maintenance of permanent and temporary erosion control features, at no expense to the Department, until the project is complete and accepted. If reconstruction of such erosion control features is necessary due to the Contractor’s negligence or carelessness or, in the case of temporary erosion control features, failure by the Contractor to install permanent erosion control features as scheduled, the Contractor shall replace such erosion control features at no expense to the Department. If reconstruction of permanent or temporary erosion control features is necessary due to factors beyond the control of the Contractor, the Department will pay for replacement under the appropriate Contract pay item or items.



Inspect all erosion control features at least once every seven calendar days and within 24 hours of the end of a storm of 0.50 inches [12 mm] or greater. Maintain all erosion control features as required in the Stormwater Pollution Prevention Plan, Contractor’s Erosion Control plan and as specified in the State of Florida Department of Environmental Protection Generic Permit for Stormwater Discharge from Large and Small Construction Activities.

