INSPECTION.

(REV 2-10-94) (7-00)

SUBARTICLE 5-9.1 (Page 44) is expanded by the following:



Provide and maintain throughout the life of the Contract, two utility boats meeting or exceeding the following minimum requirements:

Utility Boat

(A) Size


(a) Length





19 feet

(b) Width (Beam)




7 feet

(c) Load Capacity




2,400 pounds

(d) Cabin



(1) Length




None



(2) Width




None



(3) Height
None

(B) Construction


(a) Bottom Plating




3/16 inch

(b) Side and Cabin Plating



1/8 inch


(c) Deck and Cockpit



Tread Plating




1/8 inch

(d) Plating Material




Marine Aluminum


(e) Seams and Joints




Welded

(C) Drive Power


(a) Outboard





two each, 75 HP,









total 150 HP 


(b) Inboard-Outboard




None


(c) Fuel Tank





Required

(D) Equipment


(a) Mechanical Steering Control



Speed Control and Light



Control - Systems-Located



Forward




Required


(b) Electric Start Mechanism



Located Forward



Required


(c) Anchor and 300 foot Rope


20 pound

(d) Navigation Lights




Required


(e) Width of Shear Line



Rubber Bumper



3 inches

(f) Heavy Duty Seating



Accommodations



two each


Furnish fuel and docking facilities for these boats. The docking facilities’ location and adequacy are subject to the approval of the Engineer.


The Engineer and his staff will be permitted to travel on any or all floating equipment and boats used in the work performed.


Include the costs of furnishing and maintaining the boats, including fuel and docking facilities, in the Contract unit prices for other items of the Contract.

