


LANDSCAPE INSTALLATION.
(REV 3-15-13) (4-13)

ARTICLE 580-2 (Page 766) is deleted and the following substituted:
580-2 Materials.


580-2.1 Plants:



580-2.1.1 Grade Standards and Conformity with Type and Species: Only use nursery grown plant materials that comply with all required inspection, grading standards, and plant regulations in accordance with the latest edition of the Florida Department of Agriculture’s “Grades and Standards for Nursery Plants.”




Unless otherwise specified, minimum grade for all plants is Florida No. 1 or better. All plants must be the specified size and grade at the time of delivery to the site and the minimum grade maintained throughout the project construction period and plant establishment period.




Use only plants that are true to type and species and ensure that the plants not specifically covered by Florida Department of Agriculture’s “Grades and Standards for Nursery Plants” conform in type and species with the standards and designations in general acceptance by Florida nurseries. 




A minimum of two plants of each species on each shipment must be shipped with tags stating the botanical nomenclature and common name of the plant. Should discrepancies between botanical nomenclature and common name arise, the botanical name will take precedence.



580-2.1.2 Inspection and Transporting: Move nursery stock in accordance with all Federal and State regulations and accompany each shipment with the required inspection certificates for filing with the Engineer.


580-2.2 Water: Meet the requirements of Section 983.


580-2.3 Mulching: Use of cypress mulch prohibited.

