7100401

All Jobs


7100401 PAINTED PAVEMENT MARKINGS

COMMENTS FROM INTERNAL/INDUSTRY REVIEW
******************************************************************************

Pat McCann
954 254-8317
pmccann@targetengineering.com
Comments: (6-23-15)

710-4.1.1 "Wait at least 14 days after the first application of painted pavement markings...." Suggest deleting the word "first" from this sentence as it implies a second application of paint. Only one coat of paint is needed in this case.

Response: Agree, change made.
******************************************************************************

Paul Gentry CO
850-414-4118
paul.gentry@dot.state.fl.us
Comments: 6-25-15)

Section 710-4.1.1 Painted Pavement Markings (Final Surface) states that when permanent pavement markings (I.e., thermoplastic or permanent tape), are placed on concrete surfaces or newly constructed asphalt without rumble striping, the painted pavement markings (final surface) will include one application of standard paint and one application of Class B pavement markers applied to the final surface. Wait the 14 days after the first application to apply the thermoplastic. The above statement would be in conflict with 711-4.1 General (3rd paragraph) that states “before applying traffic stripes to any Portland cement concrete surface, apply a primer, sealer or surface preparation adhesive of the type recommended by the manufacturer. Having tested all preformed thermoplastic presently on the APL, I know they require a “clean” surface with a sealer application prior to the installation on asphalt. I know this requirement is also required by the APL alkyd thermoplastic manufacturers prior to their product’s installation. A coat of waterborne paint is not considered a primer, sealer or surface preparation adhesive. If thermoplastic is going to be used on concrete, the marking surface needs to be waterblasted prior to application to ensure a clean surface. 

Response:  I agree the paint on concrete is not a recommended sealer.  This has nothing to do with the requirements of 710-4.1.1.  Section 711 and 713 has language that would require the existing marking to be removed to apply the thermoplastic or tape.
******************************************************************************

David Villani
678-558-1660
davev@ennisflint.com
Comments: (6-29-15)

The proposed 710-4.1.1 implies that thermoplastic can be placed on top of temporary paint on concrete. We do not recommend this as waterborne paint is not a concrete primer. This also goes against 711-4.1 which says: Before applying traffic stripes and markings, remove any material by a method approved by the Engineer that would adversely affect the bond of the traffic stripes. Before applying traffic stripes to any portland cement concrete surface, apply a primer, sealer or surface preparation adhesive of the type recommended by the manufacturer. 

Response:  A temporary marking must to be installed prior to opening the road to traffic.  This section covers these temporary pavement markings.  When the permanent markings are installed, the sections for those materials is applicable (i.e. 701-4.1, 709-4.1, 711-4.1 and 713-4.1) which would require the paint to be removed and sealers.  No changes made.
******************************************************************************

D5 Construction

Comments: (7-6-15)

A second application of standard paint.............................Operation. Wait at least 14 days after..........................markings (final surface) to apply the thermoplastic. Comments: What happens when there is no thermoplastic to be placed and there is only the two coats of paint?

Response:  This is covered in the third paragraph.  No changes made.
******************************************************************************

