|  | FDOT Traffic Engineering Research Laboratory (TERL) Automated Flagger Assistance Device (AFAD) Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Section 990 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Section 990 and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) |  | TERL Evaluation Method |
| --- | --- | --- | --- | --- | --- | --- |
| The following compliance matrix criteria are for all AFADs. |  |  |  |  |  |  |
| 1 | 990-3.1 | Automated Flagger Assistance Device (AFAD) meets the requirements of the Manual on Uniform Traffic Control Devices (MUTCD) for Streets and Highways. |  | Provide documentation specified in MUTCD line items below. |  | Document Review, Physical Inspection and Functional Inspection |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for STOP/SLOW AFADs. |  |  |  |  |  |  |
| 2 | MUTCD Section 6L.03.02 | The AFAD’s STOP/SLOW sign has an octagonal shape, is fabricated of rigid material, and is mounted with the bottom of the sign a minimum of 6 feet above the pavement on an appropriate support. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 3 | MUTCD Section 6L.03.02 | The size of the STOP/SLOW sign is at least 24 x 24 inches with letters at least 8 inches high. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 4 | MUTCD Section 6L.03.02 | The background of the STOP face is red with white letters and border. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 5 | MUTCD Section 6L.03.02 | The background of the SLOW face is diamond-shaped and orange with black letters and border. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 6 | MUTCD Section 6L.03.02 | Both faces of the STOP/SLOW sign are retroreflectorized. |  | Indicate Approved Product List number for sign sheeting in this field. |  | Compliance Matrix Review |
| 7 | MUTCD Section 6L.03.03 | The AFAD’s STOP/SLOW sign has a means to positively lock, engage, or otherwise maintain the sign assembly in a stable condition when set in the STOP or SLOW position. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 8 | MUTCD Section 6L.03.04 | The AFAD’s STOP/SLOW sign is supplemented with active conspicuity devices by incorporating either flashing lights or beacons. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for STOP/SLOW AFADs with flashing lights. |  |  |  |  |  |  |
| 9 | MUTCD Section 6D.02.07 | If flashing lights are used on the STOP face of the paddle, their colors are all red. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| 10 | MUTCD Section 6D.02.07 | If flashing lights are used on the SLOW face of the paddle, the colors are yellow. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| 11 | MUTCD Section 6D.02.08 | If more than eight flashing lights are used, the lights are arranged such that they clearly convey the octagonal shape of the STOP face of the paddle and/or the diamond shape of the SLOW face of the paddle. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 12 | MUTCD Section 6D.02.09 | If flashing lights are used on the STOP/SLOW paddle, the flash rate is at least 50, but not more than 60, flashes per minute. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for STOP/SLOW AFADs with beacons. |  |  |  |  |  |  |
| 13 | MUTCD Section 6L.03.04 | If a Stop Beacon is used, it is mounted a maximum of 24 inches above the STOP face. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 14 | MUTCD Section 6L.03.04 | If used, a Warning Beacon is mounted a maximum of 24 inches above, below, or to the side of the SLOW face. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 15 | MUTCD Section 6L.03.04 | The Stop Beacon is not flashed or illuminated when the SLOW face is displayed, and the Warning Beacon is not flashed or illuminated when the STOP face is displayed. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| 16 | MUTCD Section 4S.01.03 | Beacons are flashed at a rate of not less than 50 or more than 60 times per minute. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| The following compliance matrix criteria are for all AFADs. |  |  |  |  |  |  |
| 17 | 990-3.1 | All assembly hardware less than 5/8 inch in diameter is type 304 or 316 passivated stainless steel. Stainless steel bolts, screws and studs meet ASTM F593 and nuts meet ASTM F594. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. |  | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 18 |  | All assembly hardware greater than or equal to 5/8 inch in diameter is galvanized. Bolts, studs, and threaded rod meet ASTM A307 and structural bolts meet ASTM F3125, A325. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. |  | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 19 |  | All electronic assemblies meet the environmental requirements of NEMA TS-4. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. - A completed NEMA TS4 2.2.3-2.2.6, 3.1.1 & 5.4 Checklist. |  | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 20 |  | The controller and associated on-board circuitry are not affected by mobile radio, or any other radio transmissions |  | Provide a statement of conformance in this field. |  | Compliance Matrix Review |
| 21 |  | An operator’s manual is furnished with each unit. |  | Provide a statement of conformance in this field. |  | Compliance Matrix Review |
| 22 |  | AFAD is marked with the manufacturer’s name or trademark, model/part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 23 |  | Portable devices and trailers are delineated on a permanent basis by affixing retroreflective sheeting in a continuous line on the face of the trailer as seen by oncoming road users. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 24 | 990-3.1.1 | Batteries are protected from overcharging and over-discharging. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 25 |  | An external battery charge level indicator is provided. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 26 |  | Automatic recharging of batteries is provided using a charge controller that includes charging status and battery charge level indicators. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| 27 |  | An AC/DC battery charger unit is provided. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 28 |  | Batteries, charge controllers, and power panels are protected from the elements and vandalism. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 29 | 990-3.1.1.1 | Solar power systems provide automatic recharging of power supply batteries to normal operating levels with meters showing charge. |  | Applicant may provide comments in this field. |  | Physical Inspection and Functional Inspection |
| 30 | 990-3.1.1.2 | Electrical system batteries are designed to provide 2 days of continuous operation without sunlight with a minimum of onsite maintenance. |  | Provide instructions on how to disconnect solar array and cellular connections. Also, provide minimum starting voltage required for the autonomy testing. |  | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 31 | 990-3.1.3 | Controller and control panel are housed in a weather, dust, and vandal resistant lockable cabinet. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 32 | 990-3.1.4 | The support chassis is self-contained and self-supporting without the use of additional equipment or tools. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 33 |  | The sign, power supply unit and all support systems are mounted on a trailer or non-trailer unit |  | Applicant may provide comments in this field. |  | Physical Inspection |
| The following compliance matrix criteria are for trailer mounted AFADs. |  |  |  |  |  |  |
| 34 |  | The trailer is equipped with class-A lights, using a plug adaptor. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 35 |  | The trailer is equipped with adjustable outrigger leveling pads, one on each of the four frame corners. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 36 |  | The trailer can be set up at the site with its own chassis and outriggers, without being hitched to a vehicle. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| 37 |  | The trailer is equipped with fenders over the tires. |  | Applicant may provide comments in this field. |  | Physical Inspection |
| 38 |  | The trailer meets all equipment specifications set forth in Chapter 316 of the Florida Statutes, and by such rule, regulation or code that may be adopted by the Department of Highway Safety and Motor Vehicles. |  | Applicant may provide comments in this field. |  | Functional Inspection |
| The following compliance matrix criteria (referencing Florida Statutes (F.S.)) are from Chapter 316, F.S. and are for trailers. |  |  |  |  |  |  |
| 39 | F.S. Section 316.221(1) | The trailer is equipped with at least two taillamps mounted on the rear, which, when lighted as required in 316.217, F.S., emit a red light plainly visible from a distance of 1,000 feet to the rear. |  | Applicant may provide comments in this field. | Functional Inspection |  |
| 40 | F.S. Section 316.221(2) | Either a taillamp or a separate lamp is placed as to illuminate with a white light the rear registration plate of the trailer and render it clearly legible from a distance of 50 feet from the rear. |  | Applicant may provide comments in this field. | Functional Inspection |  |
| 41 | F.S. Section 316.222(1) | The trailer is equipped with two or more stop lamps meeting the requirements of 316.234(1), F.S. |  | Applicant may provide comments in this field. | Functional Inspection |  |
| 42 | F.S. Section 316.222(2) | The trailer is equipped with electric turn signal lamps meeting the requirements of 316.234(2), F.S. |  | Applicant may provide comments in this field. | Functional Inspection |  |
| The following compliance matrix criteria are for all AFADs. |  |  |  |  |  |  |
| 43 | 990-3.7 | Typical application of the device is in accordance with Standard Plans, Index 102-603. |  | Provide detailed drawings that demonstrate compliance with this requirement. |  | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 44 |  | Electronic assemblies meet the requirements of NEMA TS 5 Section 4. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist - A completed NEMA TS5 13.1.3-13.1.6 Checklist. |  | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| The following compliance matrix criteria are for STOP/SLOW AFADs. |  |  |  |  |  |  |
| 45 | 990-3.7.2 | STOP/SLOW AFAD is provided with a STOP/SLOW sign that alternately displays the stop face and the slow face of a STOP/SLOW paddle. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 46 |  | If the AFAD includes a gate arm, the gate arm descends to a down position across the approach lane of traffic when the stop face is displayed and ascends upright when the slow face is displayed. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 47 |  | Gate arm is fully retro reflectorized on both sides, with vertical alternating red and white stripes at 16-inch intervals measured horizontally in accordance with the MUTCD. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 48 |  | When the arm is in the down position, the vertical aspect of the arm and sheeting is a minimum of 2 inches and the end of the arm reaches at minimum to the center of the through lane being controlled. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| The following compliance matrix criteria are for Red/Yellow Lens AFADs. |  |  |  |  |  |  |
| 49 | 990-3.7.3 | Red/Yellow Lens AFAD includes remotely operated Red/Yellow Lens that alternately displays a steadily illuminated circular red lens and a flashing circular yellow lens to control traffic. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 50 |  | AFAD includes a gate arm that descends to a position across the approach lane of traffic when the steady circular red lens is illuminated and then ascends to an upright position when the flashing circular yellow lens is illuminated. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 51 |  | Gate arm is fully retro reflectorized on both sides, with vertical alternating red and white stripes at 16-inch intervals measured horizontally in accordance with the MUTCD. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 52 |  | When the arm is in the down position, the vertical aspect of the arm and sheeting is a minimum of 2 inches and the end of the arm reaches at minimum to the center of the through lane being controlled. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 53 |  | There is no change interval between the solid red and flashing yellow indication. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |
| 54 |  | AFAD provides a steady illuminated circular yellow indication, with at least a 5 second duration, between the transition from flashing circular yellow indication and the display of the steady circular red indication. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. |  | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |  |

Document History for:

Automated Flagger Assistance Device Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Initial compliance matrix | D. Bremer | C. Morse D. Vollmer | J. Morgan | 01/25/2013 | N/A |
| 2.0 | Modified disclaimer to indicate compliance matrix is governing document. | A. Burleson | J. Morgan | J. Morgan | 05/09/2013 | No |
| 3.0 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | A. Burleson K. Moser | J. Morgan | J. Morgan | 10/31/2014 | No |
| 4.0 | Minor wording to bring to latest FHWA approved spec. Spec is currently out for FHWA approval, but there are no proposed changes in AFAD area. | D. Bremer | M. DeWitt | J. Morgan | 08/19/2015 | No |
| 5.0 | Updated to reflect latest FA approval date of 8-11-15. | A. Burleson | J. Morgan | J. Morgan | 02/05/2016 | No |
| 6.0 | Updated to reflect latest FA approval date of 10-31-19. | W. Geitz | M. DeWitt J. Morgan | D. Vollmer | 03/25/2020 | No |
| 7.0 | Update AFAD CM to reflect changes in battery operation length and remove dimming requirement, based on latest FHWA approved spec. | W. Geitz | C. Raimer M. DeWitt | D. Vollmer | 12/29/2020 | No |
| 8.0 | Updated to reflect FA 2-22-21. No changes to this CM. | W. Geitz | C. Raimer M. DeWitt | M. DeWitt | 06/03/2022 | No |
| 9.0 | Updated to latest FA date of 12-5-23 for spec 990. Removed signed and sealed drawing requirement for CM ID 26. | W. Geitz | P. Blaiklock | D. Vollmer | 01/03/2024 | No |
| 10.0 | Added MUTCD requirements. Removed date for referenced standards. Updated to latest FA date of 8-16-24 for spec 990. | W. Geitz | D. Bremer | M. DeWitt | 05/06/2025 | No |