|  | FDOT Traffic Engineering Research Laboratory (TERL) Portable Arrow Board Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Section 990 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Section 990 and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |  |  |
| --- | --- | --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

|  |  | ** Greyed out rows in table below are for TERL use only ** |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| The following compliance matrix criteria are for all arrow boards. |  |  |  |  |  |
| 1 | 990-3.1 | Device meets the physical display and operational requirements of the Manual on Uniform Traffic Control Devices (MUTCD) for Streets and Highways. |  | Applicant may provide comments in this field. | Document Review, Physical Inspection and Functional Inspection |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for all arrow boards. |  |  |  |  |  |
| 2 | MUTCD Section 6L.06.01 | The arrow board includes a matrix of elements capable of either flashing or sequential displays. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 1), PAB003 (Steps 1-3) |  |  | Init.: |
| 3 | MUTCD Section 6L.06.08 | All arrow boards are finished in non-reflective black. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 2), PAB002 (Step 1) |  |  | Init.: |
| 4 |  | The arrow board is mounted on a vehicle or a trailer. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 2) |  |  | Init.: |
| 5 | MUTCD Section 6L.06.11 | Arrow board elements are capable of at least a 50 percent dimming from full brilliance. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB004 (Steps 3, 5) |  |  | Init.: |
| 6 | MUTCD Section 6L.06.13 | The arrow board has suitable elements capable of the various operating modes. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB003 (Steps 1-3) |  |  | Init.: |
| 7 |  | The color presented by the elements is yellow. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 3), PAB003 (Step 4) |  |  | Init.: |
| 8 | MUTCD Section 6L.06.15 | The minimum element on time is 50 percent for the flashing mode, with equal intervals of 25 percent for each sequential phase. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 4), PAB003 (Steps 6, 7) |  |  | Init.: |
| 9 |  | The flashing rate is not less than 25 or more than 40 flashes per minute. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 5), PAB003 (Step 8) |  |  | Init.: |
| 10 | MUTCD Section 6L.06.16 | An arrow board has the following three mode selections: - A. A Flashing Arrow, Sequential Arrow, or Sequential Chevron mode; - B. A flashing Double Arrow mode; and - C. A flashing Caution or Alternating Diamond mode. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 6), PAB003 (Steps 1-3) |  |  | Init.: |
| The following compliance matrix criteria are for all arrow boards. |  |  |  |  |  |
| 11 | 990-3.1 | All assembly hardware less than 5/8 inch in diameter is type 304 or 316 passivated stainless steel. Stainless steel bolts, screws and studs meet ASTM F593 and nuts meet ASTM F594. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 7) |  |  | Init.: |
| 12 |  | All assembly hardware greater than or equal to 5/8 inch in diameter is galvanized. Bolts, studs, and threaded rod meet ASTM A307 and structural bolts meet ASTM F3125, Grade A325. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 8) |  |  | Init.: |
| 13 |  | Electronic assemblies meet the environmental requirements of NEMA TS 4. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. - A completed NEMA TS4 2.2.3-2.2.6, 3.1.1 & 5.4 Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 9) |  |  | Init.: |
| 14 |  | The controller and associated on-board circuitry are not affected by mobile radio, or any other radio transmissions. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): PAB001 (Step 10) |  |  | Init.: |
| 15 |  | An operator’s manual is furnished with each unit. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): PAB001 (Step 11) |  |  | Init.: |
| 16 |  | Portable arrow board is permanently marked with manufacturer’s name or trademark, model/part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 3) |  |  | Init.: |
| 17 |  | Portable devices and trailers are delineated on a permanent basis by affixing retroreflective sheeting in a continuous line on the face of the trailer as seen by oncoming road users. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 4) |  |  | Init.: |
| 18 | 990-3.1.1 | Batteries are protected from overcharging and over-discharging. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 12) |  |  | Init.: |
| 19 |  | An external battery charge level indicator is provided. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 13), PAB002 (Step 5) |  |  | Init.: |
| 20 |  | Automatic recharging of batteries is provided using a charge controller that includes charging status and battery charge level indicators. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 6) |  |  | Init.: |
| 21 |  | An AC/DC battery charger unit is provided. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 7) |  |  | Init.: |
| 22 |  | Batteries, charge controllers, and power panels are protected from the elements and vandalism. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 8) |  |  | Init.: |
| 23 | 990-3.1.1.1 | Solar power systems provide automatic recharging of power supply batteries to normal operating levels with meters showing charge. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB006 (Step 3) |  |  | Init.: |
| 24 |  | Solar array recovery time is accomplished in a maximum of three hours. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB006 (Step 3) |  |  | Init.: |
| 25 |  | Arrow board is designed to provide 180 days of continuous operation with minimum onsite maintenance. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): PAB001 (Step 14) |  |  | Init.: |
| 26 | 990-3.1.1.2 | Electrical system batteries are designed to provide 21 days of continuous operation without sunlight with a minimum of onsite maintenance. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 15), PAB005 (Step 5) |  |  | Init.: |
| 27 | 990-3.1.2 | The display housing assembly is weather tight. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): PAB001 (Step 16) |  |  | Init.: |
| 28 |  | The display assembly is equipped with an automatic dimming operational mode capable of a minimum of 50 percent dimming and a separate manual dimmer switch. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 17), PAB004 (Step 2) |  |  | Init.: |
| 29 |  | The display panel background and frame for the display assembly is painted flat black and meets Federal Specification TT-E-489. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 18) |  |  | Init.: |
| 30 |  | The display panel, when raised in the upright position, has a minimum height of 7 feet from the bottom of the panel to the ground. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 19), PAB002 (Step 9) |  |  | Init.: |
| 31 |  | The unit has an accessible mechanism to easily raise and lower the display assembly; a locking device is also provided to ensure the display panel will remain in the raised or lowered position. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 20), PAB007 (Step 2) |  |  | Init.: |
| 32 | 990-3.1.3 | Controller and control panel are housed in a weather, dust, and vandal resistant lockable cabinet. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 21), PAB002 (Step 10) |  |  | Init.: |
| 33 |  | The sign control software provides an on-site graphical representation that visibly depicts the message displayed on the sign face. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 22), PAB003 (Step 5) |  |  | Init.: |
| 34 | 990-3.1.4 | The support chassis is self-contained and self-supporting without the use of additional equipment or tools. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 11) |  |  | Init.: |
| The following compliance matrix criteria are for trailer mounted arrow boards. |  |  |  |  |  |
| 35 |  | The sign, power supply unit and all support systems are mounted on a wheeled trailer. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 12) |  |  | Init.: |
| 36 |  | The trailer is equipped with class-A lights, using a plug adaptor. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 23), PAB002 (Step 13) |  |  | Init.: |
| 37 |  | The trailer is equipped with adjustable outrigger leveling pads, one on each of the four frame corners. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 14) |  |  | Init.: |
| 38 |  | The trailer can be set up at the site with its own chassis and outriggers, without being hitched to a vehicle. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 15) |  |  | Init.: |
| 39 |  | The trailer is equipped with fenders over the tires. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 16) |  |  | Init.: |
| 40 |  | The trailer meets all equipment specifications set forth in Chapter 316 of the Florida Statutes, and by such rule, regulation or code that may be adopted by the Department of Highway Safety and Motor Vehicles. |  | Applicant may provide comments in this field. | Functional Inspection |
| The following compliance matrix criteria (referencing Florida Statutes (F.S.)) are from Chapter 316, F.S. and are for trailers. |  |  |  |  |  |
| 41 | F.S. Section 316.221(1) | The trailer is equipped with at least two taillamps mounted on the rear, which, when lighted as required in 316.217, F.S., emit a red light plainly visible from a distance of 1,000 feet to the rear. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB007 (Step 7) |  |  | Init.: |
| 42 | F.S. Section 316.221(2) | Either a taillamp or a separate lamp is placed as to illuminate with a white light the rear registration plate of the trailer and render it clearly legible from a distance of 50 feet from the rear. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB007 (Step 6) |  |  | Init.: |
| 43 | F.S. Section 316.222(1) | The trailer is equipped with two or more stop lamps meeting the requirements of 316.234(1), F.S. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB007 (Step 4) |  |  | Init.: |
| 44 | F.S. Section 316.222(2) | The trailer is equipped with electric turn signal lamps meeting the requirements of 316.234(2), F.S. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): PAB007 (Step 5) |  |  | Init.: |
| The following compliance matrix criteria are for all arrow boards. |  |  |  |  |  |
| 4545 | 990-3.2.1 | Arrow board is either: Type B which is 30 by 60 inches with a minimum of 13 lamps and a minimum legibility distance of 3/4 mile; or Type C which is 48 by 96 inches with a minimum of 15 lamps and a minimum legibility distance of 1 mile. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review, Physical Inspection and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Steps 24, 25), PAB002 (Steps 17, 18), PAB007 (Steps 10, 11) |  |  | Init.: |
| 46 | 990-3.2.1 | The lamp lens is 5 3/4 inches in diameter. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 26), PAB002 (Step 19) |  |  | Init.: |
| 47 |  | Lens includes a 360-degree hood for close-up glare reduction. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): PAB002 (Step 20) |  |  | Init.: |
| 48 |  | The bulbs provide a 350-candle power intensity for day use and an automatic reduction or dimming capacity for night use; the dimmed night operation provides adequate indication without excessive glare. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): PAB001 (Step 27), PAB004 (Step 5) |  |  | Init.: |

Document History for:

Portable Arrow Board Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Conversion from Excel to Word and adding evaluation criteria. | D. Bremer | D. Vollmer C. Morse | J. Morgan | 05/09/2013 | N/A |
| 2.0 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | A. Burleson K. Moser | J. Morgan | J. Morgan | 10/31/2014 | No |
| 3.0 | Updating to latest FHWA. There is a current FHWA review, but the proposed changes do not affect arrow board. | D. Bremer | M. DeWitt C. Morse | J. Morgan | 08/19/2015 | No |
| 4.0 | Updated to reflect latest FA approval date of 8-11-15. | A. Burleson | J. Morgan | J. Morgan | 02/05/2016 | No |
| 5.0 | Updated to reflect latest FA approval date of 8-20-18. | R. Brooks | J. Morgan F. Deasy | J. Morgan | 07/03/2019 | No |
| 6.0 | Updated to reflect latest FA approval date of 10-31-19 | W. Geitz | M. DeWitt J. Morgan | D. Vollmer | 03/27/2020 | No |
| 7.0 | Updated FA Date and replaced trailer specification to reflect compliance to Chapter 16 of the Florida Statutes. | W. Geitz | C. Raimer M. DeWitt | D. Vollmer | 12/29/2020 | No |
| 8.0 | Updated FA Date to 2/22/2021. No changes to the CM. | W. Geitz | C. Raimer | D. Vollmer | 05/27/2022 | No |
| 9.0 | Updated to latest FA date of 8-16-24 for spec 990. | W. Geitz | D. Bremer | M. DeWitt | 05/06/2025 | No |
| 10.0 | Added test cases/steps and made other changes throughout document. | A. Burleson A. Cramer | W. Geitz | M. DeWitt | 10/30/2025 | No |
| 11.0 | Updated to latest FA date of 8-28-25 for spec 990. | A. Cramer | A. Burleson | W. Geitz | 12/23/2025 | No |