|  | FDOT Traffic Engineering Research Laboratory (TERL) Video Vehicle Detection System (VVDS) Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Sections 660 and 995 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Sections 660 and 995, and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| --- | --- | --- | --- | --- | --- |
| The following compliance matrix criteria are for all VVDS. |  |  |  |  |  |
| 1 | 995-1.1 | Equipment is permanently marked with manufacturer name or trademark, part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): VVDS002 (Step 1) |  |  | Init.: |
| 2 | 995-2.1 | All parts are made of corrosion-resistant materials, such as UV stabilized or UV resistant plastic, stainless steel, anodized aluminum, brass, or gold-plated metal. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 1) |  |  | Init.: |
| 3 |  | All fasteners exposed to the elements are Type 304 or 316 passivated stainless steel. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 2) |  |  | Init.: |
| 4 |  | If the assembly includes a cabinet, the cabinet meets requirements of Section 676. |  | Provide the applicable compliance matrix from Specification 676 along with all supporting documentation that demonstrates this product meets this requirement, based on cabinet type. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 3), VVDS002 (Step 2) |  |  | Init.: |
| 5 |  | Detector meets the environmental requirements of NEMA TS 2. |  | Provide a third party test report that demonstrates the device performs all required functions during and after being subjected to the environmental testing as described in NEMA TS2 section 2.2.7, 2.2.8, and 2.2.9. Also, provide a completed Testing Laboratory and Report Checklist and NEMA TS2 2.2.7-2.2.9 Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 4) |  |  | Init.: |
| 6 | 995-2.3.1 | VVDS is provided with software that allows local and remote configuration and monitoring. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 5), VVDS004 (Step 1) |  |  | Init.: |
| 7 |  | VVDS system can display detection zones and detection activations overlaid on live video inputs. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 6), VVDS004 (Step 2) |  |  | Init.: |
| 8 |  | VVDS allows a user to edit previously defined configuration parameters, including size, placement, and sensitivity of detection zones. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 7), VVDS004 (Step 3) |  |  | Init.: |
| 9 |  | VVDS retains its programming in nonvolatile memory. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 8), VVDS004 (Step 4) |  |  | Init.: |
| 10 |  | VVDS configuration data can be saved to a computer and restored from a saved file. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 9), VVDS004 (Step 7) |  |  | Init.: |
| 11 |  | All communication addresses are user programmable. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 5), VVDS003, VVDS004 (Step 4) |  |  | Init.: |
| 12 |  | VVDS software offers an open Application Programming Interface (API) and software development kit available to the Department at no cost for integration with third party software and systems. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 10), VVDS004 (Step 9) |  |  | Init.: |
| 13 | 995-2.3.2 | Cameras that are integrated and included in a VVDS are compliant with the Code of Federal Regulations Section 200.216 Prohibition on certain telecommunications and video surveillance services or equipment. |  | Provide a signed letter of conformance that the cameras are compliant with the Code of Federal Regulations Section 200.216. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 11) |  |  | Init.: |
| 14 | 995-2.3.3 | VVDS includes a machine vision processor that allows video analysis, presence detection, data collection, and interfaces for inputs and outputs as well as storage and reporting of collected vehicle detection data. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS004 (Step 10) |  |  | Init.: |
| 15 | 995-2.3.4 | VVDS includes a minimum of one serial or Ethernet communications interface. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 13), VVDS002 (Step 3) |  |  | Init.: |
| The following compliance matrix criteria are for VVDS with serial interface. |  |  |  |  |  |
| 16 |  | Interface and connector conform to Telecommunications Industry Association (TIA)-232 standards. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 14), VVDS002 (Step 4) |  |  | Init.: |
| 17 |  | Serial ports support data rates up to 115200 bps; error detection utilizing parity bits (i.e., none, even, and odd); and stop bits (1 or 2). |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 15), VVDS003 (Step 1) |  |  | Init.: |
| The following compliance matrix criteria are for VVDS with Ethernet interface. |  |  |  |  |  |
| 18 |  | Ethernet interfaces provide a 10/100 Base TX connection. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): VVDS003 (Step 3) |  |  | Init.: |
| 19 |  | All unshielded twisted pair/shielded twisted pair network cables and connectors comply with TIA 568. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 16) |  |  | Init.: |
| The following compliance matrix criteria are for wireless communications. |  |  |  |  |  |
| 20 |  | Wireless communications are secure and FCC certified. The FCC identification number is displayed on an external label and all VVDS devices operate within the FCC frequency allocation. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 17), VVDS002 (Step 5) |  |  | Init.: |
| The following compliance matrix criteria are for VVDS with cellular communications. |  |  |  |  |  |
| 21 |  | Cellular communication devices are compatible with the cellular carrier used by the agency responsible for system operation and maintenance. |  | Provide product literature, specifications, user manual, or similar information that describes any cellular devices that are part of the system and indicates carrier(s) supported. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 18) |  |  | Init.: |
| The following compliance matrix criteria are for all VVDS. |  |  |  |  |  |
| 22 |  | System can be configured and monitored via one or more communications interface. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 19) |  |  | Init.: |
| 23 | 995-2.3.5 | Analog video inputs and outputs utilize BNC connectors. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): VVDS002 (Step 6) |  |  | Init.: |
| 24 | 995-2.3.6 | Solid state detection outputs meet the requirements of NEMA TS 2. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 20), VVDS004 (Step 11) |  |  | Init.: |
| 25 | 995-2.3.7 | System operates using a nominal input voltage of 120 volts of alternating current (VAC) and with an input voltage ranging from 89 to 135 VAC. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): VVDS005 (Steps 1-3) |  |  | Init.: |
| 26 |  | If any system device requires an operating voltage other than 120 VAC, a voltage converter is supplied. |  | Environmental test reports must demonstrate that voltage converters required for 120VAC operation were subjected to NEMA TS2 environmental testing as part of the functional system. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 21), VVDS002 (Step 7) |  |  | Init.: |
| The following compliance matrix criteria are for VVDS to be used as presence detectors. |  |  |  |  |  |
| 27 | 995-2.9 | Detector provides a minimum detection accuracy of 98% when calculated in accordance with all criteria as detailed in 995-2.9 and all subsections therein. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 22), VVDS008 |  |  | Init.: |
| 28 |  | Detector meets the requirements for modes of operation in NEMA TS2. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 23), VVDS004 (Steps 5-7) |  |  | Init.: |
| The following compliance matrix criteria are for VVDS to be used as traffic data detectors. |  |  |  |  |  |
| 29 | 995-2.10 | Vehicle detection meets the minimum total roadway segment accuracy levels of 95 % for volume, 90% for occupancy, and 90% for speed for all lanes, up to the maximum number of lanes that the device can monitor as specified by the manufacturer in accordance with 660-4.2.1 Traffic detection data is calculated in accordance with all criteria as detailed in 995-2.10 and all subsections therein. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 23), VVDS006 (Steps 1-7), VVDS007 |  |  | Init.: |
| The following compliance matrix criteria are for VVDS to be used for advance detection. |  |  |  |  |  |
| 30 | 660-2.2.1.1.2 | Advance detectors are designed to detect vehicles at variable distances upstream of an intersection stop bar. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. The advance detector must be capable of detecting vehicles a minimum of 300 feet from the stop line. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS009 |  |  | Init.: |
| The following compliance matrix criteria are for all VVDS. |  |  |  |  |  |
| 31 | 660-5 | The detection system has a manufacturer’s warranty covering defects for a minimum of 1 year from the date of final acceptance. |  | Provide product warranty documentation that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 25) |  |  | Init.: |
| 32 |  | The warranty includes providing replacements, within 10 calendar days of notification, for defective parts and equipment during the warranty period at no cost to the Department or the maintaining agency. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): VVDS001 (Step 26) |  |  | Init.: |

Document History for:

Video Vehicle Detection System Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Conversion to word and update matrix for new 660 detection spec Conversion to word and update matrix for new 660 detection spec | D. Bremer | C. Morse J. Morgan | J. Morgan | 03/13/2013 | No |
| 2.0 | Remove warranty language | D. Bremer | J. Morgan | J. Morgan | 05/09/2013 | No |
| 3.0 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | D. Bremer K. Moser | J. Morgan | J. Morgan | 10/29/2014 | No |
| 4.0 | Updated to reflect latest FHWA approved specification (FA 6-4-15). | R. Meyer | J. Morgan | J. Morgan | 10/15/2015 | No |
| 5.0 | Moved CM to 995, FA 8-26-19. | W. Geitz | M. DeWitt | D. Vollmer | 05/10/2021 | No |
| 6.0 | Update to reflect new FA Date 7-2-20 and clarify cabinet requirements | W. Geitz | C. Raimer M. DeWitt | D. Vollmer | 12/09/2021 | No |
| 7.0 | Corrected CM identifier. Added warranty information. | A. Burleson | W. Geitz | M. DeWitt | 02/01/2022 | No |
| 8.0 | Moved marking requirements to 995-1.1. Added CFR requirement for cameras. New FA Date 10-24-22. | W. Geitz | P. Blaiklock M. DeWitt | D. Vollmer | 02/23/2023 | Yes |
| 9.0 | Updated to latest FA date of 10-6-23 for specs 660 and 995. | W. Geitz | L. Audisio | D. Vollmer | 12/06/2023 | No |
| 10.0 | Added test cases and steps for test method. Updated TERL evaluation methods. Added references to test report checklists. | D. Pedraza | W. Geitz L. Audisio | D. Vollmer | 08/12/2024 | No |
| 11.0 | Removed dates from referenced standards. Added advance detector. Updated to latest FA dates of 8-8-24 and 8-7-24 for 660 and 995, respectively. | W. Geitz | L. Audisio | M. DeWitt | 03/07/2025 | No |

| REV. | DATE | DESCRIPTION | AUTHORED BY | REV MORE STRINGENT? |
| --- | --- | --- | --- | --- |
| 1.0 | 02/14/2013 | Conversion to word and update matrix for new 660 detection spec | David Bremer | No |
| 2.0 | 03/28/2013 | Remove warranty language | David Bremer | No |
| 3.0 | 12/19/2013 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | David Bremer Kelli Moser | No |
| 3.13 | 03/17/2015 | Updated to TERL approved specification. | David Bremer | No |
| 4.0 | 10/01/2015 | Updated to reflect latest FHWA approved specification (FA 6-4-15). | Ron Meyer | No |
| REV. | DATE | DESCRIPTION | AUTHORED BY | REV MORE STRINGENT? |
| 1.0 | 02/14/2013 | Conversion to word and update matrix for new 660 detection spec | David Bremer | No |
| 2.0 | 03/28/2013 | Remove warranty language | David Bremer | No |
| 3.0 | 12/19/2013 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | David Bremer Kelli Moser | No |
| 3.13 | 03/17/2015 | Updated to TERL approved specification. | David Bremer | No |
| 4.0 | 10/01/2015 | Updated to reflect latest FHWA approved specification (FA 6-4-15). | Ron Meyer | No |