|  | FDOT Traffic Engineering Research Laboratory (TERL) Wireless Magnetometer Detection System (WMDS) Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Sections 660 and 995 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Sections 660 and 995, and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| --- | --- | --- | --- | --- | --- |
| The following compliance matrix criteria are for all WMDS. |  |  |  |  |  |
| 1 | 995-1.1 | Equipment is permanently marked with manufacturer name or trademark, part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. | Physical Inspection |
| 2 | 995-2.1 | All parts are made of corrosion-resistant materials, such as UV stabilized or UV resistant plastic, stainless steel, anodized aluminum, brass, or gold-plated metal. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 3 |  | All fasteners exposed to the elements are Type 304 or 316 passivated stainless steel. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 4 |  | If the assembly includes a cabinet, it meets the requirements of Section 676. |  | Provide the applicable compliance matrix from Specification 676 along with all supporting documentation that demonstrates this product meets this requirement, based on cabinet type. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 5 |  | Detector meets the environmental requirements of NEMA TS-2. |  | Provide a third party test report that demonstrates the device performs all required functions during and after being subjected to the environmental testing as described in NEMA TS2 section 2.2.7, 2.2.8, and 2.2.9. Also, provide a completed Testing Laboratory and Report Checklist and NEMA TS2 2.2.7-2.2.9 Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 6 | 995-2.5.1 | WMDS is provided with software that allows local and remote configuration and monitoring. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 7 |  | WMDS allows a user to edit previously defined configuration parameters. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 8 |  | WMDS retains its programming in nonvolatile memory. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 9 |  | Detection system configuration data can be saved to a computer and restored from a saved file. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 10 |  | All communication addresses are user programmable. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 11 |  | Detection system software offers an open Application Programming Interface (API) and software development kit available to the Department at no cost for integration with third party software and systems. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 12 | 995-2.5.2 | Components of the detection system (such as sensors, access points, and contact closure cards) include a minimum of one serial or Ethernet communications interface. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WMDS with serial interface. |  |  |  |  |  |
| 13 |  | Interface and connector conform to Telecommunications Industry Association (TIA)-232 standards. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 14 |  | Serial ports support data rates up to 115200 bps; error detection utilizing parity bits (i.e., none, even, and odd); and stop bits (1 or 2). |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WMDS with Ethernet interface. |  |  |  |  |  |
| 15 |  | Ethernet interface provides a 10/100 Base TX connection. |  | Applicant may provide comments in this field. | Functional Inspection |
| 16 |  | All unshielded twisted pair/shielded twisted pair network cables and connectors comply with TIA 568. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WMDS with wireless communications. |  |  |  |  |  |
| 17 |  | WMDS wireless communications are secure and FCC certified. The FCC identification number is displayed on an external label and all WWVDS devices operate within their FCC frequency allocation. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WWVDS with cellular communications. |  |  |  |  |  |
| 18 |  | Cellular communication devices are compatible with the cellular carrier used by the agency responsible for system operation and maintenance. |  | Provide product literature, specifications, user manual, or similar information that describes any cellular devices that are part of the system and indicates carrier(s) supported. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 19 |  | System can be configured and monitored via one or more communications interface. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for all WMDS. |  |  |  |  |  |
| 20 | 995-2.5.3 | Solid state detection outputs meet the requirements of NEMA TS2. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 21 | 995-2.5.4 | WMDS will operate with a nominal input voltage of 120 VAC. |  | Applicant may provide comments in this field. | Functional Inspection |
| 22 |  | If any system device requires an operating voltage other than 120 VAC, a voltage converter is supplied. |  | Environmental test reports must demonstrate that voltage converters required for 120VAC operation were subjected to NEMA TS2 environmental testing as part of the functional system. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WMDS to be used as presence detectors. |  |  |  |  |  |
| 23 | 995-2.9 | Detector provides a minimum detection accuracy of 98% when calculated in accordance with all criteria detailed in 995-2.9 and all subsections therein. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 24 |  | Detector meets the requirements for Modes of Operation in NEMA TS2. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for WMDS to be used for advance detection. |  |  |  |  |  |
| 25 | 660-2.2.1.1.2 | Advance detectors are designed to detect vehicles at variable distances upstream of an intersection stop bar. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. The advance detector must be capable of detecting vehicles a minimum of 300 feet from the stop line. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| The following compliance matrix criteria are for all WMDS. |  |  |  |  |  |
| 26 | 660-5 | The detection system has a manufacturer’s warranty covering defects for a minimum of 1 year from the date of final acceptance. |  | Provide product warranty documentation that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 27 |  | The warranty includes providing replacements, within 10 calendar days of notification, for defective parts and equipment during the warranty period at no cost to the Department or the maintaining agency. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |

Document History for:

Wireless Magnetometer Detection System Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Conversion to word and update matrix for new 660 detection spec. | D. Bremer | J. Morgan | J. Morgan | 02/14/2013 | No |
| 2.0 | Edited for consistency with other matrices for spec 660. | D. Bremer | J. Morgan | J. Morgan | 03/07/2013 | No |
| 3.0 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | D. Bremer K. Moser | J. Morgan | J. Morgan | 12/19/2013 | No |
| 4.0 | Updated to reflect latest FHWA approved specification (FA 6-4-15). | D. Bremer | J. Morgan | J. Morgan | 10/01/2015 | No |
| 5.0 | Updated with cabinet requirements. Updated to FA date of 7-2-20. | W. Geitz | C. Raimer M. DeWitt | D. Vollmer | 12/18/2020 | No |
| 6.0 | Corrected CM identifier. Added warranty information. | A. Burleson | W. Geitz | M. DeWitt | 02/01/2022 | No |
| 7.0 | Update to the latest FA Date 10-24-22. | W. Geitz | P. Blaiklock M. DeWitt | D. Vollmer | 03/30/2023 | No |
| 8.0 | Updated to latest FA date of 10-6-23 for specs 660 and 995. | W. Geitz | L. Audisio | D. Vollmer | 12/11/2023 | No |
| 9.0 | Updated to latest FA date of 8-8-24 for spec 660 and FA date 8-7-24 for spec 995. | W. Geitz | D. Bremer | M. DeWitt | 03/07/2025 | No |