|  | FDOT Traffic Engineering Research Laboratory (TERL) Remote Power Management Unit (RPMU) Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Sections 685 and 996 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Sections 685 and 996, and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |
| --- | --- | --- |

| Date: |  | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| --- | --- | --- | --- | --- | --- |
| The following compliance matrix criteria are for all RPMUs. |  |  |  |  |  |
| 1 | 996-1.1 | RPMU is permanently marked with manufacturer name or trademark, part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. | Physical Inspection |
| 2 | 996-7.3.1 | RPMU supports local and remote configuration and management, including access to all user-programmable features as well as alarm monitoring, event logging, and diagnostic utilities. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 3 |  | Configuration and management functions are password protected. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 4 |  | RPMU includes an event scheduler that can store a minimum of 60 events. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 5 |  | The RPMU includes LED indicators for relay inputs and outlet status. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 6 |  | Upon loss of communications the RPMU maintains each receptacle and relay in its currently stored state of operation. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 7 |  | Upon restoration of electrical power after an outage the RPMU automatically restores each receptacle and relay to its previously stored state of operation and all configurable parameters are retained. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 8 |  | RPMU supports SNMP v2c, including trap notifications of receptacle state changes. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 9 | 996-7.3.2 | RPMU provides an Ethernet port (RJ45) for local control using a laptop PC and remote control via a network connection. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 10 | 996-7.3.3 | RPMU provides a minimum of 6 NEMA 5-15R receptacles, nominal 120 VAC, and a minimum current capacity of 12 amperes (amps). |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 11 | 996-7.3.4 | All parts are made of corrosion-resistant materials such as plastic, stainless steel, anodized aluminum, brass, or gold-plated metal. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 12 |  | All fasteners exposed to the elements must be Type 304 or 316 passivated stainless steel. |  | Provide a statement of conformance from the supplier shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 13 | 996-7.3.5 | RPMU operates properly during and after being subjected to the environmental testing procedures described in NEMA TS 2, Sections 2.2.7, 2.2.8, and 2.2.9. |  | Provide a first or third party test report that demonstrates the device performs all required functions during and after being subjected to the environmental testing as described in NEMA TS2 section 2.2.7, 2.2.8, and 2.2.9. Also, provide a completed Testing Laboratory and Report Checklist and NEMA TS2 2.2.7-2.2.9 Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
| 14 | 685-5 | The RPMU includes a manufacturer’s warranty covering defects for a minimum of 3 years from the date of final acceptance. |  | Provide product warranty documentation that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |

Document History for:

Remote Power Management Unit Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | New CM | W. Geitz | M. DeWitt C. Raimer | D. Vollmer | 08/21/2020 | N/A |
| 2.0 | Added warranty information. Corrected CM identifier. | A. Burleson | W. Geitz | M. DeWitt | 02/01/2022 | No |
| 3.0 | Moved from 685 to 996. | W. Geitz | P. Blaiklock M. DeWitt | D. Vollmer | 04/12/2023 | No |
| 4.0 | Updated to latest FA date of 12-1-23 for spec 996. | W. Geitz | L. Audisio | D. Vollmer | 01/08/2024 | No |
| 5.0 | Removed final acceptance language and year for referenced standard. Updated to latest FA dates of 10-24-23 for 685 and 10-16-24 for 996. | W. Geitz | A. Burleson | A. Burleson | 02/25/2025 | No |