|  | FDOT Traffic Engineering Research Laboratory (TERL) Internally Illuminated Sign Compliance Matrix | By signing this form, the applicant declares that he/she has read and understands the provisions of Section 995 of the FDOT Standard Specifications for Road and Bridge Construction and all implemented modifications. The requirements listed on this matrix are derived from Section 995 and are the basis for determining a product’s compliance and its acceptability for use on Florida’s roads. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

|  |  | ** Greyed out rows in table below are for TERL use only ** |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| 1 | 995-1.1 | Illuminated sign is permanently marked with manufacturer name or trademark, part number, and date of manufacture or serial number. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 1) |  |  | Init.: |
| 2 | 995-14.1 | Illuminated signs meet the design and functional requirements of the Manual on Uniform Traffic Control Devices (MUTCD) for Streets and Highways. |  | Applicant may provide comments in this field. | Document Review, Physical Inspection and Functional Inspection |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for all illuminated signs. |  |  |  |  |  |
| 3 | MUTCD Section 2A.21.03 | Regulatory, warning, and guide signs are retroreflective or illuminated to show the same shape and similar color both day and night. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review, Physical Inspection and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 1), IIS002 (Step 2), IIS003 (Steps 1, 2) |  |  | Init.: |
| 4 | MUTCD Section 2A.21.04 | Where the color black is specified for the legend or background of a sign, an opaque and nonretroreflective material is used. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 2) |  |  | Init.: |
| The following compliance matrix criteria (referencing the MUTCD) are from the MUTCD and are for all street name signs. |  |  |  |  |  |
| 5 | MUTCD Section 2D.45.28 | If using an alternative background color, the color is blue, brown, or white. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 3) |  |  | Init.: |
| 6 |  | If using an alternative background color, the legend (and border, if used) is white. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 4) |  |  | Init.: |
| 7 |  | For street name signs that use a white background, the legend (and border, if used) are black. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 5) |  |  | Init.: |
| The following compliance matrix criteria are for all illuminated signs. |  |  |  |  |  |
| 8 | 995-14.1 | Marking is accomplished by permanently affixing an indelible label, identification plate, dot peen type stamp, casting, or metal-marking. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 6) |  |  | Init.: |
| The following compliance matrix criteria are for street name signs. |  |  |  |  |  |
| 9 |  | Street name sign assembly does not exceed 9 feet in width. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 3), IIS002 (Step 7) |  |  | Init.: |
| 10 |  | Street name sign assembly is 18 square feet or less in area. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 4), IIS002 (Step 8) |  |  | Init.: |
| 11 |  | Street name sign assembly weighs 144 lbs or less. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 5) |  |  | Init.: |
| The following compliance matrix criteria are for all illuminated signs. |  |  |  |  |  |
| 12 |  | Sign assemblies are listed to the requirements of UL48 or equivalent listed. |  | Provide listing certificate for UL48 standard. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 6) |  |  | Init.: |
| 13 | 995-14.2 | Sign housing is constructed of continuous 5052 or 6063-T5 aluminum. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 7) |  |  | Init.: |
| 14 |  | All housing, corners, and door seams are continuously welded. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 9) |  |  | Init.: |
| 15 |  | All exterior surfaces of the assembly are powder-coat painted in accordance with Military Standard MIL-PRF-24712A or AAMA-2603-02. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 8) |  |  | Init.: |
| 16 |  | Finish meets the requirements of ASTM D 3359, ASTM D 3363, and ASTM D 522. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 9) |  |  | Init.: |
| 17 |  | Sign housings with interior airspace consist of a box type enclosure and separate hinged door assembly. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 10) |  |  | Init.: |
| 18 |  | The sign housing includes provisions to prevent water from entering the sign housing. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): IIS001 (Step 10) |  |  | Init.: |
| 19 |  | Drain holes in the sign larger than .125 inch are screened. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 11) |  |  | Init.: |
| 20 |  | Sign has removable sign face. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 12) |  |  | Init.: |
| 21 |  | The sign face is secured by a method that holds the sign face securely in place. Slide-in grooves are allowed to secure the face if the sign is edge lit. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 13) |  |  | Init.: |
| 22 |  | The sign face is a translucent lens constructed of 0.125 inch high impact strength polycarbonate or acrylic meeting UL48. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 11) |  |  | Init.: |
| 23 |  | Background is retroreflective sheeting coated with a transparent, pressure-sensitive adhesive film. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 12) |  |  | Init.: |
| 24 |  | Sheeting color meets the criteria in Section 994. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 13) |  |  | Init.: |
| 25 |  | Retroreflective sheeting meets the requirements of Section 994 and is listed on the APL. |  | Provide the FDOT APL Number for the Retroreflective Sign Sheeting used. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): IIS001 (Step 14) |  |  | Init.: |
| 26 |  | If a door opens upward, it has a bracket on each side to secure the door in the open position during maintenance. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 14) |  |  | Init.: |
| 27 |  | Doors are permanently and continuously sealed with a UL listed foam gasket listed to UL 157 to prevent the entry of water into the sign housing. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Physical Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 15), IIS002 (Step 15) |  |  | Init.: |
| The following compliance matrix criteria are for street name signs. |  |  |  |  |  |
| 28 |  | Each door is secured by stainless steel rotary action draw latches as follows: Signs of 5 feet up to 7 feet in width must have a minimum of three latches for each sign door. Signs over 7 feet up to 9 feet in width must have a minimum of four latches for each door. The rotary action draw latch is captive and does not become detached or allow the door to open when the sign housing is torqued or twisted. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 16) |  |  | Init.: |
| The following compliance matrix criteria are for all illuminated signs. |  |  |  |  |  |
| 29 |  | Sign assembly is designed for 150 mph wind loads meeting the requirements of the FDOT Structures Manual. |  | Provide the following: - A third party test report OR structural calculations signed and sealed by a Florida P.E. that demonstrate compliance with this requirement. - A completed Testing Laboratory and Report Checklist, if a test report is used. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 16) |  |  | Init.: |
| 30 | 995-14.3 | The sign face is illuminated evenly across the entire surface. |  | Applicant may provide comments in this field. | Functional Inspection |
|  |  | TERL Test Cases (Steps): IIS003 (Step 3) |  |  | Init.: |
| 31 |  | Contrast ratio between the background and legend is at least 4:1. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 17) |  |  | Init.: |
| 32 | 995-14.3.1 | Minimum luminance for the background portion of the sign face is no less than 87.5 lux. The luminance is determined by averaging a minimum of seven readings. Four of the readings are taken near the midpoint of a line that would span between the outside corners of the background and the outside corners of the legend. One reading is taken near the midpoint of a line that would connect the top corner readings. One reading is taken near the midpoint of a line that would connect the bottom corner readings. One reading is taken near the vertical and horizontal midpoint of the sign. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 18) |  |  | Init.: |
| 33 | 995-14.3.2 | Minimum luminance of the legend and border is 350 lux. The luminance is determined by averaging a minimum of 17 readings. There is a minimum of one reading from each letter in the legend. Readings within the legend alternate between the top, middle and bottom portion of each letter. Readings within top and bottom of the border are perpendicular to the top and bottom readings in the background. Readings within the sides of the border are taken parallel to the readings taken within each letter. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 19) |  |  | Init.: |
| 34 | 995-14.4 | All assembly hardware, including nuts, bolts, external screws and locking washers less than 5/8 inch are Type 304 or Type 316 passivated stainless steel. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 20) |  |  | Init.: |
| 35 |  | All assembly hardware greater than or equal to 5/8 inch is galvanized. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 21) |  |  | Init.: |
| 36 |  | Bolts, studs, and threaded rod meets ASTM A307. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 22) |  |  | Init.: |
| 37 |  | Structural bolts meet ASTM F3125, Grade A325. |  | Provide statement of conformance from hardware supplier that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 23) |  |  | Init.: |
| 38 | 995-14.5 | Electrical wiring meets NEC requirements for the light source provided. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): IIS001 (Step 24) |  |  | Init.: |
| 39 |  | All electrical wiring is copper. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 17) |  |  | Init.: |
| 40 |  | Internal electrical wiring is tight and secure. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 18) |  |  | Init.: |
| 41 |  | Sign includes an accessible electrical power service entrance compartment (internal or external) for connection of field wiring. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 19) |  |  | Init.: |
| 42 |  | External compartments are weather-tight. |  | Provide a statement of conformance in this field. | Compliance Matrix Review |
|  |  | TERL Test Cases (Steps): IIS001 (Step 25) |  |  | Init.: |
| 43 |  | All power supplies and ballasts are FCC approved. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 26) |  |  | Init.: |
| 44 |  | Sign has provisions for an integrated photocell. |  | Provide product literature, specifications, user manual, or similar information that shows the product meets this requirement. | Document Review and Functional Inspection |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 27), IIS003 (Steps 4, 5) |  |  | Init.: |
| 45 |  | Electrical connections are protected against corrosion. |  | Applicant may provide comments in this field. | Physical Inspection |
|  |  | TERL Test Cases (Steps): IIS002 (Step 20) |  |  | Init.: |
| 46 | 995-14.6 | Illuminated sign assembly operates properly during and after being subjected to the environmental testing procedures described in NEMA TS 4. |  | Provide the following: - A third party test report that demonstrates compliance with this requirement. - A completed Testing Laboratory and Report Checklist. - A completed NEMA TS4 2.2.3-2.2.6, 3.1.1 & 5.4 Checklist. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 28) |  |  | Init.: |
| 47 | 995-14.7 | Illuminated sign has a manufacturer’s warranty covering defects for 5 years from the date of final acceptance. |  | Provide product warranty documentation that shows the product meets this requirement. | Document Review |
|  |  |  |  | Indicate location of requested information in submittal. |  |
|  |  | TERL Test Cases (Steps): IIS001 (Step 29) |  |  | Init.: |

Document History for:

Internally Illuminated Sign Compliance Matrix

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | CM for new SSRBC Section content moved from A699. Matches content in latest version of SSRBC Section 700 (FA 8-06-13) | R. Meyer | J. Morgan | J. Morgan | 09/27/2013 | No |
| 2.0 | Replaced FDOT logo with latest approved one and added CM ID # to header. Revised document approver title. | D. Bremer K. Moser | J. Morgan | J. Morgan | 10/30/2014 | No |
| 3.0 | Updated to latest FHWA approved date of 1/16/2015. | R. Meyer | C. Morse M. DeWitt | J. Morgan | 04/29/2015 | No |
| 4.0 | Updated to latest FHWA approved date of 7/27/15. No content change. | M. Lucas | J. Morgan | J. Morgan | 10/19/2015 | No |
| 5.0 | Updated to reflect spec changes for FA 10-27-2017 update. | R. Brooks | J. Morgan | J. Morgan | 09/14/2018 | No |
| 6.0 | Updated to reflect the latest FA approval date of 8-15-2018. | R. Brooks | J. Morgan | J. Morgan | 12/17/2018 | No |
| 7.0 | Minor revision to reflect FA 8-6-20 with no changes to this CM. | W. Geitz | M. DeWitt | D. Vollmer | 12/28/2020 | No |
| 8.0 | Minor revision to reflect FA 2-12-21 with no changes to this CM. | W. Geitz | C. Raimer M. DeWitt | D. Vollmer | 07/12/2021 | No |
| 9.0 | Added warranty information. Updated to latest FA date of 8-5-21. | A. Burleson | W. Geitz | M. DeWitt | 02/01/2022 | No |
| 10.0 | Moved from 700 to 995 (FA Date 10-24-22) for new spec book. Removed section for retrofit kits. | W. Geitz | R. Washington M. DeWitt | D. Vollmer | 01/26/2023 | No |
| 11.0 | Updated to latest FA date of 10-6-23 and 10-18-23 for spec 995. | W. Geitz | D. Christian | D. Volmer | 12/07/2023 | No |
| 12.0 | Removed NEMA date and added MUTCD requirements. Updated to the latest FA date of 8-7-24 for spec 995. | W. Geitz | D. Bremer | M. DeWitt | 04/17/2025 | No |
| 13.0 | Added test cases/steps. | A. Burleson | W. Geitz | M. DeWitt | 05/08/2025 | No |
| 14.0 | Updated to latest FA date of 8-14-25 for spec 995. CM ID 32 contains more stringent requirements. | A. Cramer W. Geitz | E. Velez | D. Vollmer | 12/11/2025 | Yes |