|  | FDOT Traffic Engineering Research Laboratory (TERL) SAE J595 5.1.5 & 6.1.5 Checklist | By signing this form, the applicant declares that he/she has read and understands the provisions of the SAE J595 Standard. The requirements listed on this checklist are derived from Sections 5.1.5 and 6.1.5 and are the basis for determining a product’s compliance to these sections of the standard. |
| --- | --- | --- |

| Date: | Click here to enter a date. | Applicant’s Name (print): |  |
| --- | --- | --- | --- |
| Manufacturer: |  |  |  |
| Item, Model No.: |  | Signature: |  |

|  |  | ** Greyed out rows in table below are for TERL use only ** |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID No | Section | Requirement | Item Comply? (Yes/No/NA) | Comments (Applicant must provide information as indicated) | TERL Evaluation Method |
| 1 | 5.1.5 | Photometric measurements are taken with the RRFB assembly flashing. |  | Photometric measurements are optical power and peak intensity. | Document Review |
|  |  |  |  |  | Init.: |
| 2 |  | An integrating photometer is used to determine optical power. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 3 |  | The optical power of the RRFB is measured at all test points and the peak intensity is measured at the horizontal-vertical (H-V) point. |  | A goniophotometer can be used to measure the peak intensity at all test points. | Document Review |
|  |  |  |  |  | Init.: |
| 4 |  | An adjustment factor is calculated by dividing the peak intensity by the optical power, as measured at the H-V point. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 5 |  | The peak intensity of each test point is determined by multiplying the measured optical power at each point by the adjustment factor. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 6 | 5.1.5.1 | The device is energized, and the H-V photometric values are recorded after one minute. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 7 |  | The device is continuously energized until photometric stability occurs. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 8 |  | The photometric values are recorded at each test point. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 9 |  | The ratio between the 1 minute H-V reading and the photometric stability H-V reading is calculated and applied to all test points to determine the 1 minute photometric performance. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 10 | 5.1.5.2 | Photometric measurements are made with the device mounted in its normal operating position and all measurements are made with the geometric center of the RRFB lamp assembly of the device at least 18 m from the photometer sensor. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 11 |  | The device is mounted so that the horizontal plane through the photometer sensor axis passes through the center of the RRFB lamp assembly. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 12 |  | The vertical axis through the center of the RRFB lamp assembly is perpendicular to the horizontal plane. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 13 |  | The H-V reference point is the geometric center of the RRFB lamp assembly. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 14 | 6.1.5 | The flash pattern of the RRFB is compliant if it meets the requirements of Figure 1-Y. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 15 | Figure 1-Y | The measured values at each test point are not less than 60% of the required minimum values shown for that individual test point location. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 16 |  | The sum of the optical power measurement and the sum of the peak luminous intensity measurements for each test point within a zone, are not less than the corresponding zone total requirements shown. |  |  | Document Review |
|  |  |  |  |  | Init.: |
| 17 |  | The measurements at each discrete test point shown within the corresponding zone are the values used to calculate the specified zone total. |  |  | Document Review |
|  |  |  |  |  | Init.: |

Document History for:

SAE J595 5.1.5 & 6.1.5 Checklist

| Rev | Description | Authored and Checked | Reviewed | Approved | Approval Date | Rev More Stringent? |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | New checklist | A. Cramer | W. Geitz D. Bremer | D. Vollmer | 02/18/2026 | N/A |
| 2.0 | Clarified photometric measurements and test equipment. | A. Cramer | W. Geitz | W. Geitz | 09/24/2026 | No |